| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Control Section in Amendment Text |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Xiaoyi Wang Nokia Siemens Networks Zexian Li Nokia | xiaoyi.wang@nsn.com Zexian.Li@nokia.com |
| Re: | “802.16m AWD text”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Draft AWD Content”. Target topic: “Power control”. |  |
| Abstract | The contribution proposes the text of power control section to be included in 802.16m amendment. |  |
| Purpose | Discussion and adoption of the text proposed in this document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Power Control Section in Amendment Text

Brief introduction of Open Loop Power Control (OLPC) Schemes

Normally, the UL transmission Power Spectrum Density (PSD) is decided by OLPC equation:

PSD=SINRT+IoT+N0+α⋅PL

Where, PSD is the transmit power density. SINRT is the target SINR of UL signal. IoT is the interference exceed over thermal noise density, N0 is the thermal noise power density, α is the path loss compensation factor designated by ABS. When α < 1 only a fraction of the path loss is compensated. PL denotes path loss. This is so-called “fractional power control”.

Such a scheme is targeted at adjusting the UL TX power according to the target SINR. However, interference  among cells is not considered in the scheme. The controlling on interference is sub-optimal.

Considering the target IoT to other cell/sector, Interference Constraint Power Control: ICPC is <people are reluctant for anything new, let’s tone it down> proposed:

PSD=NRT+N0−g

Where

PSD is the transmit power density.

N0 is the thermal noise power density.

NRT is the target noise rise level designated by ABS.

Note that all values are in dB.

g is the sum of path gain (including antenna gain and shadow fading) to all other sectors, the following equation should be adopted:

g=(P-N)-Txp-PG,

where g is the sum path gain, P is the total received power (measured on preamble), N0 is the noise power density, Txp is the transmitting power of ABS, and PG is the path gain from AMS to serving ABS.

NRT values are broadcasted by ABS, as an option, ABS also could unicast a special value to any AMS, Considering NRT value are related to MCS. Normally ABS will give a set of NRT values.

E.g. NRT=[13.0, 4.0, 3.0, 2.0, 1.0, 1.0, 1.0, 0.0, 0.0, 0.0, 0.0 ]

Then AMS should use the NRT[MCS index] for power control.

Considering that power control is in a comparably slower pace than MCS adjust, as an option, we propose to use 1st order forgetting filter to smoothe NRT value for power control.

NRT=a*NRT+(1-a)NRT[MCS[n]], where MCS is the value for power control usage, MCS[n] is the actual MCS of this AMS in frame n, and a denotes the forgetting factor, 0<= a <= 1.

Simulation Results

Simulation assumptions are defined in Table 1.

Table 1

| Parameter | Value | Note |
| --- | --- | --- |
| System bandwidth | 10MHz |  |
| Number of Users per sector | 10 |  |
| Scheduling algorithm | PF |  |
| PC Period | 500.0 ms |  |
| TTI | 1 subframe |  |
| Slow fading, std. dev | 8.0 dB | Slow fading is updated every 100 ms. Correlated distance is 50m. |
| Use fast AMC | Yes | MCS is selected every TTI |
| ISD | 500m |  |
| Penetration loss | 20dB |  |
| UE speed | 3 km/h |  |
| UE Max power | 25 dBm |  |
| UE Min power | -15dBm |  |

Figure 2 shows the average throughput gain without sacrificing the coverage performance.  For fair comparison, mobile terminal has similar Tx power level in both cases, see Figure 1.

Figure 1 CDF of UE tx power

Figure 2 Throughput

Power control Proposed Text for Amendment

---------------------------------------------------------Start of the Text---------------------------------------------------------

[Add the following into the TGm Amendment Document]

15.3.x Power Control

Power control algorithm shall be supported for the UL channel with both initial calibration and periodic adjustment procedure without loss of data. Power control is based on each AMS rather than each flow. The ABS should be capable of providing accurate power measurements of the received burst signal. This value can then be compared against a reference level, and the resulting error can be feedback to the AMS through RNG_RSP message. The exact algorithm implementation is vendor-specific.

If the sub-carrier TX power specified by power control mechanisms makes the total transmit power for a given transmission exceed the maximum transmit power for the specified MCS, the transmit power shall be limited to the maximum allowed value of AMS capacity.

An AMS shall maintain the same transmit power density regardless of the number of subchannels assigned, unless the maximum power level is reached. In other words, when the number of active subchannels allocated to a user is reduced, the total transmit power shall be reduced proportionally by the AMS, without additional power control messages. When the number of subchannels is increased, the total transmit power shall also be increased proportionally. If without changing the power density, the total transmit power exceeds maximum power, AMS should decrease transmit power density of data sub-carrier proportionally. Transmit power density of UL control channel should remain same unless the transmit power of data burst is already zero but total power still exceeds maximum power.

15.3.x.1 Power Control for Initial Ranging

For initial ranging, AMS sends initial ranging code at a random selected ranging channel. The initial transmission power is decided according to measured RSS. If AMS does not receive a response, AMS may send a new initial ranging code and increase its power level by PIRstep. AMS could further increase the power until maximum transmit power reached.  The maximum transmit power for initial ranging is decided as:

PTXIRMAX=EIRxPIR,max+BSEIRP−RSS

Where EIRxPIR,max and BSEIRP are obtained from ABS through decoding S-SFH IE SP3, see table. 1.

EIRxPIR,max is the targeting receving power for initial ranging code.

BS_EIRP is the transmission power of BS.

In the case that the Rx and Tx gain of the AMS antenna are different, the AMS shall use Equation:

PTXIRMAX=EIRxPIR,max+BSEIRP−RSS+(GRxMS−GTxMS)

Where

GRxMS is the antenna gain of AMS RX.

GTxMS is the antenna gain of AMS TX.

RSS is the measured receiving signaling strength by AMS.

For the periodic ranging, once AMS sends periodic ranging code and fails to receive RNG-RSP, AMS may adjust its Tx power for the subsequent periodic ranging codes transmission up step by step to PTXIRMAX

Table. 1 S-SFH IE SP3:

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 3 | Initial ranging channel information (initial ranging region location | TBD |
|  | EIRxPIR,max | 8 |
|  | BS_EIRP | 8 |
|  | PIRstep | 2 |
|  | TTG RTG | TBD |
|  | PTXIRMAX | 2 |

15.3.x.2 Open loop Power Control

AMS Transmit Power Spectrum Density(PSD) is decided by:

PSD=NRT+N0−g

Where

PSD  is the transmit power density;

N0 is the thermal noise power;

NRT is the target noise rise level designated by BS.

g is the sum of path gain (including antenna gain and shadow fading) to all other sectors.

Note that all these values are in dB.

The estimation of g is according to the sum power of receiving DL preamble from all other sectors.

g=P−N0BSEIRP−PGserving

where g is the sum path gain, P is the total received power (measured on received preamble), N0 is the noise power density, BS_EIRP is the transmitting power of BS obtained from S-SFH, and PG is the path gain from AMS to serving BS.

Optionally, NRT values broadcast by BS could be a vector to improve the performance. In that case, NRT value are related to MCS.

Then AMS should use the NRT for power control self update using the following equation:

NRT=a*NRT+(1-a)NRT[MCS[n]]

where MCS[n] is the actual MCS index of this AMS in frame n, and a denotes the forgetting factor, 0<= a <= 1. The value of a is TBD.

If load information of adjacent sector is known (e.g. load factor is broadcast to facilitate the HO or other purpose), AMS may take the load factor into account using the following revised equation:

The estimation of g is according to the sum power of receiving DL preamble from all adjacent sectors.

g=P−N0BSEIRP−PGserving−1−LnPGn,

where g is the sum path gain, P is the total received power (measured on received preamble), N0 is the noise power density, BS_EIRP is the transmitting power of BS, and PGserving is the path gain from AMS to serving BS. Ln is the load factor (known by AMS) of sector n, PGn is the path gain between MS and sector n, estimated by AMS itself.

This revised equation increase AMS Tx power if neighbor sector is not in full load, however, considering complexity, AMS may decide not to use this equation even if the load factor is known.

15.3.x.3 Closed-loop Power Control

ABS shall send a PMC_RSP_messgae, see table 2.  . AMS may request to enter CLPC mode using PMC_REQ message, see table 3. While in CLPC mode, ABS should send PC command or compact PC command to AMS periodically. PC command are defined in 15.3.6.5.2.4   If no PC command is received after a given duration, assigned by PMC_RSP_message, AMS will consider the connection with ABS interrupt and switch back to OLPC mode automatically. Upon receiving PC command, AMS shall follow the command to adjust its pilot PSD. And the data sub-carrier PSD is decided by the data to pilot ratio of corresponding MCS

15.3.x.4 Messages related to Power control

Table.2 PMC_RSP message

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PMC_RSP message { |  |  |
| Type | 8 |  |
| Power mode | 1 | 0:OLPC 1:CLPC |
| If (Power mode=0){ |  |  |
| NRT | TBD | Noise Raise Target |
| Load factor of adjcent | TBD | Load factor information of adjacent cell/sectors |
| } else if (Power mode=1){ |  |  |
| Duration of no PC |  | The duration that no PC command is tolerated |
| } |  |  |
| } |  |  |

Table.3 PMC_REQ message

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PMC_REQ message { |  |  |
| Type | 8 |  |
| Power mode | 1 | 0:OLPC 1:CLPC |
| Reserved | 7 |  |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------