| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text for Addressing and Connections |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Dongyao Wang, Gang Shen, Kaibin Zhang, Shan Jin Alcatel-Lucent Shanghai Bell | Dongyao.wang@alcatel-sbell.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0012: Call for contributions on Project 802.16m Amendment Working Document (AWD) Content: Addressing |  |
| Abstract | Proposed Contents for the P802.16m Amendment on the Addressing and Connections |  |
| Purpose | For review, discussion and adopt in the Project 802.16m Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text for Addressing and Connections

Dongyao Wang, Gang Shen, Kaibin Zhang, Shan Jin

Alcatel-Lucent Shanghai Bell

Introduction

This contribution provides amendment text for Addressing and Connections in the Advanced Air Interface to be included in Section 15.2 of the IEEE 802.16m Amendment Working Document [1]. The text was developed with the baseline text from the Draft IEEE 802.16m System Description Document (SDD) [2].

References

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-09/0010.

- IEEE 802.16m System Description Document (SDD), IEEE 802.16m-08/003r7.

Proposed Amendment Text

The following text is proposed for inclusion in Section 15.2 of the IEEE 802.16m Amendment Working Document [1].

# 15.2.x Addressing and Connections

Each air interface shall have a 48-bit universal MAC address, as defined in IEEE Std 802®-2001. This address uniquely defines the air interface of the MS. It is used during the initial ranging process to establish the appropriate connections for an MS. It is also used as part of the authentication process by which the BS and MS each verify the identity of the other.

With the domain of BS, MS is uniquely identified by a 16-bit STID (Station Identifier). The BS shall assign a STID to the MS during network entry and, in some cases, network re-entry. Each MS registered in the network has an assigned STID. And some predefined “STIDs” are used to allocate the physical resources with the special pruposes. STID values are presented in Table 1.

Table 1 STID values

| STIDs | Value (16-bit) | Description |
| --- | --- | --- |
| Ranging STIDs | 0x0000 ~ 0x0003 | The assigned physical resources are for ranging, such as initial ranging, periodical ranging, handover ranging and contention bandwidth request |
| User Private STIDs | 0x0004 ~ 0xEFFF | Identity of the MS, and the assigned physical resources are for specific MS |
| Reserved STIDs | 0xF000 ~ 0xFEFF | Reserved for future use |
| Multicast STIDs | 0xFF00 ~ 0xFFFB | The assigned physical resources are for multicast service flows |
| Sleep Mode Multicast STID | 0xFFFC | The assigned physical resources are for those in sleep mode |
| Idle Mode Multicast STID | 0xFFFD | The assigned physical resources are for those in idle mode |
| Broadcast STID | 0xFFFF | The assigned physical resources are for broadcast service flows |

Within the domain of the MS, each connection is uniquely identified by a 4-bit FID (Flow Identifier). Two kinds of connections are defined, i.e. management connection and transport connection. For management connections, their FIDs are pre-assigned globally as shown in Table 2. After MS initialization, both the BS and the MS automatically use these pre-defined management connections. For bearer services, the BS and the MS may initiate the set-up of service flow based upon the provisioning information. The registration of an MS, or the modification of the services contracted at an MS, stimulates the higher layers of the BS and/or the MS to initiate the setup of the service flows. When admitted or active, service flows are uniquely associated with transport connections with assignment of unique FIDs within the MS. MAC management messages shall never be transferred over transport connections. Bearer or data services shall never be transferred on the management connections. In addition, during the handover process, the FIDs of both management connections and transport connections can remain unchanged.

Table 2 FID values

| FID | Value (4-bit) | Description |
| --- | --- | --- |
| Management FID | 0b0000 | Identity of the management connection |
| Transport FIDs | 0b0001 ~ 0b1010 | Identity of the transport connections |
| Reserved FIDs | 0b1011 ~ 0b1110 | Reserved for future use |
| Padding FID | 0b1111 | Used for padding information |

For the assignment of physical resources, STID should be used in the Advanced MAP to indicate the owner of the specific resources. For the transmission of data packets, the FID shall be included in the MAC PDU. Within the domain of the BS, one connection is uniquely identified by the tuple <STID, FID>.