| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on 802.16m Link Adaptation |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Xin Qi, Peter Wang, Joon Chun Nokia Siemens Networks Andrei Malkov, Zexian Li Nokia | E-mail: xin.qi@nsn.com |
| Re: | “802.16m AWD text”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Draft AWD Content”. Target topic: “Link Adaptation”. |  |
| Abstract | This contribution proposes amendment text on 802.16m link adaptation |  |
| Purpose | To incorporate the proposed text into the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Amendment Text on 802.16m Link Adaptation

Xin Qi, Peter Wang, Joon Chun

Nokia Siemens Networks

Andrei Malkov, Zexian Li

Nokia

Introduction

The contribution proposes the text of link adaptation section to be included in the 802.16m amendment [1]. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [2], it is compliant to the 802.16m SRD [3] and the 802.16m SDD [4], and it follows the style and format guidelines in [5].

Reference

- IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” Jan. 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

Proposed Amendment Text

-------------------------------------------------------------------------------------------------------------------------------------

# Link Adaptation

## DL Link Adaptation

Adaptive modulation and channel coding (AMC) scheme is supported for DL transmission. The serving ABS can adapt the modulation and coding scheme (MCS) level based on the DL channel quality indicator (CQI) reported from AMS through UL feedback channel. In time-division-duplex (TDD) system DL channel quality can be also derived from the estimate of the UL channel at ABS, assuming that UL and DL channels are reciprocal. This estimate can be done relying on the UL data transmission or sounding /reference signal from AMS.

### DL Channel Quality Acquiring by ABS

###### DL Channel Quality Estimation Based on UL Sounding for TDD

ABS broadcasts the sounding region used by the AMSs for transmitting the sounding signal in the UL. After ABS receives the averaged DL CQI (i.e., averaged on the whole bandwidth) from AMSs, ABS decides according to the CQI information which AMSs should perform UL sounding and then transmits the Sounding Request message (TBD) to the AMSs informing the parameters used for performing the sounding. The sounding parameters include sounding signal configuration and sounding channel allocations.

ABS may broadcast a per-PRU/subband CQI threshold and the informed AMSs perform opportunistic sounding based on the CQI threshold. If opportunistic sounding is enabled for an AMS, the AMS sends sounding signals on only specified sounding channels. If the CQI measured on DL for a particular PRU/subband exceeds the CQI threshold then the AMS transmits sounding signal in the corresponding UL PRU/subband. Otherwise, the AMS shall not transmit the sounding signal in the corresponding PRU/subband. ABS detects the sounding signals in allocated sounding channels and can calculate the CQI information of UL PRU/subband where AMS transmits sounding signals. When the AMS transmits sounding signal, it should adjust the sounding Tx power control level which can be used to determine the DL per PRU/subband CQI by ABS.

            - DL Channel Quality Estimation Based on UL Sounding for AMS with Multi Tx Antennas

ABS can assign AMS to send multiple sounding signals with multiple Tx chains. Maximum number of sounding signals transmitted by an AMS is the number of Tx chains of the AMS. When the number of AMS Tx chains is the same as that of AMS Rx chains and that of assigned sounding channels, AMS transmits sounding signal on each assigned sounding channel with the corresponding Tx chain. When the number of AMS Tx chains is the same as that of AMS Rx chains but larger than that of assigned sounding channels, ABS can assign a unique sounding channel to the first physical Tx antenna of the AMS and to allow the rest physical Tx antennas to share the remaining sounding channels, with which ABS can probabilistically achieve most MIMO channel information with minimized sounding channel allocation. For example, one AMS has two Tx antennas called AMS_11 and AMS_12 and another AMS has two Tx antennas called AMS_21 and AMS_22. ABS can assign a unique sounding channel to AMS_11 and another unique sounding channel to AMS_21. ABS should assign a shared sounding channel to both AMS_12 and AMS_22 to minimize sounding channel occupancy. On the other hand, ABS can also control the AMS to switch the sounding channels between the physical Tx antennas, with which ABS can deterministically achieve full MIMO channel information with full sounding channel allocations for each individual physical Tx antennas. Also, when the number of AMS Tx chains is smaller than that of AMS Rx chains, ABS can control the AMS to switch the physical Tx antennas. The signaling method is TBD.

### DL Burst Size Determination

ABS decides the MCS level and MIMO mode based on the DL channel quality information for data transmission, and further decides the resource allocation based on size of MAC PDU to be transmitted. The resource allocation includes LRU type, LRU number and LRU location etc. ABS indicates MCS level, MIMO mode and LRU type/number/location to AMS, based on which AMS decides the information size in the burst and performs channel decoding.

If MAC PDU can not fit in the allocated LRUs, padding bits will be added to the end of the PDU. ABS can adjust the channel coding rate and remove all or part of the padding bits, to either save one LRU by increasing channel coding rate or to improve performance by reducing channel coding rate. In this case, ABS transmits Burst Size Adjustment to AMS so that AMS can calculate the PDU size and perform the correct channel decoding. The Burst Size Adjustment is denoted by  and explained below.

Let

NBlk       be the number of FEC blocks in the data burst.

be the modulation and coding rate of the burst, which is indicated by the ABS based on channel quality. The corresponding channel coding rate is denoted by , and the modulation order is denoted by .

NRU       be the number of LRUs to transmit the burst, indicated by ABS.

be the minimum number of LRUs to contain  bits using the selected MCS and MIMO mode.

be the number of information bits which could be transmitted in  LRUs using the selected MCS and MIMO mode .

Nbit−      be the number of information bits which could be transmitted in  LRUs using the selected MCS and MIMO mode.

be the number of bits in the MAC PDU to be transmitted in a burst, .

be the number of information bits in the burst after coding rate adjustment

Given ,  and MIMO mode,  is determined by looking up the burst size table given in section15.3.x.1.2. When ,  padding bits need to be added to the MAC PDU to form the information bits of the burst, so that the information size of the burst could fit in  LRUs. ABS may decide to adjust the channel coding rate to either save one LUR by increasing coding rate, or to improve the performance of channel coding by reducing coding rate. For the case of increasing coding rate, the  must be greater than 1, and the channel coding rate increase should not sacrifice PER performance to below target. Selection of increasing or reducing coding rate is determined by channel state and the target BLER. The detail selection algorithm is out of scope of 16m specification. Procedure of coding rate adjustment is as follows:

- Compute , .

- If , there is no need for channel coding rate adjustment. . In this case, set , .

- If , ABS could decide to proceed the following coding rate adjustment.

  - If the coding rate adjustment is not activated,  bits are padded to the burst. The number of information bits transmitted in the burst is . In this case, set , .

  - If the coding rate adjustment is activated,  padding bits are removed, . The granularity of  is FFS. Only  bits are added to the burst. [Note: Theoretically,  is the best way to remove padding bits. However,  may not fit in the coding block size table. In this case, certain number of padding bit is still needed anyway.] The number of information bits transmitted in the burst is .  should be a value which could fit in the coding block size table as specified in the Channel Coding section. ABS decides whether coding rate is increased or reduced.

    - If coding rate is increased,

      - Set the channel coding rate adjustment indicator, .

      - .

    - If coding rate is to be reduced:

      - Set the channel coding rate adjustment indicator, rfcr=1.

      - .

information bits are encoded. After rate matching according to the Channel Coding section, the encoded bits are modulated and mapped to NRU LRUs.

rfcr and  may be transmitted in Assignment A-MAP. If , coding rate adjustment is not enabled. If , based on rfcr, , NRU,  and MIMO scheme, AMS can infer .

###### DL Burst Size Table

[Note: the burst size Table 1 is generated based on the assumption of 96 subcarriers per LRU. The number of LRUs per burst is assumed to be from the range [1,100]. The entries in the table could fit in the coding block size table. The MCS table and the coding block size table are defined by the Channel Coding and HARQ DG.]

Table 1 DL burst size table for single data stream

## UL Link Adaptation

Adaptive modulation and channel coding (AMC) scheme is supported for UL transmission. The serving ABS can adapt the modulation and coding scheme (MCS) level based on the UL channel quality estimation and the maximum transmission power by AMS. The definition of UL channel quality indicator is FFS. The UL AMC may be integrated with UL power control and interference mitigation schemes to further achieve higher spectral efficiency.

### UL Channel Quality Acquiring by ABS

###### UL Channel Quality Estimation Based on UL Sounding

ABS broadcasts the sounding region used by the MSs for transmitting the sounding signal in the UL. ABS transmits the Sounding Request message to the AMS informing the parameters used for performing the sounding. The sounding parameters include sounding signal configuration and sounding channel allocations.

In TDD system, ABS may broadcast a per-PRU/subband CQI threshold and inform MSs to perform opportunistic sounding based on the CQI threshold. If opportunistic sounding is enabled for an AMS, the AMS sends sounding signals on only specified sounding channels. If the CQI measured on DL for a particular PRU/subband exceeds the CQI threshold then the AMS transmits sounding signal in the corresponding UL PRU/subband. Otherwise, the AMS shall not transmit the sounding signal in the corresponding PRU/subband. ABS detects the sounding signals in allocated sounding channels and can calculate the CQI information of UL PRU/subband where AMS transmits sounding signals.

### UL channel quality estimation based on UL sounding for AMS with multi Tx antennas is the same as in section 15.3.x.1.1.1.1. ABS can schedule UL MIMO scheme based on UL sounding channel estimation.

-------------------------End of Proposed Amendment Text---------------------------------------------------------