| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Idle Mode Operation Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Maheshwari Shashikant NSN Shantidev Mohanty Intel Corporation Ronny Yongho Kim LG Electronics, Inc. | shashi.maheshwari@nsn.com shantidev.mohanty@intel.com ronnykim@lge.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “10.5 Power Management (Sleep, Idle)”. |  |
| Abstract | The contribution proposes the text of idle mode operation section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Idle Mode Operation Section 
for the IEEE 802.16m AWD

Maheshwari Shashikant

NSN

Shantidev Mohanty

Intel Corporation

Ronny Yongho Kim

LG Electronics, Inc.

- Introduction

Basic idle mode operation for IEEE 802.16m is described in harmonized contribution C80216m-09_0597.doc or its latest revision. This contribution proposes text additions to that contribution.

-------------------------------  Text Start  ---------------------------------------------------

- Advanced Air Interface

  - Medium access control

[Insert the followings at the subclause 15.2:]

    - Idle mode

[Editor’s note: Add the following text]

When an AMS is assigned to more than one paging groups, one of the MS’s paging groups is called primary paging group and rest of the assigned paging group is called secondary paging group.

ABSs and Idle Mode AMSs may belong to multiple paging groups. In case an AMS belongs to multiple paging groups, it starts Paging Group Location Update Timer (PG_LU_TIMER) when it leaves primary paging group. An AMS performs the paging group location update after PG_LU_TIMER and may inform its mobility (slow, medium, fast) to ABS. Based on the AMS mobility information, the ABS may assign new paging group(s) of different size(s) to AMS.

##### Broadcast paging message

[Editor’s note: Add the following text]

ABS transmits the paging information in the S-SFH SP5 during AMS's paging listening interval as shown in Figure zzz. S-SFH SP5 includes the PGID[s] that ABS belongs to and may also include the whether or not paging indicator is used using the Paging indicator usage flag. When paging indicator usage flag is set to 1, ABS transmits associated paging indicator flag in the S-SFH SP5 indicating the presence of full paging messages for the corresponding PGIDs.

Figure zzz: Transmission of paging information.

Within a paging listening interval, the frame that contains the paging message for one or group of idle mode AMSs is known to idle mode AMSs. Methods to determine the frame/subframe (within a super- frame) that contains the paging message for one or group of idle mode AMSs are TBD.

Additionally, if paging indicators are present, AMS also monitors the paging indicators. If the paging indicator flag associated with its primary PGID is set then AMS will subsequently decode the full paging message [at the predetermined location]; otherwise AMS will return to paging unavailable interval. If paging indicators are not present, AMS decodes the full paging message [at the predetermined location]. If the AMS decodes a paging message that contains its identification, the AMS performs network re-entry or location update depending on the notification indicated in the paging message. Otherwise, AMS returns to paging unavailable interval.

-------------------------------  Text End  ---------------------------------------------------