| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of Addressing Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-12 |  |
| Source(s) | Sungjin Lee, Jungje Son, Rakesh Taori Samsung Electronics Mary Chion, Sean Cai ZTE Changhoi Koo Huawei Technologies David Johnston, Muthaiah venkatachalam Intel Corporation Hua Xu Motorola Haihong Zheng, Shashikant Maheshwari, Yousuf Saifullah NSN Zexian Li, Jan Suumaki Nokia Kiseon Ryu LG Electronics Feder Peretz Alcatel Lucent Dongyao Wang, Gang Shen, Kaibin Zhang, Shan Jin Alcatel-Lucent Shanghai Bell | E-mail:steve.lee@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> E-mail: mchion@zteusa.com E-mail: ckoo@huawei.com E-mail: dj.johnston@intel.com E-mail: hua.xu@motorola.com haihong.zheng@nsn.com Zexian.li@nokia.com ksryu@lge.com pfeder@alcatel-lucent.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: Addressing. |  |
| Abstract | The contribution proposes the text of Addressing section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Addressing Section for the IEEE 802.16m AWD

Sungjin Lee, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

Mary Chion, Sean Cai

ZTE

Changhoi Koo

Huawei Technologies

David Johnston, Muthaiah venkatachalam

Intel Corporation

Hua Xu

Motorola

Haihong Zheng, Shashikant Maheshwari,

Yousuf Saifullah

NSN

Zexian Li, Jan Suumaki

Nokia

Kiseon Ryu

LG Electronics

Feder Peretz

Alcatel Lucent

Dongyao Wang, Gang Shen, Kaibin Zhang, Shan Jin

Alcatel-Lucent Shanghai Bell

- Introduction

This contribution proposes amendment text to describe the 802.16m MAC addressing section and is intended as the section to be included in the 802.16m amendment.

- Modifications to the SDD text

The most text proposed in this contribution is copied from the subclauses 10.1 in the IEEE 802.16m SDD [3]. The modifications to the SDD text are underlined

- Updates reflecting harmonization with C80216m-09_0517 and C80216m-09_0651

Section 15.2.1.2.3 is added for describing Temporary Identifier. Some specific name of management messages are deleted since they are not included in SDD yet

----------------< Start of the Proposed text >-----------------

15.2.1	Addressing

The AMS has a global address and logical addresses that identify the AMS and connections during operation.

15.2.1.1	MS MAC Address

The AMS, ARS and ABS are identified by the globally unique 48-bit IEEE Extended Unique Identifier (EUI-48™) based on the 24-bit Organizationally Unique Identifier (OUI) value administered by the IEEE Registration Authority.

15.2.1.2	Logical Identifiers

The following logical identifiers are defined in the following subsections.

15.2.1.2.1	 Station Identifier (STID)

The ABS assigns a 12 bits long STID to the AMS during network entry, and, in some cases, network re-entry, that uniquely identifies the AMS within the domain of the ABS.  Each AMS registered in the network has an assigned STID. Some specific “STIDs” are reserved, for example, for broadcast, multicast, and ranging.

15.2.1.2.2	 Flow Identifier (FID)

Each AMS connection is assigned a 4bits long FID that uniquely identifies the connection within the AMS. FIDs identify management connections and transport connections. Some specific FIDs may be pre-assigned.

15.2.1.2.3 Temporary Identifier

The network may assign a temporary Identifier to uniquely identify an AMS in the idle mode in a

particular paging group. The temporary identifier is assigned during idle mode entry or location update due to paging group change. Such identifier remains valid as long as the AMS stays in the same paging group. The temporary identifier is used in paging messages to identify the AMS. It is also used by the AMS to identify itself during its network re-entry procedure as response to paging or location update when paging group is not changed.

---------------------<End of the proposed text> ---------------------

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”