| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal text of bandwidth request for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Pei-Kai Liao, Yih-Shen Chen, Chih-Yuan Lin and Paul Cheng MediaTek Inc Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, His-Min Hsiao, Ren-Jr Chen, Yutao Hsieh ITRI | pk.liao@mediatek.com |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) content”. Target topic: “Comment on 802.16m AWD”. |  |
| Abstract | The contribution proposes the text of SFH contents to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal text of bandwidth request for IEEE 802.16m Amendment

Pei-Kai Liao, Yih-Shen Chen, Chih-Yuan Lin, Paul Cheng

MediaTek Inc.

Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, His-Min Hsiao,

Ren-Jr Chen, Yutao Hsieh

ITRI

Introduction

In the UL_Ctrl DG discussion, quick access method, including bandwidth request indicator and message, is addressed. The performance of indicator detecting plays an important role in the quick access method. We believe, with multiple antenna transmission, the detecting performance can be further improved even when multiple MSs send out the same request indicator simultaneously.

- Proposed text

Add the red text into following section of [1]:

-------------------------------  Text Start  ---------------------------------------------------

# 15.3.9.2.5 Bandwidth request Channel

Contention based random access is used to transmit bandwidth request information on this control channel. The bandwidth request (BW REQ) channel contains resources for the AMS to send a BW REQ access sequence and an optional quick access message. Prioritized bandwidth requests are supported on this channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure is described in Figure UL- 8. A 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1, 4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, AMS sends a bandwidth request indicator and a message for quick access. For the AMS with multiple antennas, it shall apply rank-1 MIMO scheme defined in section 15.3.7 to transmit the bandwidth request indicator and message if multi-antenna transmission is enabled. The 5-step regular procedure is used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The AMS may piggyback additional BW REQ information along with user data during uplink transmission (step 5).  Following Step 1 and Step 3, ABS may acknowledge the reception of bandwidth request. If AMS does not receive any acknowledgement or UL grant, it waits until the expiration of a pre-defined period and restarts the bandwidth request. The pre-defined period may be differentiated by factors such as QoS parameters (e.g. scheduling type, priority, etc). In case BW is granted immediately, there is no need for ABS to send explicit ACK.

-------------------------------  Text End  ---------------------------------------------------

Reference

[1] IEEE 80216m-09/0386, Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control