| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PAPR reduction techniques for the 802.16m |  |
| Date Submitted | 2009-03-06 |  |
| Source(s) | Andrei Malkov, Zexian Li Nokia Joon Chun, Xin Qi Nokia Siemens Networks | E-mail: andrei.malkov@nokia.com zexian.li@nokia.com joon.chun@nsn.com xin.qi@nsn.com |
| Re: | “802.16m AWD text”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Draft AWD Content”. Target topic: PAPR Reduction Technique |  |
| Abstract | This contribution summarizes the most well-known PAPR reduction methods and checks their applicability for 802.16m air interface. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |

PAPR reduction techniques for the 802.16m

# Introduction

This contribution considers well-known peak-to-average power ratio (PAPR) reduction techniques and their applicability to 802.16m air interface. PAPR reduction is very important for increasing power efficiency of the OFDM-based communication system such as 802.16m. High PAPR forces to use high power amplifier to operate with large power back-off in order to avoid non-linear distortions. However the most power efficient operating point is at or near saturation region. PAPR reduction is especially critical for AMS where cost of the device and its power consumption is very important.

PAPR of the transmit signal s(t) is defined according to the following equation:

PAPR=maxt∈0, Ts(t)2Es(t)2

where E denotes the expectation value and T denotes the symbol length. When measured at baseband oversampling factor of at least 4 should be applied for accurate results.

# PAPR reduction methods

PAPR reduction methods have been studied for many years and significant number of methods has been developed.

- Clipping: Clipping naturally happens in transmitter if power back-off is not enough. Clipping leads to a clipping noise and out-of-band radiation. Filtering after clipping can reduce out-of-band radiation, but at the same time it can cause “peak regrowth”. Repeated clipping and filtering can be applied to reduce peak regrowth in expense of complexity. Several methods for mitigation of the clipping noise at receiver were proposed. One of them  reconstructs clipped sample using other samples of oversampled signal.

- Coding: Coding methods include Golay complementary sequences [1], simple block coding scheme [2], complementary block codes (CBC) [3], modified complementary block codes (MCBC) [3] etc. An application of the Gollay Complementary sequences is limited by the fact that they can’t be used with M-QAM modulation. Simple scheme, proposed in [2], relies on lookup tables containing sequences with lower PAPR. This method doesn’t attempt to utilize those sequences for error correction/detection. CBC utilizes complement bits that are constructed from the subset of the information bits. MCBC is a modification of CBC suitable for large number of sub-carriers. Coding methods have low complexity but PAPR reduction is achieved in expense of redundancy  causing data rate loss.

- Partial Transmit Sequences (PTS): A set of sub-carriers of an OFDM symbol is divided into V non-overlapping sub-blocks [4]. Each sub-block undergoes zero-padding by inserting zeroes at those sub-carriers which are already represented in other sub-blocks. All V sub-blocks are transformed into the time-domain by IDFT resulting in V partial transmit sequences denoted by p(k), k=1…V.. The transmit sequence is obtained as a linear combination of PTSs:k=1Vp(k)b(k), where b(k) is a rotation factor. The optimization is performed over the rotation factors b(k) for each OFDM symbol. It is shown that four rotation  angles (b(k)∈±1±j) is already enough for significant peak power reduction.

- Selected mapping (SLM) [5]: This method defines U distinct rotation vectors B(k), k=1…U. The length of these vectors is equal to the number of sub-carriers of the OFDM symbol. The OFDM symbol is multiplied sub-carrier wise by rotation vector B(1) and  transformed into the time-domain by IDFT . Then the same procedure is repeated for vectors B(2)…B(U). At the end the vector with the lowest PAPR is selected for transmission.

- Interleaving [6]: The same data block is interleaved by K different interleavers. K IDFTs of the original data block and modified data blocks are calculated. PAPR of K blocks is calculated. The block with lowest PAPR is transmitted.

- Tone Reservation (TR) [7]: L sub-carriers are reserved for peak reduction purposes. The values of the signals to insert on peak reduction sub-carriers are computed by suitable Linear Programming algorithm.

- Tone Injection (TI) [7]: TI maps one constellation point of the original constellation (for example QPSK) to several constellation points of the expanded constellation (for example 16QAM). PAPR reduction is achieved by choosing constellation points of the expanded constellation.

- Active Constellation Extension (ACE) [8]: ACE modifies original constellation by moving nominal constellation points located on the outer constellation boundaries in the directions that don’t decrease Euclidean distances between constellation points.

- Nonlinear Companding Transform (NCT) [9, 10]: NCT compand original OFDM signal using strict monotone increasing function, which enlarges the small signals and compresses the large signals. Companded signal can be recovered by the inverse function at receiver.

# MIMO specific PAPR reduction methods

For MIMO with NT transmit antennas PAPR is defined according to the following equation:

PAPR=maxn=1..NTmaxt∈0, Tsn(t)2Esn(t)2

Some of the PAPR reduction methods are designed specially for MIMO schemes:

- Directed SLM/PTS [11]: This is a modification of SLM/PTS for two or more antennas. It dynamically reallocates computational complexity of the optimization process to antenna with higher PAPR.

- Cross-Antenna Rotation and Inversion (CARI) [12]: Sub-carriers of an OFDM symbol are partitioned into M non-overlapping sub-blocks: xi=xi(1)...xi(M), where i denote the index of transmit antenna. Different candidates for transmission are obtained by sub-block inversion and rotation over NT transmit antennas (“inversion” here actually means changing the sign). If NT =2 than the first four pairs of candidates are:

x1=x1(1)...x1(M) x2=x2(1)...x2(M),x1=−x1(1)...x1(M) x2=−x2(1)...x2(M),x1=x2(1)...x1(M) x2=x1(1)...x2(M), x1=−x2(1)...x1(M) x2=−x1(1)...x2(M).

The same operation is repeated for the rest of sub-blocks, giving totally 4M pairs of candidates. A pair with the best PAPR is selected for transmission.

- Polyphase Interleaving and Inversion (PII) [13]: This method is similar to CARI except that candidates for transmission are obtained by swapping sub-blocks in frequency domain but not between antennas. The method is meant for SFBC MIMO, unlike CARI, which is meant for STBC MIMO.

# Applicability for 802.16m

In order to be applicable for 802.16m air interface PAPR reduction method should satisfy several requirements:

- No/minimum data rate loss. Data rate loss occurs due to side information transmission or due to redundancy if coding methods are used. If it is not possible to avoid data rate loss completely it should be kept at a minimum level.

- No average power increase. Average power increase is a feature of several PAPR reduction methods such as TI, TR, and ACE.

- No spectrum spillage.

- No BER performance degradation.

- Moderate complexity.

lists PAPR reduction methods and there applicability to SISO/SIMO/MIMO 802.16m modes.

| Method | SISO/SIMO | MIMO | Comments |
| --- | --- | --- | --- |
| Clipping | No | No | Spectral spillage. |
| Coding | No | No | Data rate loss due to redundancy. However some of the coding methods can be used in special cases. One example is using Golay complementary sequence for sounding signals. |
| Partial Transmit Sequences (PTS) | Yes | Yes | “Simplified” version of PTS is applicable for MIMO. “Simplified” means using the same b(k) for all Tx antennas. |
| Selected mapping (SLM) | Yes | Yes | “Simplified” version of SLM is applicable for MIMO. “Simplified” means using the same rotation vectors b for all Tx antennas. |
| Interleaving | No | No | Contradictory to SDD. |
| Tone Reservation (TR) | No | No | Average power increase. Redundancy. |
| Tone Injection (TI) | No | No | Average power increase. |
| Active Constellation Extension (ACE) | No | No | Average power increase. |
| Nonlinear Companding Transform (NCT) | No | No | Inverse transform is not possible for multi-user UL signal due to fading and non-perfect power control. Spectral spillage. |
| Directed PTS/SLM | No | No | Though it is MIMO specific methods but it destroys MIMO precoding. |
| Cross-antenna rotation and inversion (CARI) | No | Yes | This method is meant for STBC. Applicability for SFBC is questionable. |
| Polyphase Interleaving and Inversion (PII) | No | Yes |  |

Table 1 Applicability of the PAPR reduction methods

# Conclusion

This contribution summarizes the most well-known PAPR reduction methods and checks their applicability for 802.16m air interface. As follows from Table 1 most of these methods don’t satisfy at least one requirement, given in section 4. Three methods (PTS, SLM, and PII) satisfy the requirements, however additional effort is needed for their adaptation to 802.16m air interface in order to avoid/minimize side information transmission.  Complexity optimization of these methods is highly desirable as well.

# References

- James A.Davis and Jonathan Jedwab, “Peak-to-mean power control in OFDM, Golay complementary sequences, and Reed-Muller codes”, IEEE Transactions on Information Theory, vol.45, No.7, November 1999.

- Jones A., Wilkinson T., Barton S, “Block coding scheme for reduction of peak to mean envelope power ratio of multicarrier transmission schemes”, IEE Electronic Letters, vol.30, No. 25, 8th December 1994.

- Tao Jiang, Guangxi Zhu, “Complement block coding for reduction in peak-to-average power ratio of OFDM signals”, IEEE Radio Communications, September 2005.

- S.H. Muller and J.B.Huber, “OFDM with reduced peak-to-average power ratio by optimum combination of partial transmit sequences”, IEE Electronic Letters, vol.33, No. 5, 27th February 1997.

- R.Bauml, R.F.H.Fisher and J.B.Huber, "Reducing the peak-to-average power ratio of multicarrier modulation by selected mapping", IEE Electronic Letters, vol.32, No. 22, 24th October 1996.

- A.Jalalath and C.Tellambura, “Reducing the peak-to-average power ratio of orthogonal frequency division multiplexing signal through bit or symbol interleaving”, IEE Electronic Letters, vol.36, No. 13, 22nd June 2000.

- A.Salvekar, C.Aldana, J.Tellado, and J.Cioffi, “Peak-to-average power ratio reductions for block transmission systems in the presence of transmit filtering”.

- B.Crongold, D.Jones, "PAR reduction in OFDM via active constellation extension", IEEE Transactions on Broadcasting, vol.49, iss.3, September 2003.

- Tao Jiang, Weidong Xiang, Paul Richardson, Daiming Qu, and Guangxi Zhu, “On the nonlinear companding transform for reduction in PAPR in MCM signals”, IEEE Transactions on wireless communications, vol.6, No.6, June 2007.

- Xian Huang, Jianhua Lu, Junli Zheng, K.Letaief and Jun Gu, “Companding Transform for reduction in Peak-to-Average Power Ratio of OFDM Signals”, IEEE Transactions on wireless communications, vol.3, No.6, November 2004.

- R.Fisher, and M.Hoch,”Directed selected mapping for peak-to-average power ratio reduction in MIMO OFDM”, IEE Electronic Letters, vol.42, No.22, October 2006.

- M.Tan, Z.Latinovic, and Y.Bar-Ness, “STBC MIMO-OFDM peak-to-average power ratio reduction by cross-antenna rotation and inversion”, IEEE Commun. Letters, vol.9, No.7, 2005.

- Z.Latinovic, and Y.Bar-Ness, “SFBC MIMO-OFDM peak-to-average power ratio reduction by polyphase interleaving and inversion”, IEEE Commun. Letters, vol.10, No.4, 2006.