| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Padding/De-padding Method for 16m CTC |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Kenichi KURI*, Isamu YOSHII*, Lei Huang** * Panasonic Corporation ** Panasonic Singapore Laboratories | Voice: +81-50-3687-6556 E-mail: kuri.kenichi@jp.panasonic.com yoshii.isamu@jp.panasonic.com Lei.Huang@sg.panasonic.com |
| Re: | 802.16m amendment working document Topic: Channel Coding and HARQ |  |
| Abstract | We present padding/de-padding method for CTC (Convolutional Turbo code). |  |
| Purpose | Discussion and adoption for 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Padding / De-padding method for 16m CTC

Kenichi KURI*, Isamu YOSHI*, Lei Huang**

Panasonic Corporation, **Panasonic Singapore Laboratories

# Introduction

The　text proposal of Channel Coding and HARQ DG [1] has defined 149 kinds of FEC block size. If the input PHY burst size to CTC encoder doesn't fit multiple FEC blocks, padding bits shall be added at the end of the input PHY burst [2]. However the receiver of legacy system can't distinguish between padding bits and coded bits before decoding. The spectral efficiency is degraded by the number of padding bit. Therefore Channel Coding & HARQ DG has discussed minimizing the number of padding bit in order to improve the spectral efficiency. If the receiver can detect the PHY burst size and the number of padding bit by signaling(explicit/implicit) before decoding, the transmitter can remove padding bits from transmission data. The receiver can utilize padding bits in order to improve error rate performance.

This contribution explains signaling procedure for detection of padding bits and removing padding bits (i.e. de-padding). The overview of padding method in Channel Coding & HARQ DG is described in section 2. Section 3 explains the signaling procedure for detection for padding bits. De-padding method and related simulation results are treated in section 4 and 5 respectively. Section 6 compares between proposal and conventional schemes.

# Overview of padding methods

Channel Coding & HARQ DG has discussed padding insertion. There are two alternatives of padding insertion, only padding in MAC, and padding in both MAC and PHY shown in Figure 1 and Figure 2.

Figure 1 Alternative-1 of only padding in MAC

Figure 2 Alternative-2 of padding in both MAC and PHY

# Signaling procedure for detection of padding bits

Potentially, there are 4 signaling procedures for detection of padding bits. Signaling information (i.e. Assignment A-MAP IE) has discussed in DL PHY Control DG [4]. Assignment A-MAP IE includes parameters related to MCS and Resource Allocation. Each signaling procedures are described as follows.

Signaling procedure-1:

Step-1: detecting PHY burst size by referring parameters (MCS and Resource allocation size)

Step-2: decoding FEC blocks

Step-3: referring to MAC Header of decoded PHY burst and detecting the number of both PHY and MAC padding bit

This signaling procedure is applied to Alternative-1 and Alternative-2.

Signaling procedure-2:

Step-1 : detecting PHY burst size by referring parameters(MCS and Resource allocation size)

Step-2 : detecting the number of PHY padding bit by referring PHY padding indication parameter

This signaling procedure is applied to Alternative-2.

Signaling procedure-3: [5]

Step-1 : detecting PHY burst size by referring parameters(Minimal index and Size offset)

Step-2 : decoding FEC blocks

Step-3 : referring to MAC Header of decoded PHY burst and detecting the number of padding bit

This signaling procedure is applied to Alternative-2.

Signaling procedure-4:

Step-1: referring to Table defined by MCS and Resource allocation size and detecting PHY burst size and the number of padding bit

This signaling procedure is applied to Alternative-2. Figure 3 shows relation between table and MAC PDU size. This table is defined by parameters which are MCS and Resource allocation size. Each value in this table indicates MAC PDU size which doesn’t include padding bits and CRC bits.

Figure 3 Relation between table and MAC PDU size

# De-padding method

There are four alternatives of de-padding method corresponding to Fig.1,2,3 and 4 (Full de-padding method, Partial de-padding method-1(A, B, Y1, W1), Partial de-padding method-2(A, B), No de-padding method).

Figure 4 Full de-padding method

Figure 5 Partial de-padding method-1 (A, B, Y1, W1)

Figure 6 Partial de-padding method-2 (A, B)

Figure 7 No de-padding method

Figure 8 shows an example of trellis diagram based on CTC tail-biting. This example assumes 4 information bits and 4 padding bits inserted to the front of information sequence. The parity bits related to padding bits are unknown because of tail-biting. Therefore full de-padding method and partial de-padding method-1 may be degraded by de-padding parity bits related to padding bits compared to partial de-padding method-2 and no de-padding method. The performance comparison among four de-padding methods is indicated in the next section.

Figure 8 example of trellis diagram based on CTC tail-biting

# Simulation results

Table 1 provides a list of simulation parameters. Figure 9 shows coded bits and the puncturing condition of each de-padding method. All de-padding methods read out 3840 bits from circular buffer, these selection bits are different according to each de-padding method.

Table 1 Simulation parameters

| Parameter | Value |
| --- | --- |
| Number of information bits for packet | 1920 [bit] |
| Number of padding bit | 64 [bit] |
| coding rate (# of transmission bits) | R=1/2 (3840bit) |
| modulation | QPSK |
| Channel model | AWGN |

Figure 9 puncturing condition of each de-padding method

Figure 10 shows the block error rate (BLER) vs. SNR performance comparison among four de-padding methods. The performance of full de-padding method and partial de-padding method-1 are almost same and better than partial de-padding method-2 and no de-padding method. We found that removing parity bits related to padding bits doesn’t affect error rate performance from this simulation result. Considering the complexity of de-padding parity bits (Y2, W2), the partial de-padding method-1 is simpler than full de-padding method. Therefore we propose that the partial de-padding method-1 are applied to rate-matching block for 16m CTC.

Figure 10 Block error rate vs. Eb/N0 performance comparison among four de-padding methods

# Comparison between proposal and conventional schemes

Table 2 indicates comparison among each scheme in terms of (1) Alternative of padding insertion, (2) signaling procedure, (3) signaling overhead, (4) advantage, and (5) drawback. There are 5 schemes as follows. Schemes 1, 2, 3 and 4 are conventional schemes. Our proposal corresponds to Scheme 5.

Schemes 1, 2 and 3 can’t apply de-padding, since the receiver can't distinguish between padding bits and coded bits before decoding. However, Schemes 4 and 5 can utilize the padding bits to improve error rate performance. Scheme 5 is superior to Scheme 4 in terms of signaling overhead.

Table 2 Comparison among each schemes

|  | Scheme-1 | Scheme-2 | Scheme-3 | Scheme-4 | Scheme -5 |
| --- | --- | --- | --- | --- | --- |
|  | No Deppading |  |  | Depadding |  |
| (1) Alternative of padding insertion | Alternative-1 | Alternative-2 | Alternative-2 | Alternative-2 | Alternative-2 |
| (2) Signaling procedure | Signaling procedure-1 | Signaling procedure-1 | Signaling procedure-3 | Signaling procedure-2 | Signaling procedure-4 |
| (3) signaling overhead | MCS=5bit resource allocation=7bit | MCS=5bit resource allocation=7bit | Minimal size = 8bit Size offset=5bit | MCS=5bit resource allocation=7bit Padding indication=3bit※ | MCS=5bit resource allocation=7bit |
| (4) Advantage | Simple Operation | Legacy Operation |  | Effect of de-padding | Effect of de-padding |
| (5) Drawback | Not obtain effect of deppading. | Not obtain effect of deppading | Not obtain effect of deppading | Increase signaling overhead |  |

※ The maximum of the FEC block resolution is 64 and MAC PDU size is calculated by byte. Therefore, 3bits padding indication parameter is required.

# Conclusion

In this contribution signaling procedure for padding insertion and de-padding method for rate-matching block are presented. We propose Alternative -2 as padding insertion method and partial depadding method with signaling procedure-4.

---------------------------- Begin Proposed Text --------------------------------------------------------------------------

15.x. Channel coding and HARQ

15.x.1 Channel coding

Channel coding procedures for downlink and uplink data channel are shown in Figure 15.x.1-1.

Figure 15.x.1-1 Channel coding procedure

15.x.1.1 Padding insertion

If the input PHY burst size to CTC encoder doesn't fit multiple FEC blocks, padding bits shall be added at the end of the input PHY burst.

15.x.1.2 Burst CRC encoding

15.x.1.3 Randomization

15.x.1.4 Burst partition

15.x.1.5 FEC block CRC encoding

15.x.1.6 FEC encoding

15.x.1.6.1 Convolutional turbo codes (CTCs)

15.x.1.6.1.1 CTC encoder

15.x.1.6.1.2 CTC interleaver

15.x.1.6.1.3 Determination of CTC circulation sates

15.x.1.6.1.4 Bit separation

15.x.1.6.1.5 Subblock interleaving

15.x.1.6.1.6 Bit grouping

15.x.1.6.1.7 Resource segmentation

15.x.1.6.1.8 Bet selection and repetition

If the FEC block includes some padding bits, the padding bits (A, B) and related parity bits (Y1, W1) are skipped from circular buffer in selection of coded bits.

---------------------------End of Text Proposal ---------------------------------------------------------------------------

# Reference

- C802.16m-09/0510, “Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment”

- C802.16m-09/0312, “Proposed Amendment Text on Channel coding”

- IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access” Dec.2008

- C802.16m-09/558, “Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment”

- C802.16m-09/0497, “Proposal for 802.16m amendment text on burst size signaling”