| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |
| --- | --- |
| Title | Proposed Text on Section 15.3.5.3.1 CRU/DRU allocation of 802.16m Amendment |
| Date Submitted | 2009-03-02 |
| Source(s) | HanGyu Cho, Seunghyun Kang, Jin Sam Kwak E-mail : hgcho@lge.com LG Electronics |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0012. ”Call for Contribution on Project 802.16m Amendment Working Document (AWD) Content” Target topic: Call for Comments on Amendment Working Document |
| Abstract | The contribution proposes the text on CRU/DRU allocation section (15.3.5.3.1) to be included in the 802.16m amendment. |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |

Proposed Text on Section 15.3.5.3.1 CRU/DRU allocation of 802.16m Amendment

HanGyu Cho, Seunghyun Kang, Jin Sam Kwak

LG Electronics

# 1. Introduction

- In the current IEEE 802.16m Amendment Working Document (IEEE 802.16m-09/0010), the CRU/DRU allocation sequence in the section 15.3.5.3.1 is TBD.

- This contribution suggests a CRU/DRU allocation sequence which performs cell-specific permutation of PRUs remained after assigning subband-based CRUs.

# 2. Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------- Text Start---------------------------------------------------------

Adopt only the modified text in below.

## Advanced Air Interface

### Physical Layer

#### Downlink Physical Structure

##### Cell-Specific Resource Mapping

###### CRU/DRU Allocation

The partition between CRUs and DRUs is done on a sector specific basis. [4]-bit subband-based CRU allocation size (CASSB,i) field is sent in the BCH for each allocated frequency partition. CASSB,i indicates the number of allocated subband-based CRUs for partition FPi in the unit of subband size. A [6]-bit miniband-based CRU allocation size (CASMB) is sent in the BCH only for partition FP0.

The number of CRUs in each frequency partition is denoted by LCRU,FPi., where

LCRU,FPi=CASSB,i⋅N1+CASMB      for i=0CASSB,i⋅N1     for 0<i<FPCT                                (182)

- The number of DRUs in each frequency partition is denoted by LDRU,FPi, where

LDRU,FPi = FPSi*N2 - LCRU,FPi   for 0 ≤ i <FPCT

The mapping of PRUFPi to CRUFPi is given by

CRUFPi[j]=PRUFPi[j]      for 0≤j<CASSB,i⋅N1PRUFPi[k+CASSB,i⋅N1]      for CASSB,i⋅N1≤j<LCRU,FPi      for  0≤i<FPCT(183)

where k=s[j−CASSB,i⋅N1].  is the CRU/DRU-allocation sequence (TBD), and 0≤s[j]<FPSi⋅N2−CASSB,i⋅N1 and is given by

s[j]=j⋅DCDA+j⋅GCD(L,DCDA)L+OCDAmodL    for 0≤j<CASMB⋅N2(184)

where

DCDA=Cell IDmodL−1+1

OCDA=Cell ID(L−1)+1

where

L=FPSi⋅N−CASSB,i⋅N1

The mapping of PRUFPi to DRUFPi is given by

DRUFPi[j]=PRUFPi[k+CASSB,i⋅N1]       for 0≤i<FPCT,    0≤j<LDRU,FPi    (185)

where k=sc[j]. sc[]is the sequence which is obtained by renumbering the remainders of the PRUs, which are not allocated for CRU, from 0 to LDRU,FPi-1.

------------------------------------------------------------Text End---------------------------------------------------------