| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Connection Management |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Sang-Heon Shin Bong Ho Kim Yerang Hur POSDATA | Voice: +1-408-986-1140 E-mail:shshin@posdata.co.kr bhkim@posdata-usa.com yehur@posdata-usa.com |
| Re: | 802.16m AWD |  |
| Abstract | This contribution proposes the amendment text for 15.2.x Connection Management. |  |
| Purpose | To be discussed and adopted by 802.16 TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Connection Management

Sang-Heon Shin, Bong Ho Kim, Yerang Hur

POSDATA

- Introduction

This contribution proposes the bi-directional connection management method by pairing DL and UL connections. Most of Internet services require bi-directional data delivery capability over the network. For example, VoIP consumes same bandwidth on downlink and uplink direction. HTTP or FTP that uses TCP as a transport layer protocol also requires bi-directional connection one way for the user/application data and the other direction for the acknowledgements, although the required bandwidth is usually heavily asymmetric on downlink and uplink direction. Therefore, it is more natural to perform bi-directional service flow or the pair of connections management on both directions.  However, service flows or connections are independently established and managed in 802.16e. So, we propose the DL/UL connection paring method for the bi-directional service flow management.

# Bi-directional SF management

## 2.1 DL/UL connection pairing

Bi-directional service flow can be established by a pair of two uni-directional connections (downlink and uplink). A counterpart connection is assigned at the creation time of the initial connection, if the connection belongs to a bi-directional service flow, and each of the uni-directional connection should have information about the counterpart connection to be pared with.

Different QoS parameters may be assigned to each connection of the bi-directional service flow.

Figure 1 DL/UL pairing

## 2.2 Bi-directional service flow setup

A bi-directional service flow management may reduce the connection establishment time. If connections are established separately on downlink and uplink, DSA message exchange procedure should be performed independently for each of the connections. The bi-directional service flow establishment, however, requires only one DSA message exchange procedure for the paired DL/UL connections. It will reduce the overall service flow establishment time, and decrease the amount of the management traffic over the air.

Figure 2 Bi-directional service flow setup with DL/UL pairing

Another benefit of the bi-directional service flow establishment is to eliminate the unnecessary service flow creation and deletion caused by the unavailability of the resource for the counterpart service flow which is required for a service. If the counterpart service flow cannot be established, then the initial service flow creation request may be rejected at the early stage.

## 2.3 Resource management using paired connections

The paired DL/UL connections may be used for an efficient bandwidth allocation. In many cases, traffic on one direction is associated with the other direction. In case of a TCP connection, TCP acknowledgement messages are generated on the opposite direction after the burst of TCP traffic. So it is predictable that the generation of UL traffic before receiving a BWR message by observing DL traffic pattern if the major traffic is on DL direction. It may be used to allocate UL bandwidth preemptively to reduce the uplink delay or BWR collision.

Figure 3 shows an example of the preemptive bandwidth grant on paired UL connection for TCP acknowledgement. User data are delivered over TCP on downlink direction. The BS may be able to predict the acknowledgement messages for the TCP segments will be generated after the some processing time. Although BW request from MS is not received yet, BS may send a bandwidth grant for the expected TCP acknowledgement. In this way, TCP acknowledgment can be delivered without delay or collision on UL direction.

Figure 3 BW allocation on paired connection for TCP Ack

- Proposed Text for Connection Management

----------------------------------------  Start of the proposed text  ------------------------------------------

15.2.x Connection Management

15.2.x.x Transport connections

Transport connection is uni-directional and established with unique FID assigned during service flow establishment procedure. Each admitted/active service flow is uniquely mapped to a transport connection. Transport connection is released when the associated service flow is removed. To reduce bandwidth usage, the ABS and AMS may establish/change/release multiple connections using a single message transaction on a management connection.

In this case, QoS parameter sets of the management message establishing the transport connection may include the associated FID for indicating the connection is associated with another transport connection in the other direction. Figure x describes the procedure of establishing transport connections with pairing downlink and uplink connections.

Figure x Service flow establishment with and without pairing DL and UL connections.

Transport connections can be pre-provisioned or dynamically created. Pre-provisioned connections are those

established by system for an AMS during the AMS network entry. On the other hand, ABS or AMS can create

new connections dynamically if required. A connection can be created, changed, or torn down on demand.

-------------------------------------------  End of the proposed text   -------------------------------------------

- References

[1] IEEE 802.16m-08/003r7, IEEE 802.16m System Description Document, Feb 2009.

[2] IEEE 802.16m-09/0010, IEEE 802.16m Amendment Working Document, Jan 2009.

[3] IEEE P802.16 Rev2/D9, Draft Standard for Local and Metropolitan Area Networks, Part 16: Air Interface 
for Broadband Wireless Access, Jan 2009.