| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Control Section for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Dongcheol Kim,Wookbong Lee, Jin Sam Kwak, Yeong-Hyeon Kwon, Sungho Moon, Jong Young Han, HanGyu Cho LG Electronics | Voice : +82-31-450-7919 E-mail: dongcheol.kim@lge.com |
| Re: | “802.16m AWD ”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Draft AWD Content”. Target topic: “Power control” |  |
| Abstract | The contribution proposes the text on Power Control to be included in the 802.16m amendment |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Power Control Section for the IEEE 802.16m Amendment

Dongcheol Kim, Wookbong Lee, Jin Sam Kwak, Yeong-Hyeon Kwon,

Sungho Moon, Jong Young Han, HanGyu Cho

LG Electronics

# Introduction

This contribution includes the proposed text on the power control section for the IEEE 802.16m Amendment. The technical text is inherited from the 802.16m SRD [1], the IEEE 802.16m SDD [2], and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

# Proposed Open-Loop Power Control (OLPC)

  - Proposed OLPC formula

The main concept of the OLPC proposed here is to set MS Tx power such that it meets the target SINR given by the serving BS. In order to capture new features adopted in 16m, the proposed OLPC has the following two features: (1) It controls the uplink transmit power according to the number of streams to support the uplink collaborative spatial multiplexing (CSM). (2) It controls the interference to other cells/sectors efficiently combined with fractional frequency reuse (FFR) scheme which is well defined in the current SDD [2].

Power control equation of OLPC is proposed as equation (X-0) below. The transmit power level per stream for all uplink physical channel transmission is set by:

(X-0)

Where

(X-1)

Where

- : Transmit power level per stream at an AMS for achieving a given

- : Path loss estimation at an AMS for the serving cell.

- : Path loss estimation at an AMS for the strongest interfering cell.

- NI : Estimated average power level (dBm) of noise and interference per a subcarrier at the serving cell. NI level for each frequency partition is broadcasted through the S-SFH.

- : The SINR target is for transmitting the data or control channel. In case of transmitting data channel, the target SINR is set as in Table 4. In case of transmitting the control channel, target SINR is set as in Table .

- : The correction term for AMS-specific power offset. It is controlled by ABS with power control messages through UL A-MAP for mode change between OLPC and CLPC.

- : Transmit power level adjustment controlled by an AMS.

- : The number of streams indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams per user. In case of CSM, this value shall be set to Mt_A where Mt_A is the aggregated number of streams. In case of control channel transmission, this value shall be set to one.

- : The target IoT level to other sector(s), which is broadcasted by ABS through the S-SFH and is different for each frequency partition. More details on target IoT for each frequency partition is referred to section [TBD; 20.1 in SDD] and 15.3.6.

Table 1 SINR target for control channel

| Uplink Control Channel | SINR_Target |
| --- | --- |
| P-FFCH | [TBD] |
| S-FFCH | [TBD] |
| HARQ Channel | [TBD] |
| Sounding Channel | [TBD] |
| … | … |

  - Consideration on the number of multiplexed streams ()

The legacy power control mechanism has been based on the single user transmission per allocated resource units. In order to extend the UL PC operation into the multi-user support, it is required to control the transmission power such that the interference level is maintained to the level of single user transmission when using multi-user transmission. Table 1 compares three cases - SIMO, CSM with and without consideration on the number of streams – in terms of system throughput, cell edge-user throughput, and IoT level.

In case of CSM, it is assumed that there are two users, each with one stream. For the CSM case without consideration on the number of streams, both system throughput and the cell edge-user throughput are significantly decreased due to the increasing IoT level. When considering the number of streams in power control, multi-user MIMO gain can be efficiently obtained by keeping the IoT value at the level of SIMO case.

Table 1 Throughput/IoT Level comparison between SU-MIMO and MU-MIMO

|  |  | SIMO | CSM without consideration on the number of streams | CSM with consideration on the number of streams |
| --- | --- | --- | --- | --- |
| System throughput (spectral efficiency) |  | 2.5752 Mbps (0.70 bps/Hz) | 2.24 Mbps (0.602 bps/Hz) | 2.9231 Mbps (0.79 bps/Hz) |
| 5%-tile user throughput (spectral efficiency) |  | 57.79 kbps (0.0156 bps/Hz) | 51.71 kbps (0.0140 bps/Hz) | 64.43kbps (0.0174 bps/Hz) |
| IoT level | Mean | 5.959 | 6.3417 | 5.9195 |
|  | Std. | 0.6836 | 0.7505 | 0.5401 |

  - Consideration on target IoT to other cells

According to (X-0), proposed power control performs power adjustment considering target IoT such that transmit power does not exceeds the power level meeting the target IoT to other cells.

Table 3 compares power control schemes with and without power adjustment considering target IoT. When using the proposed power adjustment considering target IoT, both average user throughput and cell edge-user throughput gain are obtained.

Table 3 Throughput comparison w/ and w/o power adjustment considering target IoT

|  | Without power adjustment | With power adjustment considering target IoT |
| --- | --- | --- |
| Average user throughput (spectral efficiency) | 311.40kbps (0.830 bps/Hz) | 324.85kbps (4.3 %↑) (0.866 bps/Hz) |
| 5% tile user throughput (spectral efficiency) | 46.84 kbps (0.012 bps/Hz) | 73.58 kbps (57 %↑) (0.020 bps/Hz) |

# Proposed Closed-Loop Power Control (CLPC)

The transmit power level per stream for all uplink physical channels is set by:

(Y)

Where

- : Transmit power level per stream at an AMS.

- : The latest transmitted maximum power level among different uplink physical channels transmitted concurrently.

- : The difference of the desired SINRs between the previous and new MCS levels for a uplink physical channel. Desired SINR for control channel is specified in Table . Desired SINR for each MCS level is specified in Table 4.

- : Power difference sent by the corresponding ABS through PC A-MAP.

- : The number of streams indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams per user. In case of CSM, this value shall be set to Mt_A where Mt_A is the aggregated number of streams.  In case of control channel transmission, this value shall be set to one.

Table 4 Desired SINR level for MCS level

| MCS level | SINR_Target |
| --- | --- |
| 0 | [TBD] |
| 1 | [TBD] |
| 2 | [TBD] |
| 3 | [TBD] |
| … | … |

# References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Jan. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Text proposal for the 802.16m amendment

============================== Start of Proposed Text =================================

# XX.10 Power Control

## xx.10.1 Downlink Power Control

The ABS shall be capable of controlling the transmit power per subframe and per user. With downlink power control, each user-specific information or control information would be received by the AMS with the controlled power level.

The per pilot tone power and the per data tone power can jointly be adjusted for adaptive downlink power control. In the case of dedicated pilots this is done on a per user basis and in the case of common pilots this is done jointly for the users sharing the pilots.

### xx.10.2. Uplink Power Control

Uplink power control is performed to compensate the path loss, shadowing, fast fading and implementation loss.

Uplink power control is also used to control inter-cell and intra-cell interference level. Uplink power control consists of two different modes: open-loop power control (OLPC) and closed-loop power control (CLPC). ABS transmits necessary information through S-SFH for noise and interference level, and target IoT or PC A-MAP for CLPC to AMSs to support uplink power control. The parameters of power control algorithm are optimized on system-wide basis by the ABS, and broadcasted periodically or triggered by events. Uplink power control for calculating the power of a stream considers the number of streams multiplexed in the same allocated resource.

#### XX.10.2.1 Open-loop power control (OLPC)

The OLPC compensates the long term channel variations such as shadowing and pathloss, and implementation loss. The AMS shall determine the transmit power based on the transmission parameters sent by the ABS.

The transmit power level per stream for all uplink physical channel transmission is set by:

(X-0)

Where

(X-1)

Where

- : Transmit power level per stream at an AMS for achieving a given

- : Path loss estimation at an AMS for the serving cell.

- : Path loss estimation at an AMS for the strongest interfering cell.

- NI : Estimated average power level (dBm) of noise and interference per a subcarrier at the serving cell. NI level for each frequency partition is broadcasted through the S-SFH.

- : The SINR target is for transmitting the data or control channel. In case of transmitting data channel, the target SINR is set as in Table Y. In case of transmitting the control channel, target SINR is set as in Table X.

- : The correction term for AMS-specific power offset. It is controlled by ABS with power control messages through UL A-MAP for mode change between OLPC and CLPC.

- : Transmit power level adjustment controlled by an AMS.

- : The number of streams indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams per user. In case of CSM, this value shall be set to Mt_A where Mt_A is the aggregated number of streams. In case of control channel transmission, this value shall be set to one.

- : The target IoT level to other sector(s), which is broadcasted by ABS through the S-SFH and is different for each frequency partition. More details on target IoT for each frequency partition is referred to section [TBD; 20.1 in SDD] and 15.3.6.

Table X SINR target for control channel

| Uplink Control Channel | SINR_Target |
| --- | --- |
| P-FFCH | [TBD] |
| S-FFCH | [TBD] |
| HARQ Channel | [TBD] |
| Sounding Channel | [TBD] |
| … | … |

#### XX.10.2.2 Closed-loop power control (CLPC)

The CLPC compensates channel variation by adjusting AMS transmit power through PC A-MAP from ABS.

To maintain at the ABS a power density consistent with the modulation and FEC rate used by each AMS, the ABS changes the AMS TX power, through PC A-MAP, as well as the AMS-assigned modulation and FEC rate.

The transmit power level per stream for all uplink physical channels is set by:

(Y)

Where

- : Transmit power level per stream at an AMS.

- : The latest transmitted maximum power level among different uplink physical channels transmitted concurrently.

- : The difference of the desired SINRs between the previous and new MCS levels for a uplink physical channel. Desired SINR for control channel is specified in Table . Desired SINR for each MCS level is specified in Table Y.

- : Power difference sent by the corresponding ABS through PC A-MAP.

- : The number of streams indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams per user. In case of CSM, this value shall be set to Mt_A where Mt_A is the aggregated number of streams. In case of control channel transmission, this value shall be set to one.

Table Y Desired SINR level for MCS level

| MCS level | SINR_Target |
| --- | --- |
| 0 | [TBD] |
| 1 | [TBD] |
| 2 | [TBD] |
| 3 | [TBD] |
| … | … |

============================== End of Proposed Text =================================

# ANNEX: Simulation Assumptions

Table 5. Simulation Assumptions for Table 1 (1/2)

| Topic | Baseline System Assumptions |
| --- | --- |
| Basic modulation | QPSK, 16QAM |
| Duplexing scheme | TDD |
| Subchannelization | PUSC |
| Resource Allocation Granularity | PUSC: 1 slot (1 slot = 1 subchannel x 3 OFDMA symbols) |
| Downlink pilot structure | Specific to subchannelization scheme PUSC |
| Multi-antenna Transmission format | SIMO (1x2), CSM |
| Receiver Structure | MRC for SIMO, MMSE for CSM |
| Data Channel coding | Convolutional Turbo Coding (CTC) |
| Control Channel Coding | CDMA Codes (PUSC 2 symbols) for Initial Ranging and Handover, CDMA Codes (PUSC 1 symbol) for Periodic Ranging and Bandwidth Request, CQICH (6 bits) |
| Scheduling | Proportional Fairness for full buffer data only |
| Link adaptation | QPSK(1/2) with repetition 1/2/4/6, QPSK(3/4), 16QAM(1/2), 16QAM(3/4) |
| Link to System Mapping | MMIB |
| H-ARQ | Chase Combining Synchronous, Nonadaptive, 1 frame ACK/NACK delay, Maximum 4 HARQ Retransmissions, initial target PER of 20% |
| Power Control | Simulation scenario specific |
| Interference Model | Frequency selective interference model for PUSC/AMC, no interference awareness at receiver |
| Frequency Reuse | 3 Sectors with Frequency Reuse of 1 |
| Control signaling | Initial Ranging, Periodic Ranging, Handover Ranging, Bandwidth Request, Fast Feedback/CQI Channel, Sounding |

Table 6. System-level Simulation Assumptions for Table 1 (2/2)

| Scenario/ Parameters | Baseline Configuration (Calibration & SRD) | Urban Macrocell |
| --- | --- | --- |
| Requirement | Mandatory | Mandatory |
| Site-to-Site distance | 1.5 km | 0.5 km 1.5 km |
| Carrier Frequency | 2.5 GHz | 2.5 GHz |
| Operating Bandwidth | 10 MHz for TDD | 10 MHz for TDD |
| BS Height | 32 m | 32 m |
| MS Tx Power | 23 dBm | 23 dBm |
| MS Height | 1.5 m | 1.5 m |
| Penetration Loss | 10 dB | 10 dB |
| Path Loss Model | Loss (dB) = 130.62+37.6log10(R) (R in km) | Loss (dB) = 35.2+35log10(R)+ 26log10(f[GHz]/2) (R in m) |
| Lognormal Shadowing Standard Deviation | 8 dB | 8 dB |
| Correlation distance for shadowing | 50m | TBD [50m] |
| Mobility | 0-120 km/hr | 0-120 kmph |
| Channel Mix | ITU Ped B * 3 km/hr – 60% ITU Veh A 30 km/hr – 30% ITU Veh A 120 km/hr – 10% | Mix or specific |
| Spatial Channel Model | ITU with spatial correlation a) Uncorrelated b) Correlated (BS Correlation Coefficient = 0.5) | [Correlation based implementation 3.2.7] |

Table 7. Simulation Assumptions for Table 2

| Parameters | Baseline Configuration |
| --- | --- |
| Site-to-Site Distance | 1.5km |
| Carrier Frequency | 2.5GHz |
| Operating Bandwidth | 10MHz for TDD |
| BS Height | 32m |
| MS MAX Tx Power | 23dBm |
| MS Height | 1.5m |
| Penetration Loss | 10dB |
| Pathloss model | Loss(dB)=130.62+37.6log10(R)+cablelossR=distance in km, cableloss 2dB |
| Lognormal Shadowing Standard Deviation | 8dB |
| Correlation Distance for Shadowing | 50m |
| Channel Type | ITU Ped B |
| Mobility | 3km/hr |
| Spatial Channel Model | ITU with spatial correlation |
| Basic Modulation | QPSK, 16QAM |
| Duplexing Scheme | TDD |
| Subchannelization | CRU Allocation |
| Multi-antenna Transmission Format | 1x2 SIMO |
| Receiver Structure | MMSE |
| Data Channel Coding | Convolutional Turbo Coding (CTC) |
| Scheduling | PF for full buffer data only 10 active users / sector Fixed control overhead of 3 symbols, 15 symbols for data, 4 partitions of 48 sub-channels |
| Link Adaptation | 7 levels, QPSK(1/2) with repetition 1/2/3/4, QPSK(3/4), 16QAM(1/2), 16QAM(3/4) |
| Link to System Mapping | MMIB |
| HARQ | Adaptive HARQ, Maximum 4 retransmissions, retransmission duration 3 frames |
| Interference Model | Average interference on used band |
| Frequency Reuse Factor(FRF) | 3 sectors with FRF=1, FRF 1/3 for FFR operation |
| Users dropped | Uniformly in entire cell |