| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Inter-RAT Handover |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Roberto Albanese, Zexian Li Nokia Haihong Zheng, Shashikant Meheshwari, Yousuf Saifullah NSN | roberto.albanese@nokia.com haihong.zheng@nokia.com |
| Re: | “802.16m AWD text”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Draft AWD Content”. Target topic: HO |  |
| Abstract | The contribution proposes text to the 802.16m Amendment for Inter-RAT Handover. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Inter-RAT Handover

- Introduction

This contribution proposes amendment text for inter-RAT handover.

- Text proposal for the 802.16m amendment

================= Start of Proposed Text ========================

- 15.2.x Handover

15.2.x.1 Inter-RAT handover support

15.2.x.1.1 Network topology acquisition

The ABS may advertise presence and/or information about other RATs in order to assist the AMS with network discovery and selection. The WirelssMAN-OFDMA advanced system provides a mechanism for AMS to obtain information from ABS about other access networks in its vicinity, either by making a query or listening to system information broadcast. The ABSs may also indicate the boundary area of the IEEE 802.16m network by advertising a network boundary indication. Upon receiving the indication, the AMS may perform channel measurement on the other RATs.

15.2.x.1.2 Network topology advertisement

In the overlapping coverage area of the WirelessMAN-OFDMA Advanced system and another RAT, the ABS may broadcast or multicast the indication and/or the system information of neighboring target RATs, which may be used by AMS to access the target network.

AMS’s capabilities for inter-RAT operation shall be negotiated with ABS during network entry (e.g. support for the various RATs, single or dual radio operation). The access network information may also include operator preferences based on the available RATs. The information may be restricted to specific access technologies, based on the AMS’s current location and preferences.

15.2.x.1.3 Generic inter-RAT HO procedure

The WirelessMAN-OFDMA Advanced system provides mechanisms for conducting inter-RAT measurements and reporting; they are FFS. Further, it may forward handover related messages with other access technologies such as IEEE 802.11, 3GPP and 3GPP2. The specifics of these handover messages may be defined elsewhere, e.g. IEEE 802.21.

15.2.x.1.4 Enhanced inter-RAT HO procedure

15.2.x.1.4.1 Dual RF Support

An AMS with dual RF may connect to both an ABS and a BS operating on other RAT simultaneously during handover. The second RF is enabled when inter-RAT handover/measurement is initiated. The network entry and connection setup processes with the target BS are all conducted over the secondary radio interface. The connection with the serving ABS is kept alive until handover completes.

In order to reduce the battery consumption, the second radio is only switched on when needed. After detecting the presence indication of another RAT, the dual mode AMS may switch on the secondary radio to scan for the different RATs, based on its PRL (Preferred Roaming List) and other policies defined.

15.2.x.1.4.2 Single RF Support

An AMS with a single RF may connect to only one RAT at a time. The AMS will use the source RAT to prepare the target RAT system. MAC management messages directed to the target RAT are forwarded through the source system to the target system. Once target RAT preparation is complete the AMS may switch from source RF to target RF and complete network entry in target RAT.

============================== End of Proposed Text ===============

- References

[1] IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access Systems,” Jan. 2009.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”