| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed IEEE 802.16m Amendment Text on Handover with Multi-Carrier Support |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Kelvin Chou, Yih-Shen Chen, I-Kang Fu and Paul Cheng MediaTek Inc. | Kelvin.Chou@mediatek.com |
| Re: | New Topic for 802.16m AWD - Handover |  |
| Abstract | This contribution proposes IEEE 802.16m AWD text on the HO procedure. |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed IEEE 802.16m Amendment on Handover with Multi-Carrier Support

Kelvin Chou, Yih-Shen Cheng, I-Kang Fu and Paul Cheng

MediaTek Inc.

- Introduction

This contribution proposes amendment text on the handover procedures for the WirelessMAN-OFDMA Advance System with multi-carrier support.

The content of our proposed text (a ToC) is summarized as follows:

------------------------------------------------------------------------------------------------------------------

15.2.x MAC handover procedures

15.2.x.4 Handover process with multi-carrier support

15.2.x.4.1 AMS scanning of target carriers

15.2.x.4.2 HO preparation and execution

------------------------------------------------------------------------------------------------------------------

- Proposed Text

----------------------------------------------------------- Text Start ---------------------------------------------------------------

15.2.x MAC handover procedures

15.2.x.4 Handover process with multi-carrier support

The multi-carrier handover (MCHO) is defined as the handover process which involves multiple radio carriers. The sub-clause defines the process of HO with multi-carrier support.

15.2.x.4.1 AMS scanning of target carriers

The AAI_NBR-ADV message shall carriers neighbor ABS’s multi-carrier configuration information to facilitate AMS’s scanning of neighbor ABS’s fully configured carriers. The scanning follows the procedures defined in section 15.2.x.1.2 except that AMS scans each fully configured carrier of the neighbor ABSs as advertised in the AAI_NBR-ADV. The AMS may also scan inactive carriers of the serving ABS. Figure xx4 illustrates the example message flows for neighbor ABS advertisements and scanning of fully configured carriers of serving and neighbor ABSs.

[Note: This figure is modified from Rev2/D8, Annex D, Figure D.1]

Figure xx4 — Example message flows for neighbor ABS advertisements and scanning of fully configured carriers of serving and neighbor ABSs.

An AMS capable of concurrently processing multiple radio carriers may perform scanning with neighbor ABSs using one or more of its available radio carriers while maintaining normal operation with the serving ABS on the primary carrier and secondary carriers. In this case, the AMS may inform the serving ABS through AAI_SCN-REQ its carriers to be assigned for scanning operations to avoid resource allocation on those carriers, as illustrated in Figure xx5.

[Note: This figure is modified from Rev2/D8, Annex D, Figure D.1]

Figure xx5 — Example message flows for scanning of fully configured carriers of serving and neighbor ABSs using available radio carriers of the AMS

15.2.x.4.2 Multi-carrier handover (MCHO) process

[Note: The following paragraphs are modified from SDD]
The multi-carrier handover (MCHO) is defined as the handover process which involves multiple radio carriers during handover initiation, preparation or execution.

The AMS may get the information on OFDMA multi-carrier capabilities of one or more possible target ABSs from the AAI_BSHO-CMD message sent by the serving ABS.

An AMS in multi-carrier operation follows the handover operation defined in 15.2.x.2. MAC management messages in relation with handover preparation and initiation between the AMS and the serving ABS are transmitted over the primary carrier of the AMS.

If the HO_Reentry_Mode in AAI_BSHO-CMD message is set to 1, the AMS performs network reentry with the target ABS on the assigned fully configured carrier at action time while continuously communicating with serving ABS until completion of network reentry at the target ABS. Multiplexing of network reentry signaling with target ABS and communications with serving ABS is done via multiple radio carriers, i.e. the AMS performs network reentry on one of its available radio carriers while maintaining data communication with the serving ABS on the serving primary and secondary carriers. In this case, the AMS may inform the serving ABS through AAI_HO-IND the carrier to be assigned for network reentry operations to avoid resource allocation on those carriers. Figure xx6 shows an example call flow of MCHO in which the AMS maintain data communication with the serving ABS on one radio carrier while performing network reentry to the target ABS on another radio carrier.

Figure xx6 — A call flow for multi-carrier HO in which the AMS performs network reentry to the target ABS on one radio carrier while maintaining communication with the serving ABS on another.

In case AMS is capable to process multiple carriers at the same time, the target primary carrier can be different than the one chosen in serving cell. Figure xx7 shows an example MCHO call flow of the case in which AMS is capable to process multiple carriers at the same time and the target primary carrier is different from the serving primary carrier.

Figure xx7 — A call flow for multi-carrier HO in which the AMS performs network reentry on the target primary carrier which is different from the serving primary carrier

------------------------------------------------------------ Text End ---------------------------------------------------------------