| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized AWD Text Proposal for Inter-RAT Handover procedure |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Jin Lee, Giwon Park, Ronny Yongho Kim, Kiseon Ryu and Jin Sam Kwak LG Electronics Inc Vivek Gupta, Shantidev Mohanty Intel Corporation Roberto Albanese, Zexian Li Nokia Haihong Zheng, Shashikant Meheshwari, Yousuf Saifullah NSN | E-mail: {jins978, gwpark, ronnykim}@lge.com vivekggupta@ieee.org roberto.albanese@nokia.com haihong.zheng@nokia.com |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “Handover (Inter-RAT)” |  |
| Abstract | The contribution proposes the text of inter-RAT HO section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Text Proposal for Inter-RAT Handover Procedure

Jin Lee, Giwon Park, Ronny Yongho Kim, Kiseon Ryu and Jin Sam Kwak

LG Electronics, Inc.

Vivek Gupta, Shantidev Mohanty

Intel Corporation

Roberto Albanese, Zexian Li

Nokia

Haihong Zheng, Shashikant Meheshwari, Yousuf Saifullah

NSN

============================== Start of the Text=================================

4. Abbreviation and acronyms

MIHF : Media Independent Handover Function

15.2 Medium Access Control

15.2.y Inter RAT Capability Negotiation

AMS’s capabilities for inter-RAT operation shall be negotiated with ABS during network entry (e.g., basic capability negotiation). Negotiated Inter-RAT capability is used to provide appropriate other RAT information or to initiate handover procedure with supported other RAT.

A-SBC-REQ/RSP may include the following parameters.

| Name | Type | Length | Value | Scope |
| --- | --- | --- | --- | --- |
| Inter-RAT Operation Mode | xx | 1 | Bit 0: single radio mode operation for inter RAT handover Bit 1: multi radio mode operation for inter RAT handover Bit 2-7: Reserved, set to zero | A- SBC-REQ |
| Supported Inter RAT type | yy | 1 | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero | A- SBC-REQ A- SBC-RSP |
| MIH Capability Supported | zz | 1 | Indicates the capability of IEEE 802.21 Media Independent Handover Services. Each bit set to 1 indicates that the corresponding service is supported. - If bit # 0 is set to 1 in the A-SBC-REQ/RSP message, the ABS/AMS is permitted to send MIH_MOB-MSG messages as further indicated through bits #1 – #3 .If bit #0 is set to 0, bits #1 – #3 shall be set to 0. - If bit #4 is set to be 1 in the A-SBC-RSP message, the AMS is allowed to transmit an MIH information service request in an MIH Initial Service Request message. - When bit #5 is set to be 1 in the A-SBC-RSP message, the AMS is allowed to transmit an MIH request for ES/CS Capability discovery in an MIH Initial Service Request message. Bit #0 = MIH (Media Independent Handover) support Bit #1 = Event Service support Bit #2 = Command Service support Bit #3 = Information Service support Bit #4 = Information Service support during network entry Bit #5 = ES/CS capability discovery support during network entry Bit #6~7: reserved | A-SBC-REQ A-SBC-RSP |

15.2.x Inter-RAT Handover Procedure

15.2.x.1 Network topology acquisition

IEEE 802.16m systems advertise information about other RATs to assist the AMS with network discovery and selection. IEEE 802.16m systems provide a mechanism for AMS to obtain information about other access networks in the vicinity of the AMS from an ABS either by making a query or listening to a system information broadcast. This mechanism can be used both before and after AMS authentication. IEEE 802.16m system may obtain the other access network information from an information server. The ABSs may also indicate the boundary area of the IEEE 802.16m network by advertising a network boundary indication. Upon receiving the indication, the AMS may perform channel measurement on the other RATs. The access network information may also include operator preferences based on the available RATs. The information may be restricted to specific access technologies, based on the AMS’s current location and preferences.

15.2.x.1.1 Passive Other RAT Discovery

ABS may broadcast information such as the presence of another RAT and/or specific information of another RAT and an indication of the boundary area. Upon receiving such information, the AMS may obtain the remaining information of other RAT from a MIH server, and start scanning process.

[A-MOB_ORAT-ADV indicated by Superframe Header (SFH) includes other access network information. SFH defines other RAT information flag. If the value of the other RAT information flag is set, this implies that current super frame carries other RAT information. SFH can be able to contain SFN (start frame number) where A-MOB_ORAT-ADV is transmitted in order to notify AMS of the start frame of the message. A-MOB_ORAT-ADV message is structured in MAC management message on traffic channel and can be transmitted as broadcast (see Table 1). Other RAT information indicated by SFH is illustrated in Fig.x

Fig x. Other RAT information delivery

ABS broadcasts other RAT information to the AMSs through the A-MOB_ORAT-ADV message. A-MOB_ORAT-ADV message includes the following:

- RAT Type: This field specifies air interface technology type.

- Pre-registration supported: This field indicates whether pre-registration is supported or not for Inter RAT handover.

- PHY Profile ID: The PHY Profile ID contains the information related to scan the corresponding RAT. The contents of PHY profile ID are TBD.

- Inter-RAT TLV: The Inter-RAT TLV is a compound TLV list that specifies information for different radio access technologies defined by different standard bodies such as IEEE, 3GPP and 3GPP2. Example of Inter-RAT TLV is defined in A.1

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| A-MOB_ORAT-ADV_Message_format() { | - | - |
| Management Message Type = Undecided | 8 | - |
| N_NEIGHBORS | 8 | Number of Inter-RAT neighbors |
| For ( j = 0; j< N_NEIGHBORS; j++) { | - | - |
| Length | 8 | Length of message information including all fields within the FOR loop. |
| RAT Type | 8 | 0x0: 802.11 0x1: GERAN(GSM/GPRS/EGPRS) 0x2: UTRAN 0x3: E-UTRAN 0x4: CDMA 2000 0x5-7: Reserved, set to zero |
| Pre-registration supported | 1 | 0: Not supported 1: Supported |
| PHY Profile ID | X | TBD |
| } |  |  |
| TLV Encoded Inter-RAT information | Variable | TLV-specific |
| } //End of A-MOB_ORAT-ADV |  |  |

Table 1. A-MOB_ORAT-ADV message format]

15.2.x.1.2 Active Other RAT Discovery

15.2.x.1.2.1 Active Other RAT Discovery with MIHF Support

MIHF support is the support of IEEE Std 802.21 specific features and functions. The 802.16m entity may send or receive the MOB_MIH-MSG to or from the peer 802.16m entity in order to convey MIHF frames carrying the 802.21 MIH protocol messages. When MIH query capability during network entry is enabled, which is notified with MIH Capability Supported TLV in A-SBC-REQ/RSP, PKM messages may be used to exchange MIH frames for MIH queries. The AMS may submit an MIH query by sending a PKM-REQ message with MIH Initial Request code containing an MIHF frame encapsulating the query. Upon receiving this message the ABS acknowledges the request by sending a PKM-RSP message with MIH Acknowledge code. This message does not contain the response to the MIH query, but contains a Cycle TLV which indicates when the response is expected to be ready for delivery to the AMS. This message also contains a Query ID, which the AMS may use to correlate the query with the response, and the delivery method (unicast or broadcast) that the ABS should use. When a unicast delivery method has been negotiated, then if the ABS is ready to transmit the MIH response, the ABS shall allocate bandwidth for the AMS in the UL-MAP in the MAC frame indicated by the Cycle TLV. Upon receiving this UL allocation, the AMS shall transmit at least a Bandwidth request PDU. If the AMS has no data to transmit, the BR field of the Bandwidth request PDU shall be set to 0. The ABS may use the receipt of the Bandwidth request PDU to assert the continued presence of the AMS. If the AMS does not send at least a Bandwidth Request PDU, the ABS shall abort the network entry procedure for the AMS, otherwise it shall send a PKM-RSP message with MIH Comeback Response code containing the encapsulated MIH response. The MIH Comeback Response message shall also contain the Query ID previously sent in the MIH Acknowledge message, which the AMS may use to correlate the MIH response with the MIH query. When a broadcast delivery method has been negotiated, then if the ABS is ready to transmit the MIH response, the ABS shall transmit a SII-ADV message containing the MIH response in the MAC frame indicated by the Cycle TLV. If the ABS is not ready to transmit the MIH response at the time indicated by the Cycle TLV, the AMS and ABS shall wait for another cycle and repeat the procedures specified in the preceding paragraph. The maximum number of times the AMS and ABS shall perform those procedures is determined by the MIH max cycles system parameter.

15.2.x.1.2.2 Active Other RAT Discovery Using 802.16m Scanning

AMS shall initiate other RAT discovery using scanning procedure. The single radio AMS shall negotiate scanning procedure before scanning commencement. [If an AMS’s location information is available, the AMS may transmit its location information with scanning request message and the ABS may respond with recommended RAT information based on the MS’s location information.]

15.2.x.1.2.3 Generic Active Network Discovery and Selection Procedure

During the target RAT selection process, the AMS may communicate with an information repository using its 16m connection to learn operator-defined rules and preferences that affect the inter-RAT handoff decisions. The handoff policy may be pre-provisioned in the AMS and may be updated when AMS requests the information repository for network discovery and selection information. The target RAT discovery and selection procedure is shown as follows.

16m Access

Target-RAT Access

Information Repository

2. Target RAT information

1.AMS connected to 16m Access

3. Policy Request

4. Policy Response

5. Evaluate Target RAT(s) for HO

6. RAT(s) Information Request

7. RAT (s) Information Response

8. Negotiate scan/sleep interval with 16m ABS to perform Target-RAT(s) measurement

9. Turn on Target-RAT(s) radio, configure measurement reporting

10. Target-RAT Measurements

11. Target-RAT Selection and HO Decision

12. Handover Procedure

AMS

Figure y. Target RAT Discovery and Selection

- The AMS is connected to 16m access network

- The AMS learns about the presence of other-RAT(s) [in SFH] and then obtains the system parameters and configuration information [from the Multi-RAT information MAC management message].

- The AMS requests inter-RAT handover policy from the information repository.

- The information repository provides the updates inter-RAT handover policy to the AMS.

- The AMS evaluates target RATs for handover

- The AMS requests more information from the information repository. This can be a unicast information retrieval using MIH messages.

- The information repository provides information about target RATs as requested by the AMS.

- In the single radio case, the AMS negotiates with the 802.16m BS about scan/sleep intervals so that it can evaluate the link connections at target RATs.

- The device turns on the other radios and configures measurement reporting for target RATs.

- The device conducts measurements and these reports are sent by the AMS to the 16m ABS for evaluation.

- The AMS/ABS selects a target RAT for handover.

- The AMS in conjunction with ABS and target access conducts the handover procedure.

15.2.x.2 Generic inter RAT HO procedure

The WirelessMAN-OFDMA Advanced system provides mechanisms for conducting inter-RAT measurements and reporting; they are FFS. Further, it may forward handover related messages with other access technologies such as IEEE 802.11, 3GPP and 3GPP2. The specifics of these handover messages may be defined elsewhere, e.g. IEEE 802.21.

15.2.x.2.1 Generic MIH MAC container

MOB_MIH-MSG defined in IEEE 802.16 REV2 is used to convey IEEE 802.21 handover related messages.

[15.2.x.2.2 Generic Other RAT MAC container

Generic Other RAT MAC container (called A-MOB_ORAT-MSG) is used to convey other RAT control messages which are defined in elsewhere, e.g., 3GPP, 3GPP2. A-MOB_ORAT-MSG consists of Generic MAC header, other RAT subheader and the other RAT control message payload. Other RAT subheader format is FFS.

The A-MOB_ORAT-MSG would be transmitted on the primary connection.

| Syntax | Size | Notes |
| --- | --- | --- |
| A-MOB_ORAT-MSG Message_Format() { |  |  |
| Management Message Type = xx | 8 bits |  |
| TLV encoded information | variable | TLV specific |
| } |  |  |

Table 2. A-MOB-ORAT-MSG format ]

15.2.x.2.3 Measurements

While the AMS is attached to the IEEE 802.16m network and is in active mode, the AMS may need to perform radio measurements on other RATs when directed by the IEEE 802.16m network. The IEEE 802.16m network will provide the AMS required neighbor cell list information and measurement controls. When needed the IEEE 802.16m ABS will be responsible for configuring and activating the measurements on the AMS via dedicated signaling message with appropriately defined IEs.

For single-radio AMSs, measurement gaps are needed to allow the AMS to switch to the other RAT and do radio measurements. These measurement gaps may be AMS controlled or network-controlled. In case of network-controlled scenarios the IEEE 802.16m ABS is responsible for configuring the gap pattern and providing it to the AMS through dedicated signaling. AMSs with a dual receiver can perform measurements on other RATs neighbor cells without tuning away from the IEEE 802.16m network.

In order to assist the IEEE 802.16m ABS, the AMS shall inform the system of its gap-related capabilities. This capability needs to be transferred along with other AMS capabilities. The AMS needs to indicate if it has a dual receiver. In cases that the measurement gaps are not required, the IEEE 802.16m ABS can configure measurements on cells of other RATs without the need to configure measurement gaps. No DL gap patterns will be required for AMSs which are capable of simultaneous reception on the involved frequency bands. No UL gap patterns will be required for AMSs which are capable of simultaneous transmission in one access and conducting measurements on another access.

15.2.x.2.3.1 Scanning

[When AMS’s location information is available, ABS may provide neighbor other RAT information based on the AMS’s location information.] The AMS conducts scanning of neighboring target RAT cells for handover decision. Scanning is triggered by

- AMS: when serving serving channel quality on current RAT falls below a certain threshold

- ABS: the serving ABS may direct AMS to perform scanning via scanning control signaling

[15.2.x.2.3.2 Measurement parameters

The AMS may measure the following parameters when considering handover to IEEE 802.11:

- RSSI: received Signal Strength Indicator

The AMS may measure the following parameters when considering handover to 3GPP/3GPP2 RATs:

- RSSI: received Signal Strength Indicator

- RSRP: Reference Signal Received Power]

15.2.x.2.3.2.3 Measurement Reporting

After completion of scanning, AMS reports scanning results to a serving ABS via A-MOB_SCN-REP.

15.2.x.3 Enhanced inter-RAT HO procedure

15.2.x.3.1 Dual Transmitter/Dual Receiver Support

In addition to the HO procedures specified in section 10.3.4.2, an AMS with dual RF may connect to both an ABS and a BS operating on other RAT simultaneously during handover. The second RF is enabled when inter RAT handover is initiated. The network entry and connection setup process with the target BS are all conducted over the secondary radio interface. The connection with the serving BS is kept alive until handover completes.

In order to reduce the battery consumption, the second radio is only switched on when needed. After detecting the presence indication of another RAT, the dual mode AMS may switch on the secondary radio to scan for the different RATs, based on its PRL (Preferred Roaming List) and other policies defined.

15.2.x.3.2 Single Transmitter/Single Receiver Support

An AMS with a single RF may connect to only one RAT at a time. The AMS will use the source RAT to prepare the target RAT system. Once target RAT preparation is completed the AMS may switch from source RF to target RF and complete network entry in target RAT. Only one RF is active at any time during the handover

Since only one radio can be active at a time in these types of devices, they use the source radio and the back-end connection between the source and target network to prepare the target network for handover. The control signalling and flow for single transmitter/receiver based handovers are shown below.

Authentication

Pre-Registration

[Context Transfer]

AMS

802.16m ABS

WiMAX ASN

Handover Command

Target Network

Figure Z. Control Signaling through MAC Container

15.2.x.3.2.1 Handover preparation with other RAT

The pre-registration procedure enables an AMS to perform network entry to other RAT system prior to handover. The ABS may instruct the AMS whether target system allows pre-registration [via A-MOB_ORAT-ADV or A-MOB_SCN-RSP when other RAT information is included (e.g., ABS may advertise pre-registration capability of neighbor other RATs)].

[For pre-registration procedure, AMS performs other RAT scanning procedure as indicated in 15.2.x.3.2.1 and transmits and receives pre-registration related messages using Generic Other RAT MAC container. ]

15.2.x.3.2.2 Handover Execution & Completion

Once an AMS decides other RAT handover, the AMS requests other RAT handover to a serving ABS. Upon receiving handover response from the ABS, AMS switches its radio over the target RAT and may turn off a serving 16m radio.

[Annexes (Informative)

A.1 Inter-RAT TLVs

| Inter-RAT Type | Value | Scope |
| --- | --- | --- |
| IEEE 802.11 | Network Subtype | A- MOB_ORAT-ADV |
|  | SSID |  |
|  | Data Rate |  |
| 3GPP | Network Type |  |
|  | Network Subtype |  |
|  | Operator Id |  |
|  | FrequencyBand |  |
|  | ChannelBandwidth |  |
| 3GPP2 | Network Type |  |
|  | Network Subtype |  |
|  | Operator Id |  |
|  | Band Class |  |
|  | CarrierFrequency |  |
|  | ChannelBandwidth |  |

A.2 Single radio handover procedure

AMS

802.16 ABS

WiMAX ASN

Target Network

AS/ AAA

15. Release Source Connection and Resources

1. AMS Connected. Uplink/Downlink Traffic

7. Target Context Request

8. Target Context Response

13. Proxy Binding Update (PMIP)

14. Proxy Binding Ack (PMIP)

Common GW/ HA

12. Target Network Entry PHY/MAC Establishment

2. Target Radio ON

11. Handover to Target Command

Target Connected. Uplink/Downlink Traffic

9. Target Resource Preparation

10. Target Handover Response

5. Target Registration Request

6. Target Authentication

Authentication & Authorization

4. Decision to HO

3. Measurement Reporting

Figure xxx. Single radio handover procedure]

============================== End of Proposed Text =================================