| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of Emergency Service flows for the IEEE 802.16m Amendment |  |
| Date Submitted | 2003-03-02 |  |
| Source(s) | Aeran Youn, Taegon Kong, Ronny(Yongho) Kim LG Electronics LG R&D Complex, 533 Hogye-1dong, Dongan-gu, Anyang, 431-749, Korea | Voice: +82-31-450-7188 E-mail: aryoun@lge.com |
| Re: | IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content.” Target topic: “Connection Management” |  |
| Abstract | The contribution proposes the text of contents regarding Connection Management section to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text of Emergency Service flows for the IEEE802.16m Amendment

Aeran Youn, Taegon Kong, Ronny (Yongho) Kim

LG Electronics

# 1. Introduction

This contribution includes the proposed text for Emergency Service flows in the IEEE 802.16m Amendment. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4]. In this contribution, amendment text is proposed for the Connection Management section in which 16m members have enough consensus.

# 2. Modifications from the SDD and key descriptions

The text proposed in this contribution is based on sub-clauses “10.9.3 Emergency service flows” in the IEEE 802.16m SDD [2].

The modifications to the SDD text are summarized as below:

- We created new subsection “15.2.x.x Emergency Service flows” that will be aligned with IEEE 802.16m Amendment Working Document [5].

- In this contribution, we propose two schemes that perform an emergency service notification and a resource allocation for emergency service.

# 2.1 Emergency Service notification in initial ranging

For reducing Emergency Service flow setup procedure, the AMS requests Emergency Service flow setup during initial ranging using a predefined ranging preamble code and a MAC management message (A-MOB_RNG-REQ) for emergency service notification. The MAC management message includes Emergency Service indicator and data delivery service type. The type of data delivery service can be omitted when the AMS requests default service for Emergency Service such as VoIP or SMS. The QoS parameters for the default service are defined for Emergency Service flow. In response to the MAC management message from the AMS, the ABS grants default resources to the AMS in accordance with data delivery service type without going through the complete service flow setup procedure. The ABS periodically allocates uplink resources to the AMS according to the default period and duration per requested service type.

# 2.2 Emergency Service notification in service flow setup

When an AMS requests Emergency Service in normal state, it shall use either bandwidth request or Emergency service request sub-header piggybacked to the UL data.

In the case of using 5-step bandwidth request procedure, the AMS transmits bandwidth request message to the ABS including a Flow ID that is reserved for Emergency Service request [Figure 1]. The Emergency Service Flow ID is xxxx (TBD).

Figure 1 Emergency Service notification using bandwidth request message in 5-step

In the case of 3-step quick bandwidth request procedure, the bandwidth request indicator and quick access message includes Emergency Flow ID for Emergency Service notification [Figure 2].

Figure 2 Emergency Service notification using bandwidth request indicator and quick access message in 3-step

The ABS periodically allocates uplink resources for Emergency Service to the AMS according to the bandwidth request message. The AMS also requests the period or length for resources allocation using piggybacked bandwidth request with extended (sub) header.

# 3. References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”

# 4. Text Proposal for IEEE802.16m Amendment

============= Start of text proposal =================================================

15.2.x.x Emergency service flows

15.2.x.x.1 Emergency Service notification in initial ranging

The AMS requests Emergency Service flow setup during initial ranging process using a predefined ranging preamble code and a MAC management message (A-MOB_RNG-REQ) for emergency service notification. The MAC management message includes Emergency Service indicator and data delivery service type. The type of data delivery service can be omitted when the AMS requests default service for Emergency Service. The QoS parameters for the default service are defined for Emergency Service flow. In response to the Emergency Service notification from the AMS, the ABS grants default resources to the AMS without going through the complete service flow setup procedure. The ABS periodically allocates uplink resources for Emergency Service to the AMS according to the default period and duration per requested service type.

15.2.x.x.2 Emergency Service notification in service flow setup

When an AMS requests Emergency Service in a normal state, it shall use either bandwidth request or Emergency service request sub-header piggybacked to the UL data. In the case of using 5-step bandwidth request procedure, the AMS transmits bandwidth request message to the ABS including a Flow ID which is reserved for Emergency Service request. The Emergency Service Flow ID is xxxx (TBD). In the case of 3-step quick bandwidth request procedure, the bandwidth request indicator and quick access message includes Emergency Flow ID for Emergency Service notification.

The ABS allocates uplink resources for Emergency Service to the AMS periodically according to the default period and duration. The AMS also requests the period or length for resource allocation using piggybacked bandwidth request with extended (sub) header.

============= End of text proposal =================================================