| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | HARQ and ARQ interaction mechanism for 16m AWD |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Youngbin Chang, Rakesh Taori, Anil Agiwal, Baowei Ji, Jungje Son Samsung Electronics Xiangying Yang, Muthaiah Venkatachalam Intel Corporation Mary Chion, Sean Cai ZTE | E-mail:yb.chang@samsung.com E-mail: xiangying.yang@intel.com E-mail: mchion@zteusa.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: HARQ function |  |
| Abstract | The contribution proposes the text of HARQ and ARQ interaction section to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

HARQ and ARQ interaction mechanism for 16m AWD

Youngbin Chang, Rakesh Taori, Anil Agiwal, Baowei Ji, Jungje Son

Samsung Electronics

Xiangying Yang, Muthaiah Venkatachalam

Intel Corporation

Mary Chion, Sean Cai

ZTE

# Introduction

This contribution is dealing with ARQ-HARQ mechanism for 16m AWD [1].All text description is written based on 10.2.3 HARQ and ARQ interaction section in 16m SDD [2].

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.2.x as follows:]

15.2.x. HARQ function

15.2.x.x HARQ and ARQ interaction

When both ARQ and HARQ are applied for a flow, HARQ and ARQ interactions described here can be applied to the corresponding flow.

If the HARQ entity in the transmitter determines that the HARQ process was terminated with an unsuccessful outcome, the HARQ entity in the transmitter informs the ARQ entity in the transmitter about the failure of the HARQ burst. The ARQ entity in the transmitter can then initiate retransmission and re-segmentation of the ARQ blocks that correlate to the failed HARQ burst.

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r7, “802.16m System Description Document (SDD)”