| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ mechanism for 16m AWD |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Baowei Ji, Rakesh Taori, Jungje Son Samsung Electronics Roberto Albanese Nokia Shashikant Maheshwari, Yousuf Saifullah Nokia Siemens Networks Ming-Hung Tao, Mamadou Kone, Ying-Chuan Hsiao ITRI | yb.chang@samsung.com, roberto.albanese@nokia.com shashi.maheshwari@nsn.com MHTao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: ARQ |  |
| Abstract | This contribution proposes an ARQ mechanism to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ mechanism for 16m AWD

Youngbin Chang, Anil Agiwal, Baowei Ji, Rakesh Taori, Jungje Son

Samsung Electronics

Roberto Albanese

Nokia

Shashikant Maheshwari, Yousuf Saifullah

Nokia Siemens Networks

Ming-Hung Tao, Mamadou Kone, Ying-Chuan Hsiao

ITRI

# Introduction

This contribution is dealing with ARQ mechanism for 16m AWD [1]. The text proposal contains two things

1. ARQ block generation and transmission – Initial transmission

2. ARQ feedback simplification

All text description is written based on 10.4.ARQ section text in 16m SDD [2].

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Insert the texts in subclause 15.2.x as follows:]

15.2.x ARQ mechanism

The ARQ mechanism is optional. When ARQ is enabled, it is on a per-connection basis.. ARQ parameters shall be specified and negotiated during connection setup..

A connection shall not have a mixture of ARQ and non-ARQ traffic. The scope of a specific instance of ARQ is limited to one unidirectional flow.

Transmitter may send ARQ feedback polling request to the receiver, to update the reception status of the transmitted ARQ blocks. Receiver sends an ARQ feedback when an ARQ feedback polling request is received from the transmitter or when an ARQ block has been missing for a predetermined period.

15.2.x.1 ARQ block generation and transmission

An ARQ block is generated from one or multiple MAC SDU(s) or MAC SDU fragment(s) of the same flow. ARQ blocks can be variable in size.

ARQ block is constructed by fragmenting MAC SDU or packing MAC SDUs and/or MAC SDU fragments. The fragmentation or packing information for the ARQ block is included in MAC PDU using an MAC extended header.The detail header format of fragmentation and packing information is FFS.

When transmitter generates a MAC PDU for transmission, MAC PDU payload may contain one or more ARQ blocks. If the MAC PDU payload contains traffic from a single connection, PDU payload itself shall be a single ARQ block. If traffic from multiple connections is multiplexed into one MAC PDU, the MAC PDU payload contains multiple ARQ blocks. The number of ARQ blocks in a MAC PDU payload shall be equal to the number of connections.

The ARQ blocks of a connection are sequentially numbered. The ARQ block SN (sequence number) is included in MAC PDU using an MAC extended header.. The location of ARQ SN is FFS. The original MAC SDU ordering shall be maintained.

In case of ARQ block retransmission without rearrangement, the MAC PDU shall contain the same ARQ block and corresponding fragmentation and packing information, which was used in the initial transmission.

If a transmitter fails to transmit ARQ block, ARQ block may be retransmitted with or without rearrangement. ARQ block retransmission with rearrangement is FFS.

15.2.x.2 ARQ feedback IE format

Table xxx defines the ARQ Feedback IE for ARQ block. It is used by the receiver to signal positive or negative acknowledgments. A set of IEs of this format may be transported either as a packed payload (“piggybacked”) within a packed MAC PDU or as a payload of a standalone MAC PDU.

Table xxx – ARQ feedback IE format for ARQ block

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST){ | variable |  |
| Flow ID | 4 | The ID of the flow being referenced |
| LAST | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG = 0, ARQ block up to and including SN has been received successfully. FLAG = 1, ARQ block less than SN has been received successfully. |
| While(FLAG == 1) { |  |  |
| Selective ACK MAP | 8 | Each bit represents ACK or NAK of corresponding ARQ block. ‘0’ is NAK and ‘1’ is ACK. First MSB of first ACK MAP represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP is corresponding to contiguous SN. |
| FLAG | 1 | 0 = No more selective ACK MAP 1 = Another set of selective ACK MAP and FLAG follows |
| } |  |  |
| Reserved | variable | Byte alignment |
| } |  |  |

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-08/003r7, “802.16m System Description Document (SDD)”