| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ in 802.16m |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Roberto Albanese, Andrea Bacioccola Nokia Shashikant Meheshwari, Yousuf Saifullah NSN | roberto.albanese@nokia.com shashi.maheshwari@nsn.com |
| Re: | “802.16m AWD text”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Draft AWD Content”. Target topic: ARQ |  |
| Abstract | The contribution proposes text to the 802.16m Amendment for ARQ Support. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ in 802.16m

- Introduction

This contribution proposes amendment text for the support of ARQ.

- Text proposal for the 802.16m amendment

================= Start of Proposed Text ========================

[Insert the following abbreviations and acronymns (Section 4):]

MFPEH		Multiplexing/Fragmentation/Packing Extended Header

[Insert the following paragraph in Section 15.2:]

- 15.2.x ARQ mechanism

The ARQ mechanism may be enabled for the connection. When ARQ is enabled for a connection, It may be enabled on a per-connection basis. ARQ parameters shall be specified and negotiated during service flow creation.

A connection shall not have a mixture of ARQ and non-ARQ traffic. The scope of a specific instance of ARQ is limited to one unidirectional connection.

Transmitter may send ARQ feedback polling request to the receiver, to update the reception status of the transmitted ARQ blocks. Receiver sends an ARQ feedback when an ARQ feedback polling request is received from the transmitter or when an ARQ block has been missing for a predetermined period.

The ARQ feedback information and the ARQ feedback polling request may be sent as a standalone MAC PDU, or they may be piggybacked on an existing connection.

Cumulative and selective ACK types are used by the receiver for sending an ARQ feedback. ARQ feedback cannot be fragmented.

- 15.2.x.1 ARQ block usage

An ARQ block is generated from one or multiple MAC SDU(s) or MAC SDU fragment(s) of the same flow. ARQ blocks can be variable in size.

ARQ block is constructed by fragmenting MAC SDU or packing MAC SDUs and/or MAC SDU fragments. The fragmentation or packing information for the ARQ block is included in MAC PDU using MFPEH (Multiplexing/Fragmentation/Packing Extended Header).

When transmitter generates a MAC PDU for transmission, MAC PDU payload may contain one or more ARQ blocks. If the MAC PDU payload contains traffic from a single flow, PDU payload itself shall be a single ARQ block. If traffic from multiple flows is multiplexed into one MAC PDU, the MAC PDU payload contains multiple ARQ blocks. The number of ARQ blocks in a MAC PDU payload shall be equal to the number of flows.

The ARQ blocks of a flow are sequentially numbered. The ARQ block SN (sequence number) is included in MAC PDU using MFPEH. The original MAC SDU ordering shall be maintained.

If a transmitter fails to transmit ARQ block, ARQ block may be retransmitted with or without rearrangement.

In case of ARQ block retransmission without rearrangement, the MAC PDU shall contain the same ARQ block and corresponding fragmentation and packing information, which was used in the initial transmission.

ARQ block retransmission with rearrangement is FFS.

- 15.2.x.2 ARQ feedback polling request

The ARQ transmitter may poll the receiver to send an ARQ feedback on the status of the receiver window. If the ARQ transmitter is the ABS, then ABS, along with the ARQ feedback polling request, may also provide the resources for ARQ receiver to transmit the ARQ feedback.

Table xxx shows the information to be sent in the ARQ feedback polling request by the ARQ transmitter. ARQ feedback polling request is sent using extended header.

Table xxx – ARQ feedback polling request Extended Header

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Flow ID | 4 | The ID of the flow being referenced |

- 15.2.x.3 ARQ feedback IE format

Table yyy defines the ARQ Feedback IE used by the receiver to signal positive or negative acknowledgments. A set of IEs of this format may be transported either as a packed payload (“piggybacked”) within a packed MAC PDU or as a payload of a standalone MAC PDU.

Table yyy – ARQ feedback IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE { |  |  |
| Flow ID | 4 | The ID of the flow being referenced |
| LAST | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| ACK TYPE | 1 | 0 = Cumulative ACK 1 = Cumulative ACK with Selective ACK |
| SN | 9 |  |
| If(ACK TYPE == 1) { |  |  |
| N_ACK_MAP | 3 | The number of selective ACK MAPs = N_ACK_MAP + 1 |
| Selective ACK MAP | variable | Each bit represents ACK or NAK of corresponding ARQ block. ‘0’ is NAK and ‘1’ is ACK. Unit of selective ACK MAP is 16 bit. Size of Selective ACK MAP is 16*(N_ACK_MAP+1). First MSB of first ACK MAP represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP is corresponding to contiguous ARQ SN. |
| } |  |  |
| Reserved | variable | Byte alignment |
| } |  |  |

============================== End of Proposed Text ===============