| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Related to Idle Mode for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Giwon Park, GeneBeck Hahn, Eunjong Lee, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: gwpark@lge.com |
| Re: | IEEE 802.16m-09/0012, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. Target topic: “Idle Mode” |  |
| Abstract | The contribution proposes the texts of contents regarding the idle mode operations to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Related to Idle Mode 
for the IEEE 802.16m Amendment

Giwon Park, GeneBeck Hahn, Eunjong Lee, Kiseon Ryu, Yongho Kim, Jin Sam Kwak

LG Electronics.

- Text proposal for the 802.16m amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

#### 15.2.2.2.4. Operation during paging listening interval

The AMS derives the start of the paging listening interval based on the paging cycle and paging offset. At the beginning of paging listening interval, the AMS scans and synchronizes on the A-PREAMBLE of its preferred ABS. The AMS decodes the SFH. The AMS shall confirm whether it exists in the same paging group as it has most recently belonged by getting PGID information.

During paging listening interval, AMS monitors SFH. If SFH indicates change in system broadcast information (e.g. change in system configuration count) then AMS should acquire the latest system broadcast information at the pre-determined time when the system information is broadcasted by the ABS.

During paging listening interval, AMS also monitors the paging indication bitmaps in SFH. If the bit of the paging indication bitmap associated with its frame is set, then AMS will subsequently decode the full paging message at its paging message offset which is assigned at idle mode initiation; otherwise AMS will return to paging unavailable interval.

If the AMS decodes a paging message that contains its identification, the AMS performs network re-entry or location update depending on the notification indicated in the paging message. Otherwise, AMS returns to paging unavailable interval.

-----------------------------------------------------------End of the Text---------------------------------------------------------