| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | - Performance evaluation of codebook based precoding |  |
| Date Submitted | 2009-03-01 |  |
| Source(s) | - Shaohua Li - Nokia Siemens Networks | E-mail: shaohua.li@nsn.com |
| Re: | TGm MIMO DG on MIMO Draft Amendment text |  |
| Abstract | Performance evaluation of codebook based precoding |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Codebook proposal and performance evaluation

- Shaohua Li

Nokia Siemens networks

- Introduction

In 802.16m MIMO DG, the detail of base codebook for 4 Tx has not been decided. In the contribution, we propose one base codebook design. Simulation results with the proposed codebook and other proposals are provided.

- Base codebook proposal

In [1], in order to optimize the codebook both for uncorrelated channel and correlated channel, we propose a new codebook. The proposed codebook is a mixture codebook of 16e codebook and DFT codebook [1], which achieve very good performance in correlated channel, and it also achieve pretty good performance in uncorrelated channel compare with 16e codebook. In this contribution, we provide system level simulation to justify the advantage of this codebook.

For comparison, we also provide system level simulation for other candidate codebooks [2]-[5], which are listed in Table 1.

Table 1: Candidate codebooks

| Reference | Author | Label used in figures |
| --- | --- | --- |
| 806.16e | 802.16e | 6bits_16e |
| C80216m-09_0056 | Guangjie Li | 6bits_09/0056 |
| C80216m-09_0279 | David Mazzarese | 6bits_09/0279 |
| C80216m-09_0106r1 | Yang Tang | 6bits_09/0106 |

- Performance evaluation

In this section, system level result is provided for codebook evaluation. The simulation assumption and parameters are listed in Appendix A. In the system level simulation, both uncorrelated channel and correlated channel are evaluated for CL SU MIMO and CL MU MIMO.

In Figure 1, performance for SU MIMO with uncorrelated channel is given. From Figure 1, we can see that our codebook is better than 16e codebook in this case. Our codebook shows better performance than Huawei and sumsung’s codebook. Figure 2 shows the performance of SU MIMO with correlated channel. From Figure 2, our codebook achieves the best performance. Our codebook has more than 3% gains over 16e codebook.

In Figure 3 and Figure 4, performance for MU MIMO with uncorrelated channel and correlated channel is given. In uncorrelated channel, we have very small performance degradation, but with correlated channel, we achieve at least 3% performance gain over intel’s codebook. Compared with sumsung and Huawei’s codewook, our codebook has 1% performance gain.

Figure 1: Performance of rank-1 SU MIMO with uncorrelated channel

Figure 2: Performance of rank-1 SU MIMO with correlated channel

Figure 3: Performance of ZF MU MIMO with uncorrelated channel

Figure 4: Performance of ZF MU MIMO with correlated channel

- Summary

In this contribution, the new base codebook performance is evaluated. In most cases, the proposed codebook achieves the best overall performance in scenarios with correlated channel and uncorrelated channels.

[1]. IEEE C802.16m-08/1156, ” Codebook design for one stream IEEE 802.16m MIMO Schemes”

[2]. IEEE 802.16e-2005, “IEEE Standard for Local and Metropolitan Area Networks - Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” Feb. 28, 2006.

[3] C80216m-09/0056r2, Performance evaluation of codebook based precoding.

[4] C80216m-08/0279, Proposed Text of MIMO Sections for the IEEE 802.16m Amendment.

[5] C802.16m-08/0106r1, Proposal for 6bit codebooks for 4 transmit antennas.

- Appendix A: Simulation conditions

Table 2: Assumption

| Assumption | Notes |
| --- | --- |
| Number of Antennas | 4 transmitter, 2 receiver [4Tx, 2Rx] |
| Antenna configuration | ULA: 0.5 lambda; |
| MIMO Scheme | - Closed-loop single user with dynamic rank adaptation - Zero-forcing multiple user MIMO Schedule from 1 to 2 users dynamically based on the same rank-1 PMI feedback. No SU/MU mode adaptation. |
| Channel correlation Scenario | 1. Uncorrelated Channel : Zero Correlation 2. High correlated channel: 0.5 lambda antenna spacing, angular spread of 3 degree |
| Antenna Calibration | Ideal antenna calibration (mandatory) |

Table 3: Parameters

| Parameter | Assumption |
| --- | --- |
| OFDM parameters | 10 MHz |
| Permutation | Localized |
| Scheduling Unit | Whole band |
| Link Adaptation | QPSK 1/2 with repetition 1/2/4/6, QPSK 3/4, 16QAM 1/2, 16QAM 3/4, 64QAM 1/2, 64QAM 2/3, 64QAM 3/4, 64QAM 5/6 |
| HARQ | Chase combining, non-adaptive, synchronous. HARQ with maximum 4 retransmissions, ACK/NACK delay, no error on ACK/NACK. |
| Scheduling | No control overhead |
| MIMO receiver | Linear Minimum Mean Squared Error (LMMSE) |
| Data Channel Estimation | Perfect data channel estimation |
| Feedback Channel Measurement | Perfect feedback channel measurement |
| Cellular Layout | Hexagonal grid, 19 cell sites, wrap-around, 3 sectors per site |
| Shadowing standard deviation | 8 dB |
| Antenna pattern (horizontal) (For 3-sector cell sites with fixed antenna patterns) | Aθ=−min12θθ3dB2,Am θ3dB = 70 degrees, Am = 20 dB |
| Users per sector | 10 (SU MIMO) and 20 (MU MIMO) |
| Scheduling Criterion | Proportional Fair (PF for all the scheduled users) |