| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Amendment Text Proposal for Idle Mode |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Shuyu Ma, Mary Chion, Li Wang, Hongyun Qu, Tricci So, Zhaohua Lu, Nan Li ZTE Corporation | mailto: ma.shuyu@zte.com.cn |
| Re: | IEEE 802.16m-0012 – Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content; Section 10.5 Power Management (Idle) |  |
| Abstract | Contribution for idle mode process supporting WirelessMAN OFDMA reference system |  |
| Purpose | Review and adopt proposed Amendment text in next revision of 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Amendment Text Proposal for Idle Mode

Shuyu Ma, Mary Chion, Li Wang, Hongyun Qu, Tricci So, Zhaohua Lu,Nan Li

ZTE Corporation

# 1. Introduction

The WirelessMAN-OFDMA Reference System and WirelessMAN-OFDMA Advanced System may co-exist. Seamless handling of the idle operation between these two systems should be supported. This contribution proposes a method to support this operation.

A network supporting legacy systems may contains a mixture of YBS, ABS with LZone and MZone and ABS with MZone only. An ABS, while in idle mode, may move across the boundaries of a YBS and a ABS and perform reselection of preferred BS. The following problems exist for paging/idle operation in a network supporting legacy system :

- When AMS moves between a YBS to a MZone of ABS and performs reselection of preferred BS, location update must be triggered to allow updates of paging parameters regardless of the paging group assignments of the BSs.

- When AMS moves between a LZone and MZone of a ABS and performs reselection, location update must be triggered to allow updates of paging parameters.

- A ABS that has both LZone and MZone needs to decide in which zone the paging message should be broadcasted for an AMS.

# 2. Proposed solution

To solve the issues stated in the previous section, the following rules should be observed while assigning BSs to paging groups:

- A paging group should not contain a mixture of YBS and MZone of ABS.

- For an ABS that support co-existence with LZone and MZone, it should be assigned to at least two different paging groups in which LZone is assigned to one Paging Group and MZone is assigned to the other.

The above rules in paging group assignment allows location update to be performed whenever a idle AMS moves from a YBS to a MZone of ABS or LZone to MZone.

# 3. Text Proposal for AWD

The following text is proposed as a delta version to what is described in harmonized contribution C80216m-09_0597.doc or its latest revision.

---------------------------------------------------- Start of the Text Proposal ----------------------------------------------

15.2.x.	Idle mode

An ABS may be a member of one or more paging groups that may have different cycle and offset. For an ABS has both LZone and MZone, different paging group(s) shall be assigned to LZone and MZone respectively.

-----------------------------------------------------End of the Text Proposal ----------------------------------------------

# References

- “The Draft IEEE 802.16m System Description Document”, IEEE 802.16m-08/003r7.

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-08/050