| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the Draft P802.16m Amendment on the PHY Structure for BWREQ channel |  |
| Date Submitted | 2009-03-06 |  |
| Source(s) | Yuan Zhu, Xiangying Yang, Roshni Srinivasan, Jong-kae Fwu, Hujun Yin | E-mail: (yuan.y.zhu, Xiangying.Yang, Roshni.m.Srinivasan, Jong-kae.fwu, Hujun.Yin)@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m amendment working document |  |
| Abstract | Proposed text for the PHY structure for UL control in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the Draft P802.16m Amendment on the PHY Structure for BWREQ Control

Yuan Zhu, Xiangying Yang, Roshni Srinivasan, Jong-Kae (JK) Fwu, Hujun Yin

Intel

Introduction

This contribution provides additional amendment text for the BWREQ channel Structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The text was developed based on UL Control Drafting group output [2].

References

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-08/050

- “Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control”, IEEE C802.16m-09/386.

Proposed Amendment Text

The following text is proposed for inclusion in Section 15 of the IEEE 802.16m Amendment Working Document [1].

[  ------------------- Recommended AWD Text Proposal #10 --------------------]

# Bandwidth request Channel

[Proposed text starts]

Contention based random access is used to transmit bandwidth request information on this control channel.The bandwidth request (BW REQ) channel contains resources for the AMS to send a BW REQ access sequence and an optional quick access message. Prioritized bandwidth requests are supported on this channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure for MZone or LZone with AMC is described in Figure UL- . In these cases, aA 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1, 4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, AMS sends a bandwidth request indicator and a message for quick access. The 5-step regular procedure is used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The AMS may piggyback additional BW REQ information along with user data during uplink transmission (step 5).  Following Step 1 and Step 3, ABS may acknowledge the reception of bandwidth request. If AMS does not receive any acknowledgement or UL grant, it waits until the expiration of a pre-defined period and restarts the bandwidth request. The pre-defined period may be differentiated by factors such as QoS parameters (e.g. scheduling type, priority, etc). In case BW is granted immediately, there is no need for ABS to send explicit ACK.

Figure UL- , Bandwidth Request Procedure in MZone or LZone with AMC.

The bandwidth request procedure for LZone with PUSC is described in Figure UL-n. In LZone with PUSC, only a 5-step regular procedure is supported. In step 1, AMS sends a bandwidth request indicator only. The rest of bandwidth request procedure in LZone with PUSC shall be the same as the 5-step procedure in Figure UL-8.

Figure UL-n Bandwidth Request Procedure in LZone with PUSC

As shown in Figure UL- , in the LZone with PUSC, one BW REQ tile consists of 4 contiguous subcarriers by 6 OFDM symbols. The total 72 subcarriers of one LZone BRCH are indexed firstly in frequency, secondly in OFDM symbol and lastly in tile as in Figure UL- . Each BW REQ tile carries a BW REQ access sequence.

In the MZone, the total 108 subcarriers of one MZone BW REQ channel are indexed one tile at a time, firstly in frequency, then in OFDMA symbol as in Figure UL- .

Figure UL- , 4x6 BW REQ tile structure.

Figure UL- , 6x6 BW REQ Tile Structure in the Advance Air Interface.

A BW REQ tile in the Advanced Air Interface specifications is defined as 6 contiguous subcarriers by 6 OFDM symbols. As illustrated in Figure UL-1, a BW REQ access sequence (BW REQ preamble) shall be transmitted in step-1 of the bandwidth request procedure. As shown in Figure 1, the BW REQ preamble shall be transmitted on a resource that spans 4 subcarriers by 6 OFDM symbols. Additionally, a quick access BW REQ message shall be carried in the data portion of the tile that spans 2 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel shall comprise of 3 distributed BW REQ tiles for frequency diversity. The procedure for allocation of resources for transmission of UL control information and the formation of DRUs for such transmission is TBD.

Figure 1: 6x6 BW REQ Tile Structure in the Advance Air Interface

Let  denote a total of 16 bits of information which can be carried in the quick access message. The last 4 bits  shall be carried in the BW REQ preamble using the preamble index. The combined resource in the data portions of the three tiles that form the BRCH shall be used to transmit the first 12 bits of information, . Figure 2 illustrates the construction of the BW REQ preamble and quick access message for a 6x6 tile structure.

Figure 2: Bandwidth Request Channel structure with 6x6 tiles

The logical preamble sequence index, BWREQ_PREAMBLE_INDEX, which is configured by the upper layer and 4 bits of the bandwidth request message shall be used to select three sequences of length 24 from Table 1 in order to construct 72 preamble symbols. As shown in Figure 2 , the 72 preamble symbols shall be scrambled before multiplexing with data symbols in the BRCH.

Table 1: BRCH Preamble sequences

| 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 |
| 2 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 |
| 3 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 |
| 4 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 |
| 5 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 |
| 6 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 |
| 7 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 |
| 8 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 |
| 9 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 |
| 10 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 |
| 11 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 |
| 12 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 | -1 |
| 13 | 1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 |
| 14 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 |
| 15 | 1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 |
| 16 | 1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 |
| 17 | 1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 |
| 18 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 | -1 |
| 19 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | 1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 | -1 |
| 20 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 | -1 |
| 21 | 1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 | 1 |
| 22 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | 1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 | -1 |
| 23 | 1 | 1 | -1 | 1 | 1 | 1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | -1 | 1 | -1 | -1 | -1 | 1 | 1 | 1 | -1 | 1 | 1 |

The preamble sequences transmitted in the three BW REQ tiles of a BRCH are defined as

where  is BRCH channel index,  is symbol index in the preamble portion of a BW REQ tile,  is BW REQ tile index,  is sequence index in BW REQ tile .

The mapping between the combination of the logical preamble sequence index, BWREQ_PREAMBLE_INDEX and the 4 bits of the bandwidth request message  to the physical preamble index,  is as below equation.

um,=mod( BWREQ_PREAMBLE_INDEX+ bin2dec(), 24)

The first 12 bits of information  in the quick access message transmitted in the BRCH shall be encoded into 36 bits using the linear block code (36, 12) described in section Error: Reference source not found. The 36 coded bits shall then be BPSK modulated as described in Section TBD and scrambled to generate 36 data symbols, .  The combined data portions of the three distributed BW REQ tiles that form the BRCH shall be used to transmit these data symbols. The data symbols shall be scaled by  before multiplexing with preamble symbols and the scaling factor  is configured by the MAC layer. The mapping of the 36 data symbols to the data portions of three distributed 6×6 BW REQ tiles in a BRCH is given in Table 2. The final 108 modulated BRCH symbols shall be scaled by a factor of  to normalize the total transmitted energy.

Table 2: Mapping of data symbols to subcarriers in the data portion of the BW REQ Tile

| BW REQ tile index |  |
| --- | --- |
| 0 |  |
| 1 |  |
| 2 |  |

In order to support operation in the legacy mode, a BW REQ tile shall be defined as 4 contiguous subcarriers by 6 OFDM symbols. As shown in Figure 3, only the BW REQ access sequence or BW REQ preamble shall be transmitted in all 24 subcarriers that form the BW REQ tile. In this case, the BWREQ_PREAMBLE_INDEX shall be randomly selected from all available logical preamble sequences, as shown in Figure 3.

Figure 3: 4x6 BW REQ tile structure

Figure 4: 4x6 Bandwidth Request Channel structure with 4x6 tiles

[Proposed text Ends]

[  ------------------- Recommended AWD Text Proposal #13 --------------------]

# Bandwidth request Channel

[Proposed text starts]

When the 3-step quick access bandwidth request is used, a quick access message is sent in step-1 along with a BW REQ preamble sequence. The quick access message and the corresponding preamble index contain 16-bit information, including bandwidth request ID that uniquely identifies the MS, the request size, QoS ID. The message format is as follows

|  | Length (bit) | Description | note |
| --- | --- | --- | --- |
| BWREQ ID | 10 | Uniquely identifies the MS | Negotiated during MS network entry |
| BWREQ size | 2 | 00: 1PRU (with lowest MCS defined in cell) 01: 2PRU 10: 3PRU 11: max buffer request negotiated during MS network entry for this QoS class | Buffer size report with ariable granularity |
| QoS ID [Flow ID] | 1 [2] [4] | 0: 50ms 1: 100ms | A QoS ID corresponding to a specific latency requirement. This implies that best-effort traffic will only perform 5-step regular BWREQ. [4bit Flow-ID consistent with connection addressing] |
| Reserved | [3] |  |  |

When 3-step quick access is used, MS shall set BWREQ_PREAMBLE_INDEX according to equation XX-4 before accessing the BW REQ channel.

BWREQ_PREAMBLE_INDEX=                equation XX-4

where  is BWREQ ID and  is BW REQ opportunity index monotonically increasing with time.

[Proposed text ends]