| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced Bandwidth Grant |  |
| Date Submitted | 2009-03-04 |  |
| Source(s) | Kuk-Jin Song, Yerang Hur Hyung-Joon Jeon, Seunghyun Lee Jaehyeong Kim, Kyounghwan Lee Bong Ho Kim POSDATA | Voice: +1-408-986-1143 E-mail:song@posdata.co.kr yehur@posdata-usa.com hjjun@posdata.co.kr huhuya@posdata.co.kr jaekim@posdata.co.kr khlee@posdata-usa.com bhkim@posdata-usa.com |
| Re: | CR for IEEE 802.16m SDD - 10.10 QoS |  |
| Abstract | Bandwidth allocation with the usage of uplink bandwidth grants indicated in the grant |  |
| Purpose | To discuss and adopt the proposed text in 802.16 SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Enhanced Bandwidth Grant

Kuk-Jin Song, Yerang Hur, Hyung-Joon Jeon, Seunghyun Lee,

Jaehyeong Kim, Kyounghwan Lee, Bong Ho Kim

POSDATA

- Introduction

In WirelessMAN OFDMA reference system, it takes several frames to receive a response from the MS after the BS sends a BS-initiated request message to the MS. If contention of BWR codes is heavy in the system, the BS should wait to receive the response message for long. This is illustrated in Figure 1.

Figure 1 Handshaking in WirelessMAN OFDMA reference systems requiring at least 5 frames to receive a response message after the BS-initiated request message is transmitted

We propose that the BS indicates the frame offset of the UL burst for the response message, when the BS-initiated request message is transmitted. When the bandwidth is granted, the BS will indicate the grant will be used for transmitting the response message in response to the BS-initiated request message. In such a case, the MS shall send the response message in the allocated burst with the highest priority. Then the BS can expect the response message at the specified frame as shown in Figure 2.

Figure 2 Handshaking in 802.16m systems allowing the MS to transmit the response message to the BS-initiated request message at the pre-specified frame.

Secondly, in WirelessMAN OFDMA reference systems, the BS grants the uplink bandwidth addressed by the MS Basic CID for the unicast bandwidth requests. It can cause a mismatch between the expectation of the BS when the bandwidth is granted and the execution of the MS when the bandwidth is utilized. For example, when the BS expects the ARQ feedback for the connections with ARQ enabled, if the MS delays the ARQ feedback due to uplink traffic, the overall system performance can be degraded.

Therefore, also we propose that the BS should be able to classify the grants and indicate the purpose of its grant when the bandwidth is allocated. If the purpose of the grant is provided by the BS, the MS shall use such grants for the purpose indicated by the BS.

- Proposed Text for the 802.16m SDD

------------------------------  Start of the proposed text  --------------------------------------------------------------

10.10.3  Enhanced Bandwidth Grant Mechanism

For the bandwidth requested by the AMS, the uplink grant can be identified by Station ID of the AMS, however, to facilitate scheduling of the ABS, the ABS may indicate the purpose of the grant the ABS provides. In such a case, the ABS may indicate which grant is for the management connection and which grant is for the transport connection. If the ABS indicates the purpose of a grant, the AMS shall use granted bandwidth accordingly. For each granted allocation, the AMS may perform piggyback request or bandwidth stealing based on the uplink grant scheduling type usage rules.

When the ABS transmits a BS-initiated request message, the ABS may allocate an UL burst for the AMS to transmit the corresponding response message in the frame specified in the BS-initiated request message. In such a case, the ABS indicates that the purpose of the granted UL burst is for the response message to the BS-initiated request message and the AMS shall send the response message with the highest priority in the allocated burst.

---------------------------  End of the proposed text   ---------------------------------------------------------------

References

- IEEE 802.16m-08/003r7. The Draft IEEE 802.16m System Description Document, February 2009.

- IEEE P802.16Rev2/D9. Draft Standard for Local and Metropolitan Area Networks Part 16: Air Interface for Broadband Wireless Access Systems, January 2009.