| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text for Secondary Fast Feedback Control Channel |  |
| Date Submitted | March 5, 2009 |  |
| Source(s) | Tsao-Tsen (Jason) Chen, Kai Yu, Sten Sjöberg , Per-Erik Östling, Per Ernström Ericsson AB | Email: {jason.t.chen, kai.a.yu, sten.ingemar.sjoberg, per-erik.ostling, per.ernstrom}@ericsson.com |
| Re: | 802.16m-09/012, Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content Ballot id: tgmawd |  |
| Abstract | This contribution proposes the design of secondary fast feedback channel to the UL Ctrl DG text. |  |
| Purpose | Discussion and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text for Secondary Fast Feedback Control Channel

Tsao-Tsen (Jason) Chen, Kai Yu, Sten Sjöberg, Per-Erik Östling, Per Ernström

Ericsson AB

Introduction

Based on the performance and complexity comparisons 1 of different channel coding proposals for the secondary fast feedback channel (SFBCH), we propose that the SFBCH payload be encoded by tail-biting convolutional code (TBCC). Also, the TBCC encoding is done only once for each payload; that is, there is no need to divide larger payload (when  13 bits) into 2 smaller payloads followed by 2 separate encodings.

TBCC is a class of linear block code and can be described by the basis sequences' table format in section 15.3.9.2.1.2.1. However it can be more efficiently described by the set of generator polynomials and the associated expurgation and puncturing tables.

Proposed Text

Add the texts, figure and table marked in blue in the following sections of 2

[  ------------------- AWD Text Proposal #6 of UL Ctrl DG --------------------]

# Secondary Fast Feedback Control Channel

The SFBCH is comprised of 3 distributed FMTs with [2] [4] pilots allocated in each FMT. Pilot sequence is TBD.

The SFBCH symbol generation procedure is shown in Figure UL-5. First, the SFBCH payload information bits a0a1a2⋯al−1 a0a1a2⋯aL−1are encoded to N bits b0b1b2⋯bN−1using the linear block codes (tail-biting convolutional codes) described in Section 15.3.9.2.1.2.1.

l≤12a0a1a2⋯al−1(N,l)

12<l≤24a0a1a2⋯al−1a0a1a2⋯al2−1al2al2+1al2+2⋯al−1N2b0b1b2⋯bN2−1(N2,l2)N2bN2bN2+1bN2+2⋯bN−1(N2,l−l2)The coded sequence b0b1b2⋯bNis then modulated to N2symbols c0c1c2⋯cN2−1using QPSK. The value of N is TBD.  The modulated symbols c0c1c2⋯cN2−1 and pilot sequence (TBD) are combined to form sequence d0d1d2⋯d35 and then mapped to the data subcarriers of the SFBCH FMTs as shown in Figure UL-6.

Figure UL-5, Mapping of information in the SFBCH.

Figure UL-6, SFBCH comprising of three distributed 2x6 UL FMTs.

# Channel coding for Secondary Fast Feedback Control Channel

The  L (7≤K≤12 7  L  24) information bits in the SFBCH shall be encoded using tail-biting convolutional code (TBCC) with codeword length. Let the  L information bits be denoted by a0,a1,…,aK−1 a0a1a2⋯aL−1 and the N bits codeword is denoted byb0,b1,…,bN−1. vi,nn=0,⋯,K−1i=0,⋯,N−1

Figure UL-7: Tail-biting convolutional codes for encoding SFBCH with payload sizes 7 to 24 bits. For payload sizes 7 to 11 bits, zeros are inserted into the information bits input to the encoder, and the first-level puncturing and the second-level puncturing do not apply.

The set of generator polynomials for the mother TBCC is given in octal format by (16, 112, 556, 636, 656). For N=60, the information bits, (a0, a1,…, aL-1), in the SFBCH shall be encoded to 60 encoded bits, (b0, b1, …, b59). For 7≤L≤11, 5 to 1 zeros, respectively, are inserted into the information bits to perform the expurgation operation. The resulting length-12 zero-padded information bits vectors are encoded by the rate 1/5 mother TBCC to generate 60 encoded bits (see Figure UL-7). For 7≤L≤11, the input bits vectors are given by

(ā0,..., ā 11)=(a0,a1,a2,0,a3,0,a4,0,a5,0,a6,0) for L=7,

(ā 0,..., ā 11)=(a0,a1,a2,a3,a4,0,a5,0,a6,0,a7,0) for L=8,

(ā 0,..., ā 11)=(a0,a1,a2,a3,a4,a5,0,a6,a7,0,a8,0) for L=9,

(ā 0,..., ā 11)=(a0,a1,a2,a3,a4,a5,0,a6,a7,a8,a9,0) for L=10,

(ā 0,..., ā 11)=(a0,a1,a2,a3,a4,a5,a6,a7,a8,a9,a10,0) for L=11.

For 12≤L≤24, the input bits vector, (ā 0, ā 1,…, ā L-1)=(a0, a1,…, aL-1), are encoded by the rate 1/5 mother TBCC, the rate 1/4 TBCC, or the rate 1/3 TBCC according to the first-level puncturing given in Table UL-3. The rate 1/4 TBCC and rate 1/3 TBCC are punctured from the rate 1/5 mother TBCC by using only 3 and 4, respectively, generator polynomials from the set (16, 112, 556, 636, 656). The encoded bits from the TBCC encoders are further punctured by the second-level puncturing given in Table UL-3. The TBCC encoder is shown in Figure UL-7. Take the (60,21) (L=21) TBCC as an example. The 21 information bits are first encoded by the TBCC with the set of generator polynomials (112, 556, 636), and 63 encoded bits are generated. The (3m)th,(3m+1)th, and (3m+2)th encoded bits are the encoder outputs from generator polynomials 112, 556, and 636, respectively, for the mth input information bit, where m=0,1,…,20. During the second-level puncturing, the 0th, 27th, and 54th encoded bits are further punctured and the remaining 60 encoded bits exit the encoder.

Table UL-3: The puncturing patterns for generating (60,L) TBCC from the rate 1/5 mother TBCC with the set of generator polynomials (16,112,556,636,656). In the first-level puncturing, TBCC with all or part of the generator polynomials (in the second column) from the mother code set (16,112,556,636,656) are used to encode payloads from 12 bits to 24 bits. In the second-level puncturing, some (or none) of the encoded bits from the TBCC encoders (after the first-level puncturing) are further punctured according to the bit positions listed in the third column.

| L | First-level puncturing: the set of generator polynomials for the TBCC encoding | Second-level puncturing: positions of bits to be punctured after the TBCC encoding |
| --- | --- | --- |
| 12 | (16,112,556,636,656) | none |
| 13 | (16,112,556,636,656) | (1,16,31,46,61) |
| 14 | (16,112,556,636,656) | (3,8,13,18,23,28,33,38,43,48) |
| 15 | (16,112,636,656) | none |
| 16 | (16,112,636,656) | (0,16,32,48) |
| 17 | (16,112,636,656) | (0,8,16,24,32,40,48,56) |
| 18 | (16,112,636,656) | (0,4,8,12,16,20,24,28,32,36,40,44) |
| 19 | (16,112,636,656) | (0,4,8,12,16,20,24,32,36,40,48,52,56, 64,68,72) |
| 20 | (112,556,636) | none |
| 21 | (112,556,636) | (0,27,54) |
| 22 | (112,636,656) | (2,15,23,31,52,60) |
| 23 | (112,636,656) | (0,3,12,21,30,33,42,51,60) |
| 24 | (112,636,656) | (1,7,13,19,25,31,37,43,49,55,61,67) |

References

- IEEE C80216m-09_0506, “Tail-Biting Convolutional Codes with Expurgation and Rate-Compatible Puncturing for the Secondary Fast Feedback Channel”.

- IEEE C80216m-09_0386, “Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control”.