| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Sounding Allocation in 16m (reply comments on UL control DG) |  |
| Date Submitted | 2009-3-6 |  |
| Source(s) | Dong LI, Hongwei YANG, Keying WU, Xiaolong ZHU Alcatel-Lucent Shanghai Bell Co., Ltd. | Voice: + 86-21-50554550; Fax:+ 86-21-50554554 Mail to: Hongwei.yang@alcatel-sbell.com.cn |
| Re: | Reply comment for sounding definition in UL control DG. |  |
| Abstract | This contribution proposes the text amendment for the uplink sounding of 802.16m based on the definition of SDD. |  |
| Purpose | Adopt the proposed text into amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal on Sounding Allocation in 16m

Dong LI, Hongwei YANG, Keying WU, Xiaolong ZHU

Alcatel-Lucent Shanghai Bell Co., Ltd.

# 1 Introduction

This contribution proposes the text amendment as the reply comments for the UL control DG texts in contribution [2].

2. Text proposals of reply comments for AWD comment #210 (i.e. comment 8 in C80216m-09_0386)

--------------------------------------- Reply comment 1 for AWD comment 210------------------------------------

[Note: CDM and FDM have respective advantages and disadvantages and are suitable for different scenarios. Both shall be supported in the specification.]

# Multiplexing for multi-antenna and multi-AMS

The uplink sounding channels of multiple AMS and multiple antennas per AMS can be multiplexed through [Option 1: decimation separation or cyclic shift separation][Option2: decimation separation] in each sounding allocation. Also, in case of multiple UL subframes for sounding, time division separation can be applied by assigning different AMS to different UL subframe. For cyclic shift separation each AMS occupies all subcarriers within sounding allocation and uses the different sounding waveform [Editor's note: remove this sentence if Option 2 will be adopted]. For frequency decimation separation each AMS uses decimated subcarrier subset from the sounding allocation set with different frequency offset.

------------------------------------------------- End of reply comment 1 -----------------------------------------------

--------------------------------------- Reply comment 2 for AWD comment 210------------------------------------

[Note: suggest adding a new subsection for sounding allocation]

15.3.9.2.3.3 Sounding allocation

The sounding alloction for AMSs shall be constrainted to the sounding symbols within one frame. Each sounding symbol is exclusively contained in an uplink subframe. The sounding allocation shall be based on the sounding resource unit (SRU), which consists of 18 adjacent subcarriers in frequency corresponding to one PRU over the sounding symbol. Three SRU indexing methods can be used in the sounding allocation to AMSs, that is, DL PRU matched logical indexing, UL PRU matched logical indexing and physical indexing. The DL and UL PRU matched logical indexing methods can provide more consistent sounding allocations with the respective corresponding data subchannels for precoding and hence they can be used to facilitate the efficient closed-loop sounding-based DL and UL precoding operation. The physical indexing is the same as that in type-A sounding definition of WirelessMAN-OFDMA system. The ABS can configure the sounding symbols and the applied SRU indexing methods through the Sounding_Configuration_IE as shown in table X. The sounding configuration IE can be carriered in BCH or the non-user specific information of the USCCH.

The sounding channel allocation is based on the SRU and the sounding channels of multiple AMSs and multiple antennas of per AMS can be multiplexed on the allocated SRUs through decimation separation or cyclic shift separation. The sounding channels can be allocated by ABS through Sounding Allocation IE, which is carried in the USCCH. The format of Sounding Allocation IE is shown in Table Y.

[Notes: In the current 16m SDD, multi-BS MIMO is accepted as an advanced MIMO technique to mitigate inter-cell interference (ICI) in Section 20.2. It is also accepted in Section 11.8.4.1 to improve the cell-edge and sector throughput. To enable efficient sounding-based multi-BS MIMO operation, the following texts proposals are suggested to be adopted in AWD]

To support sounding based multi-BS MIMO, two or more neighboring BSs can coordinate their sounding channel allocation to ensure the orthogonality between sounding signals transmitted in neighboring cells.

Table X Sounding Configuration IE format

| Syntax | # of bits | Notes |
| --- | --- | --- |
| Sounding_Configuration_IE() { |  |  |
| Sounding existence | 1 | 0: not exist; 1: exist In persistent sounding configuration, sounding resource can be de-allocated by setting it to 0 |
| If (Sounding existence = 1) { |  |  |
| Sounding count, SC | 2 | # of sounding symbols = sounding count + 1 Note that it is assumed the first [or last (TBD)] # UL subframes contain sounding symbols. |
| for(i=0;i<SC+1;i++) { |  |  |
| Sounding index | 2 | SRU indexing method indication for the i-th sounding symbol 0b00: DL PRU matched logical indexing 0b01: UL PRU matched logical indexing 0b10: Physical indexing 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |

Table Y Sounding Allocation IE format

| Syntax | # of bits | Notes |
| --- | --- | --- |
| Sounding_Allocation_IE() { |  |  |
| Num sounding symbols | 2 | Number of sounding symbols (Nsnd) allocated from 1 (0b00) to 4 (0b11) |
| for (i=0;i<Nsnd; i++) { |  |  |
| Sounding symbol index | 2 | Index of the sounding symbol |
| If (sounding index = 0b00 or 0b10) { |  | DL/UL PRU matched logical indexing |
| Frequency partition bitmap | 4 | Bitmap of frequency partitions for sounding allocation. Number of frequency partitions is denoted as Nfp |
| for (j=0;j<Nfp;j++) { |  |  |
| SRU offset | [4] |  |
| SRU number | [4] |  |
| } |  |  |
| } else { |  |  |
| SRU offset | [6] |  |
| SRU number | [6] |  |
| } |  |  |
| Separation type | 1 | 0: decimation separation 1: cyclic shift separation |
| If (separation type = 0) { |  |  |
| Decimation value, D | 3 | 0b000: D=2 0b001: D=3 0b010: D=6 0b011: D=9 0b100: D=18 0b101~0b111: reserved |
| } else { |  |  |
| Max cyclic shift index, P | 3 | 0b000: P=2 0b001: P=3 0b010: P=6 0b011: P=9 0b100: P=18 0b101~0b111: reserved |
| } |  |  |
| Num of Station ID | 5 |  |
| for (j=0;j<num of STID;j++) { |  |  |
| Station ID | [n] |  |
| Multi-antenna flag | 1 | 0: AMS sounds the single antenna only (the first physical antenna or the visual antenna corresponding to the data stream) 1: AMS sounds all antennas |
| Precoding indicator | 1 | Generally, precoding indicator is used for multi-antenna flag = 0 0: Unprecoded sounding is used 1: Precoded sounding is used |
| If (separation type = 0) { |  |  |
| Decimation offset, d | 5 | Relative starting offset position for the first sounding occupied subcarrier in the SRUs |
| } else { |  |  |
| Cyclic shift index, n | 5 | Specifies a frequency-domain phase ramp to be multiplied to the sounding sequence. The value of n ranges from 0 to P-1 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

------------------------------------------------- End of reply comment 2 -----------------------------------------------

References

- IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-09/0386, “Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control”