| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO midamble proposal |  |
| Date Submitted | 2009-03-06 |  |
| Source(s) | Alexei Davydov Niu Huaning Guangjie Li Jong-kae Fwu Yang-seok Choi Intel Corporation | alexei.davydov@intel.com |
| Re: | Call for Comments on Amendment Working Document, IEEE 802.16m-09/0012 |  |
| Abstract | The contribution proposes the text of downlink reference signal section to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO midamble proposal

Alexei Davydov, Niu Huaning, Li Guangjie, Jong-kae Fwu, Yang-seok Choi

Intel Corporation

- Introduction

To facilitate efficient closed-loop and open-loop downlink MIMO operation, accurate PMI and CQI estimation is necessary at the MS. There are two types of reference signals that can be used for that purpose: a dedicated pilots and a midamble. The dedicated pilots are usually precoded with the same precoding matrix as corresponding data subcarriers and carrier information about equivalent channel for data detection. The dedicated pilots are more suitable for CQI estimation, since the MS does not need to know the precoding matrix used on data subcarriers. In this case CQI estimation at the MS can be performed in transparent way. In contrast, the midable sequence is transmitted with no precoding and suitable for acquiring full MIMO channel at the MS for PMI selection in closed loop MIMO.

The pilot structure has been adopted in [1] for different configuration of transmit antennas, while MIMO midamble definition has not been described. In this contribution we propose a physical structure of the MIMO midamble.

- Motivation

A physical structure of downlink MIMO midamble and design requirements have been proposed in [2]. There are additional requirements that should be considered for the downlink midamble design:

- Complexity: downlink sub-frames with midamble sequence should not introduce the additional mapping rule

- PAPR: to exploit low PAPR properties of the midamble sequence a mixture of data, pilots and midamble subcarriers within the same OFDM symbol should be avoided

- Channel measurements efficiency: the midamble pilots should be evenly spaced across the frequency, to simplify filter design and improve the channel estimation performance

- Flexibility: The midamble structure should support 2, 4 and 8 transmit antenna configuration in the flexible way

This contribution proposes alternative physical structure of the MIMO midamble that satisfies to the additional requirements and to the requirements specified in [2]. The recommended midamble has a similar structure as midamble structure of the reference WirelessMAN-OFDMA system. It allows efficient channel estimation, flexible extension to any number of transmit antennas, usage of low PAPR sequence and does not require changes in the mapping rules.

- Text proposal for inclusion in the 802.16m AWD

-------------------------------  Text Start  ---------------------------------------------------

Line 34, page 34, insert 15.3.5.4.2 MIMO midamble

15.3.5.4.2 MIMO midamble

MIMO midamble is used for PMI selection in closed loop MIMO. The midamble signal occupies the first OFDMA symbol of a DL sub-frame. For the type-1 subframe case, the remaining 5 consecutive symbols are formed to be a type-3 subframe. For the type-2 subframe case, the remaining 6 consecutive symbols are formed to be a type-1 subframe.

Multiple antennas in the midamble symbol are multiplexed through decimation separation. For frequency decimation separation each transmit antenna of the BS uses a non-overlapping, decimated subcarrier subset from the number of used subcarriers (Nused). The orthogonally between subcarrier subsets is guaranteed by a unique frequency offset.

-------------------------------  Text End  ---------------------------------------------------

- References

[1] IEEE 802.16m Amendment Working Document (AWD), IEEE 802.16m-09/0010

[2] “Performance evaluation of MIMO midamble”, IEEE C802.16m-09/408