| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Amendment Text on 802.16m Amendment on the PHY Structure for UL Control - Recommended AWD Text Proposal #8 |  |
| Date Submitted | 2009-03-02 |  |
| Source(s) | Peter Wang, Xin Qi, Joon Chun, Adrian Boariu Nokia Siemens Networks Andrei Malkov, Zexian Li Nokia | E-mail: peter.wang@nsn.com xin.qi@nsn.com |
| Re: | 802.16m amendment working document |  |
| Abstract | Based on DG output C80216m-09/0386 new text is proposed regarding sounding contents |  |
| Purpose | To incorporate the proposed text into the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Amendment Text on 802.16m Amendment on the PHY Structure for UL Control - Recommended AWD Text Proposal #8

Peter Wang, Xin Qi, Joon Chun, Adrian Boariu

Nokia Siemens Networks

Andrei Malkov, Zexian Li

Nokia

Introduction

The contribution proposes the text remedy of sounding channel in UL control PHY structure section to be included in the 802.16m amendment [1]. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9 [2], it is compliant to the 802.16m SRD [3] and the 802.16m SDD [4], and it follows the style and format guidelines in [5].

Overview of Sounding Multiplexing for Multi-antenna

ABS can assign AMS to send multiple sounding signals with multiple Tx chains. Maximum number of sounding signals transmitted by an AMS is the number of Tx chains of the AMS. When the number of AMS Tx chains is the same as that of AMS Rx chains and that of assigned sounding channels, AMS transmits sounding signal on each assigned sounding channel with the corresponding Tx chain. When the number of AMS Tx chains is the same as that of AMS Rx chains but larger than that of assigned sounding channels, ABS can assign a unique sounding channel to the first physical Tx antenna of the AMS and to allow the rest physical Tx antennas to share the remaining sounding channels, with which ABS can probabilistically achieve most MIMO channel information with minimized sounding channel allocation. For example, one AMS has two Tx antennas called AMS_11 and AMS_12 and another AMS has two Tx antennas called AMS_21 and AMS_22. ABS can assign a unique sounding channel to AMS_11 and another unique sounding channel to AMS_21. ABS should assign a shared sounding channel to both AMS_12 and AMS_22 to minimize sounding channel occupancy. On the other hand, ABS can also control the AMS to switch the sounding channels between the physical Tx antennas, with which ABS can deterministically achieve full MIMO channel information with full sounding channel allocations for each individual physical Tx antennas. Also, when the number of AMS Tx chains is smaller than that of AMS Rx chains, ABS can control the AMS to switch the physical Tx antennas. The signaling method is TBD.

Reference

- IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”

- IEEE P802.16 Rev2/D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” Jan. 2009.

- IEEE 802.16m-07/002r8, “802.16m System Requirements”

- IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

- IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

Proposed Remedy

### ------------------------------------------------------------------------------------------------------------------------------------

### 15.3.9.2.3 Sounding Channel

Optionally, ABS shall broadcast CQI threshold per frequency partition and the informed AMSs perform sounding based on the CQI threshold. If the CQI measured on DL for a particular PRU/subband exceeds the CQI threshold then the AMS transmits sounding signal in the corresponding UL PRU/subband. Otherwise, the AMS shall not transmit the sounding signal in the corresponding PRU/subband. ABS detects the sounding signals in allocated sounding channels and can calculate the CQI information of UL PRU/subband where AMS transmits sounding signals.

# 15.3.9.2.3.2. Multiplexing for multi-antenna and multi-AMS

The uplink sounding channels of multiple AMS and multiple antennas per AMS can be multiplexed through [Option 1: decimation separation or cyclic shift separation][Option2: decimation separation] in each sounding allocation. Also, in case of multiple UL subframes for sounding, time division separation can be applied by assigning different AMS to different UL subframe. For cyclic shift separation each AMS occupies all subcarriers within sounding allocation and uses the different sounding waveform [Editor's note: remove this sentence if Option 2 will be adopted]. For frequency decimation separation each AMS uses decimated subcarrier subset from the sounding allocation set with different frequency offset.

ABS can assign AMS to send multiple sounding signals with multiple Tx chains. When the number of AMS Tx chains is the same as that of AMS Rx chains but larger than that of assigned sounding channels, ABS can assign a unique sounding channel to the first physical Tx antenna of the AMS and to allow the rest physical Tx antennas to share the remaining sounding channels, with which ABS can probabilistically achieve most MIMO channel information with minimized sounding channel allocation. The signaling method is TBD.

-------------------------End of Proposed Amendment Text---------------------------------------------------------