| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the PFBCH contents for UL Control for IEEE 802.16m Amendment (reply comment on UL control DG AWD comment #213) |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Jeong Julie Lee, Hongmei Sun, Rath Vannithamby, Jong-Kae (JK) Fwu, Hujun Yin, Guangjie Li, Yuan Zhu, Yang-seok Choi, Roshni M Srinivasan, Apostolos Papathanassiou Intel Corporation | E-mail: jeong.julie.lee@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for reply comments on Project 802.16m Amendment Working Document (AWD). |  |
| Abstract | The contribution proposes the text of PFBCH contents in the UL Control section to be included in the 802.16m amendment. |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on the PFBCH Contents for UL Control for the Draft P802.16m Amendment

Jeong Julie Lee, Hongmei Sun, Rath Vannithamby, Jong-Kae (JK) Fwu, Hujun Yin, Guangjie Li, Yuan Zhu, Yang-seok Choi, Roshni M Srinivasan, Apostolos Papathanassiou,

Intel Corporation

Introduction

This contribution provides amendment text for the PFBCH feedback contents to be included in the text from the UL Control Drafting group [2] and to be adopted in the IEEE 802.16m Amendment Working Document [1].

References

- IEEE 802.16m-08/050, IEEE 802.16m Amendment Working Document

- IEEE 802.16m-09/386, Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control.

[Reply Comment on comment # 213: Include the blue text in the following Sections of the IEEE 802.16m Amendment Working Document [1].]

# Uplink Control Information Content

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified into the following categories: (1) Channel quality feedback, (2) MIMO feedback, (3) HARQ feedback (ACK/NACK), (4) Uplink synchronization signals, (5) Bandwidth requests, and (6) E-MBS feedback and 7) Fractional Frequency Reuse (FFR) band selection feedback (DRU only).

# Fast Feedback Control Channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

In order to support subband based MIMO feedback modes in 15.3.6.2.8.4, both PFBCH and SFBCH can be allocated to the AMS to send feedback, while only PFBCH can be allocated for wideband based MIMO feedback mode.

# Primary Fast Feedback Control Channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback and MIMO feedback. PFBCH is semi statically allocated to AMS by higher layer to send periodic feedback such as CQI, RI and PMI (TBD), where each feedback content has pre-configured feedback frequency.

15.3.9.3.1.1.1		Primary Fast Feedback Control Channel Feedback Mode

Table UL-Pxx1 shows feedback contents to be carried over PFBCH to support Frequency Selective Scheduling (FSS), Fractional Frequency Reuse (FFR) and MIMO feedback modes in 15.3.6.2.8.4 (note: dependency on MIMO group decision). The detailed PFBCH Feedback mode is TBD.

BW-REQ Indicator (TBD), Event-driven SFBCH allocation indicator, and FFR band selection Indicator are sent when needed.

Table UL-Pxx1. PFBCH Feedback Content

| PFBCH Feedback Content | Parameters (TBD) | Note |
| --- | --- | --- |
| Wideband CQI |  | Sent for all MIMO Feedback mode |
| Rank Indicator |  | Sent for MIMO Feedback mode 0 – 5 OL/CL SU MIMO (wideband/subband) |
| Bandwidth Request Indicator (TBD) |  | This is used to request UL bandwidth. |
| Event Driven SFBCH allocation indicator |  | This indicates that AMS requests SFBCH allocation triggered by some event. |
| WB PMI (TBD) |  | Sent for MIMO Feedback mode 4,5,7,8 CL SU/MU MIMO (wideband/subband) (Note: dependency on MIMO DG decision) |
| FFR band selection Indicator |  | Band selection for FFR to tell ABS which subband an AMS sends CQI feedback in DRU mode. |