| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Bandwidth Request Procedure |  |
| Date Submitted | 2009-03-06 |  |
| Source(s) | Hua Xu, Stavros Tzavidas, Igor Filipovich | Hua.xu@motorola.com |
| Re: | Call for Contributions and Comments on Project 802.16m AWD, IEEE 80216m-08/050, ULCtl Draft |  |
| Abstract | This contribution proposes text to be adopted as part of the Bandwidth Request Procedure. |  |
| Purpose | For review and adoption of proposed text in AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Bandwidth Request Procedure

Hua Xu, Stavros Tzavidas, Igor Filipovich

Motorola

# Introduction

In this contribution we are proposing text for the Bandwidth Request procedure.

# Text Proposal

It is proposed that the following text be included in the section related to the description of the Bandwidth Request Procedure:

============================== Start of Proposed Text =================

x.x  Bandwidth Request Procedure

The random access based bandwidth request procedure is described in Figure x.x. It consists of either a regular 5-step procedure (steps 1 to 5) or an optional 3-step quick access procedure. In the case of 3-step quick access procedure steps 2 and 3 are not executed. The 5-step regular procedure is used independently by the AMS or as a fallback for the 3-step bandwidth request quick access procedure.

Figure UL-8 Bandwidth Request Procedure

- Step 1: The AMS sends a Bandwidth Request Indicator (BW_REQ_IND) and a message for quick access that may indicate information such as:

- AMS addressing and/or

- request size and/or

- uplink transmit power report (FFS), and/or

- QoS identifiers (FFS),

- Step 2: The ABS shall transmit Bandwidth Request Indicator Acknowledgment (BW_REQ_ACK) message within a pre-defined period of time, and may allocate uplink grant for each of the acknowledged Bandwidth Request Indicators (it may do so based on certain policy).

- Step 3: The AMS transmits Bandwidth Request Message (BW_REQ_MSG) and if there is enough room in the UL grant it can piggyback additional user data.

- Step 4: The ABS shall acknowledge the reception of Bandwidth Request Message (BW_REQ_MSG) and allocate UL grant.

- Step 5: The AMS transmits user data and optionally BW_REQ_MSG

If the AMS does not receive any acknowledgement or UL grant upon transmission of the BW_REQ_IND  it shall wait until the expiration of a pre-defined period and restart the bandwidth request procedure. The ABS should indicate in the BW_REQ_ACK message that no more acknowledgments are to follow in the current pre-defined period in which case the AMS shall restart the BW Request Procedure in the first available opportunity.

============================== End of Proposed Text ==================