| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Sub-band CQI Feedback |  |
| Date Submitted | 2009-03-06 |  |
| Source(s) | Vinod Ramachandran, Suresh Kalyanasundaram, Hua Xu Motorola | Hua.xu@motorola.com |
| Re: | IEEE 802.16m-09/0012: Call for Contributions and Comments on Project 802.16m AWD, IEEE 80216m-08/050, ULCtl Draft |  |
| Abstract | This contribution proposes Sub-band CQI feedback reporting modes. |  |
| Purpose | For review and adoption of proposed text in AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Sub-band CQI Feedback

Vinod Ramachandran, Suresh Kalyanasundaram, Hua Xu

Motorola Inc.,

1. Introduction

This contribution proposes two modes of receiving sub-band CQI information from mobile stations in IEEE 802.16m. The solutions are aimed at reducing the overhead in the uplink feedback channel. We also propose a text for the amendment working document.

2. Modes of Sub-band CQI Reporting

We classify sub-band CQI reporting in to two modes, namely periodic sub-band CQI reporting and on-demand sub-band CQI reporting.

A. Periodic Sub-band CQI Reporting

In this mode, the mobile stations report sub-band CQI periodically to the base station. In every reporting opportunity, each mobile station computes and sends the CQI of its best K sub-bands. By best sub-bands, we mean the sub-bands with the highest channel quality. The value of K and the periodicity of reporting can be configured by the base station. It should be noted that the value of K and the periodicity can be different for the different mobile stations. For example, mobile station 1 may report the CQI of its best two sub-bands while mobile station 2 may report the CQI of its best five sub-bands. The value of K for each mobile station can depend on several parameters e.g., the QoS of the mobile station.

B. On-demand Sub-band CQI Reporting

In addition to the periodic CQI reporting, it may be useful to have an on-demand CQI reporting where the base station requests the mobile stations to report sub-band CQI. The proposed sub-band CQI feedback scheme is base station controlled i.e., the base station determines the number of best sub-bands for which the mobile stations need to report the CQI. In this mode, each mobile station may be asked to report the CQI of a variable number of its best sub-bands. For example, mobile station 1 may be asked to report the CQI of its best two sub-bands while mobile station 2 may be asked to report the CQI of its best five sub-bands. The number of best sub-bands for which CQI is requested from each of the mobile stations is decided by the base station.

A mobile station that is definitely going to be scheduled within the next few frames may be made an on-demand CQI request. The base station may determine that the mobile station needs to be scheduled in the next few frames based on different parameters such as the QoS requirements of the mobile station, the throughput that the mobile station has received so far, and high priority message queued in the buffer. In addition, the base station may request a mobile station to report the CQI of a fewer number of its best sub-bands, when the mobile station is located higher in the scheduler’s priority list and therefore, likely to be allocated its best sub-bands.  In addition to the above parameters, the number of sub-bands can also be determined based on the available uplink feedback channel bandwidth. The determined number of sub-bands may be communicated to the mobile station as part of a CQI request.

3. Proposed Text

We propose that the following text be included in the IEEE 802.16m amendment working document:

x.x.x	Channel quality feedback

There are two modes of sub-band CQI reporting, namely, periodic reporting and on-demand reporting.

x.x.x.1	Periodic sub-band CQI reporting

In this mode, the mobile stations shall periodically report the CQI of their best K sub-bands. The value of K and the periodicity can be different for each mobile station. The periodicity of reporting and the value of K for each mobile station shall be configured by the base station through a dedicated CQI-Config message, whose contents are shown in Table X1.

Table X1. CQI-Config message content

| Message Content | Description | Values | Size (in bits) |
| --- | --- | --- | --- |
| Periodicity | Periodicity of CQI reporting from mobile stations | 5, 10, 20, 40, 80, 160, 256, OFF (in ms) | 3 |
| FrameOffset | Offset in number of frames | Integer (0 to 255) | 8 |
| NumberofBestSubbands, K | Number of best sub-bands for which CQI shall be reported by the mobile station | Integer (1 to N) N is the total number of sub-bands | log2(N) |

The contents of the periodic sub-band CQI reporting message from a mobile station is shown in Table X2.

Table X2. Periodic sub-band CQI reporting message content

| Message Content | Description | Values | Size (in bits) |
| --- | --- | --- | --- |
| SubbandIdentities | IDs of the sub-bands for which the CQI are being reported | K values in the range of Integer (1 to N) N is the total number of sub-bands | K*log2(N) |
| CQIIndices | Actual CQI indices for each of the sub-bands | FFS | K*4 (4 bits for each sub-band) |

x.x.x.2	On-demand sub-band CQI reporting

In this mode, the base station shall explicitly request a mobile station for a one-time report of the CQI of its best K sub-bands. The number of sub-bands (K) for which the CQI is requested can be different for each mobile station and shall be determined by the base station. The contents of the on-demand CQI request are shown in Table X3.

Table X3. On-demand CQI request message content

| Message Content | Description | Values | Size (in bits) |
| --- | --- | --- | --- |
| NumberofBestSubbands, K | Number of best sub-bands for which CQI shall be reported by the mobile station | Integer (1 to N) N is the total number of sub-bands | log2(N) |

The actual CQI report sent by a mobile station in response to the on-demand CQI request is the same as shown in Table X2.

============================== End of Proposed Text ===============