| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional material related to the "Recommended AWD Text Proposal #4 " in C8021.6m-09/0558 (AMAP Resource Mapping) |  |
| Date Submitted | 2009-03-8 |  |
| Source(s) | Yi Hsuan, Roshni Srinivasan, Shantidev Mohanty, Hujun Yin, Sassan Ahmadi Intel Corporation Jinsoo Choi, Sunam Kim, Kiseon Ryu, Jin Sam Kwak LG Electronics Hyunkyu Yu, Jaeweon Cho, Heewon Kang, Hokyu Choi Samsung Electronics | yi.hsuan@intel.com emptylie@lge.com hk.yu@samsung.com |
| Re: | In reply to DL control DG comments |  |
| Abstract | This contribution addresses additional material related to the "Recommended AWD Text Proposal #4 " in "C80216m-09_0558.doc" (AMAP Resource Mapping) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional Material Related to the "Recommended AWD Text Proposal #4 " in "C80216m-09_0558.doc" (AMAP Resource Mapping)

Yi Hsuan, Roshni Srinivasan, Shantidev Mohanty, Hujun Yin, Sassan Ahmadi

Intel Corporation

Jinsoo Choi, Sunam Kim, Kiseon Ryu, Jin Sam Kwak

LG Electronics

Hyunkyu Yu, Jaeweon Cho, Heewon Kang, Hokyu Choi

Samsung Electronics

- Introduction

As suggested in comment 137 in 80216m-09_0014, this contribution addresses additional material related to the "Recommended AWD Text Proposal #4 " in "C80216m-09_0558.doc" or its latest revision.

Figures 4 and 5 in C8021.6m-09/0558 have been updated to include repetition and text has been updated in 15.3.6.4.2.2.

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

# DL Control Structure

# Advanced Preamble

# DL Control Channels

# Advanced MAP (A-MAP)

# Resource Mapping of DL Control Channels

# Superframe Header

# Advanced MAP (A-MAP)

SFBC with precoding shall be used for A-MAP region

[  ------------------- Recommended AWD Text Proposal #4 --------------------]

# Non-user Specific A-MAP

Figure 4 – Chain of Non-user specific A-MAP-IE to A-A-MAP symbols

The Non-user specific A-MAP bit sequence shall be encoded with a fixed MCS.

The encoded sequences shall be repeated NRep, NS-A-MAP times.

The repeated bit sequences shall be modulated using QPSK.

# Assignment A-MAP

The Assignment A-MAP (A-A-MAP) shall include one or multiple A-A-MAP-IEs and each of them is encoded separately. Figure  describes the procedure for constructing A-A-MAP symbols.

Figure 5 – Chain of A-A-MAP-IE to A-A-MAP symbols

Each A-A-MAP IE shall be appended with 16-bit CRC.

The resulting sequence of bits shall be encoded by a TBCC channel encoder.

Depending on the assignment IE size, a rate matching function may be applied.

Coded bits can be repeated to improve the robustness of an A-A-MAP channel based on the link condition of a particular AMS.

After rate matching and repetition, the encoded bit sequences shall be modulated using QPSK. For a given system configuration, assignment A-MAP IEs can be encoded with two different effective code rates. The exact code rates are TBD.

# HARQ Feedback A-MAP

HARQ feedback A-MAP (HF-A-MAP) contains HARQ-feedback-IEs for ACK/NACK feedback information to uplink data transmission.

Figure 7 – Chain of HF-A-MAP IE to HF-A-MAP symbols

Figure 7 shows the construction procedure of HF-A-MAP symbols from HF-A-MAP-IE.

# Power Control A-MAP

Power Control A-MAP (PC-A-MAP) contains PC-A-MAP-IEs for closed-loop power control of the uplink transmission. ABS shall transmit PC-A-MAP-IE to every AMS which operates in closed-loop power control mode.

Figure 7 – Chain of PC-A-MAP IE to PC-A-MAP symbols

Figure 7 shows the construction procedure of PC-A-MAP symbols from PC-A-MAP-IE.

The i-th PC-A-MAP-IE shall have the size of 2 bits according to power correction value.

Then the repetition is performed with , where  is the number of repetition and explicitly signaled.

The repeated bit sequence shall be modulated as a QPSK symbol and be scaled by  (), where  is the number of PC-A-MAP-IEs and  is the value determined by management entity to satisfy the link performance.

[  ----------------End of Recommended AWD Text Proposal #4 -----------------]

-------------------------------  Text End  ---------------------------------------------------