| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional material related to the "Recommended AWD Text Proposal #9" in "C80216m-09_0558.doc (Persistent Allocation A-MAP IE) |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | Shantidev Mohanty, Roshni Srinivasan, Kamran Etemad, Sassan Ahmadi, Aran Bergman Intel Corporation Shashikant Maheshwari, Haihong Zheng, Xiaoyi Wang, Xin Qi, Yousuf Saifullah Nokia Siemens Networks Zexian Li Nokia Hua Xu Motorola Kiran Kuchi, J. Klutto Milleth CeWit Jianmin Lu Huawei Technologies Co.,Ltd. | E-mail: {shantidev.mohanty, roshni.m.srinivasan}@intel.com shashi.maheshwari@nsn.com Zexian.li@nokia.com Hua.xu@motorola.com kkuchi@cewit.org.in lujianmin@huawei.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | The contribution proposes text for the DL Control Structure section for inclusion in the 802.16m amendment. |  |
| Abstract | The resource allocation scheme of 802.16e/Rev2 is extended to enable persistent allocations |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional material related to the "Recommended AWD Text Proposal #9" in "C80216m-09_0558.doc (Persistent Allocation A-MAP IE)

Shantidev Mohanty, Roshni Srinivasan, Kamran Etemad, Sassan Ahmadi, Aran Bergman

Intel Corporation

Shashikant Maheshwari, Haihong Zheng, Xiaoyi Wang, Xin Qi, Yousuf Saifullah,

Nokia Siemens Networks

Zexian Li

Nokia

Hua Xu

Motorola

Kiran Kuchi, J. Klutto Milleth

CeWit

Jianmin Lu

Huawei Technologies Co.,Ltd.

# Introduction

As suggested in comment 142 in 80216m-09_0014, this contribution addresses additional material related to the "Recommended AWD Text Proposal #9 " in "C80216m-09_0558.doc" or its latest revision.

For periodic type of traffic such as VoIP where the packet arrival is predictable, it is not necessary to send the burst assignment signaling for each packet transmission. Burst assignment can be sent once at the beginning to assign a periodically recurring resource to an AMS with a given period.  This type of assignment is referred to as a persistent allocation.  Both the assigned resource and MCS are fixed until the persistent allocation is released or overridden.

The persistent assignment scheme based on the scheme described in 802.16e/REV2 is summarized below:

- The ABS assigns a periodically recurring resource to an AMS by sending a newly defined Persistent A-MAP IE.

- In a series of HARQ packets, the persistent allocation is applicable for the first sub-burst of each packet.

  - HARQ retransmissions are non-persistently assigned

- The ABS de-allocates resources by sending another persistent A-MAP IE (TBD) to terminate the outstanding assignment.

As an example, in Figure 1, the base station can make a DL persistent allocation with a period of 4 frames.  In the first DL sub-frame, the base station transmits the persistent allocation to the mobile station using the newly defined persistent A-MAP IE.  Then, for each occurrence of the persistent allocation, the mobile station attempts to decode a new packet using the resources corresponding to the persistent allocation.

Figure 1 - Persistent Allocation Example

The persistent scheduling A-MAP IE format proposed in this contribution is based on the persistent scheduling IE format of IEEE 802.16e/REV2

- Text proposal for inclusion in the 802.16m amendment as proposed in "C80216m-09_0558.doc" or its latest revision

-------------------------------  Text Start  ---------------------------------------------------

- 4. Abbreviations and acronyms

# DL Control Structure

# Advanced Preamble

# DL Control Channels

# Superframe Header

# Primary Superframe Header

# Secondary Superframe Header

# Advanced MAP (A-MAP)

# Non-user Specific A-MAP

# Assignment A-MAP

# HARQ Feedback A-MAP

# Power Control A-MAP

# Resource Mapping of DL Control Channels

# Superframe Header

# Primary Superframe Header

# Secondary Superframe Header

# Advanced MAP (A-MAP)

# Non-user Specific A-MAP

# Assignment A-MAP

# HARQ Feedback A-MAP

# Power Control A-MAP

# DL Control Information Elements

# Broadcast Control Information Elements

# P-SFH IE

# S-SFH IE

# Unicast Control Information Elements

# Non-user-specific A-MAP IE

# Assignment A-MAP IE

      - Group Resource Assignment A-MAP IE

[  ------------------- Recommended AWD Text Proposal #9 --------------------]

      - [DL PA A-MAP IE

Table XXX: DL Persistent A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL Persistent A-MAP IE () { | -- | -- |
| AMAP type | 4 | DL Persistent A-MAP IE |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| ACK Region Index | 1 | The index of the ACK region associated with all sub-bursts defined in this DL Persistent A-MAP IE |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| Region information | TBD | TBD Information that specifies the region relevant to this persistent scheduling instance |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition S-SFH |
| } | -- | -- |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Duration Indicator | [1] | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| MAP ACK Channel Index | TBD | TBD Depends on MAP ACK channel definition in UL control Index to a MAP ACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this de-allocation. |
| RCID_IE() | variable | Specifies the station ID in RCID format |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| If (Duration Indicator == 1) { | - | - |
| Duration | TBD | Duration in number of LRUs. |
| } |  |  |
| Resource Offset | TBD | Indicates the start of this persistent allocation in the allocation region. |
| If (Persistent Flag == 1) { |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator == 1) { | - | - |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| MAP NACK Channel Index | 6 | TBD Index to a shared MAP NACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| MAP ACK Channel Index | 6 | TBD Index to a MAP ACK channel within the Fast Feedback region |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| If (Allocation MCS indicator ==1) { |  |  |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator ==1) { |  |  |
| Boosting | [3] | Specifies the boosting used for this allocation |
| } |  |  |
| Allocation ACID indicator | 1 | If Allocation ACID Indicator is 1, it indicates that ACID is explicitly assigned for this allocation. Otherwise, this allocation will use the same ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation ACID indicator ==1) { |  |  |
| ACID | [5] | Specifies the HARQ channel identifier used for this allocation |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator ==1) { |  |  |
| Boosting |  |  |
| } |  |  |
| Allocation ACID indicator |  |  |
| If (Allocation ACID indicator ==1) { |  |  |
| ACID | [4][5] | HARQ channel identifier |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| MCRC | [16] | 16 bit masked CRC |
| } |  |  |

Persistent Flag

The persistent flag shall be set to 1 if the assignment is persistent and shall be set to 0 if the assignment is non-persistent.

DRU Offset

The DRU offset shall be set to the first DRU in the time-frequency resource assignment with respect to the lowest numbered OFDM symbol and the lowest numbered subchannel in the Allocation region.

Duration Indicator

Duration Indicator flag determines whether or not Duration is specified for an allocation. If this flag is 1, it indicates that Duration is explicitly assigned for an allocation. Otherwise, the allocation has the same Duration as the previous allocation. This flag shall be 1 for the first allocation in a Allocation region.

Duration

Duration specifies the size (# DRUs) of an allocation/de-allocation/reallocation in a Allocation region.

Allocation Period and N_ACID Indicator

If Allocation Period and Index Indicator is 1, it indicates that allocation period, ACK and NACK channel index (allocation period and Number of ACID (N_ACID) is explicitly assigned for an allocation. Otherwise, the allocation will use the same allocation period and N_ACID as the previous allocation. This flag shall be 1 for the first allocation in a Allocation region.

Allocation Period (ap)

The allocation period (ap) shall be set to one less than the period of the persistent allocation, in units of sub-frames/frames.  For example, as illustrated below, if ap=0b00011, then the period of the persistent allocation is four frames, and the time-frequency resource assignment is valid in frames N, N+4, N+8, etc.

N_ACID

The values of ACID field (N0) and N_ACID field (N) are used together to specify an implicit cycling of HARQ channel identifiers as follows.  N0 is used as the HARQ channel identifier corresponding to the first occurrence of the persistent allocation. For each next allocation this value is incremented modulo (N + 1)

As illustrated below, if N_ACID = 0b011 (meaning the period Num_HARQ_Chan = 4), and if ACID = 2, the HARQ channel identifier follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc.

ACID

The ACID field shall be set to the initial value of HARQ channel identifier as described above.

AI_SN

The AI_SN field value shall be set to the initial ARQ identifier sequence number for each HARQ channel.  The AI_SN toggles between 0 and 1 for each    particular HARQ channel.  For example, if the period equals 4 frames, N_ACID = 4, ACID = 2, and AI_SN = 0, the ACID follows the pattern 2, 3, 4, 5, 2, 3, 4, 5, etc, and the AI_SN follows the pattern 0, 0, 0, 0, 1, 1, 1, 1, etc.

ACK_channel

The ACK_channel field shall be set to the number of the ACK channel within the HARQ ACK Region.  The mobile station shall use the indicated ACK channel for transmitting acknowledgment information for each packet received using the time-frequency resource referred to by this persistent allocation.

MAP NACK Channel Index

The MAP NACK channel index is persistently allocated within the Fast Feedback region.  The mobile station shall used the indicated MAP NACK channel to report MAP decoding error in frames where it as a persistent resource allocation assigned with this instance of the persistent IE. The value 111111 is reserved.  When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation.

MAP ACK Channel Index

The MAP ACK channel is allocated non-persistently within the Fast Feedback region.  The mobile station shall use the indicated MAP ACK channel to report successful receipt of the persistent allocation IE. The value 111111 is reserved.  When MAP ACK Channel Index = 111111, it indicates NO MAP ACK channel is assigned to this allocation

      - UL PA A-MAP IE]

# Power Control A-MAP IE

[  ----------------End of Recommended AWD Text Proposal #9 -----------------]

-------------------------------  Text End  ---------------------------------------------------