| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional material related to the "Recommended AWD Text Proposal #3 " in "C802.16m-09/0558.doc" (P-SFH and S-SFH) |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Mihyun Lee, Rakesh Taori, Hokyu Choi and Heewon Kang Samsung Electronics Roshni Srinivasan and Yi Hsuan Intel Corporation Jinsoo Choi, Hangyu Cho, Sungho Moon and Jin Sam Kwak LG Electronics Yih-Shen Chen and Yu-Hao Chang MediaTek Jungnam Yun, Dongjun Lee, Yerang Hur Posdata Kevin Power and Masato Okuda Fujitsu Xiaoyi Wang and Xin Qi Nokia Siemens Networks Zexian Li Nokia | mihyun.mac.lee@samsung.com roshni.m.srinivasan@intel.com {emptylie, hgcho, msungho, samji}@lge.com kevin.power@uk.fujitsu.com okuda@jp.fujitsu.com |
| Re: | Call for reply comment on IEEE 802.16m-09/0558r1, “Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment.” Specific topic: “The recommended AWD text proposal #3 |  |
| Abstract | The contribution proposes text for the DL Control Structure section for inclusion in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional material related to the "Recommended AWD Text Proposal #3" in "C802.16m-09/0558.doc"(P-SFH and S-SFH)

Mihyun Lee, Rakesh Taori, Hokyu Choi and Heewon Kang

Samsung Electronics

Roshni Srinivasan and Yi Hsuan

Intel Corporation

Jinsoo Choi, Hangyu Cho, Sungho Moon and Jin Sam Kwak

LG Electronics

Yih-Shen Chen and Yu-Hao Chang

MediaTek

Jungnam Yun, Dongjun Lee, Yerang Hur

Posdata

Kevin Power and Masato Okuda

Fujitsu

Xiaoyi Wang and Xin Qi

Nokia Siemens Networks

Zexian Li

Nokia

- Introduction

As suggested in comment 136 in 80216m-09_0014, this contribution addresses additional material related to the "Recommended AWD Text Proposal #3" in "C80216m-09_0558.doc" or its latest revision.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

[  ------------------- Recommended AWD Text Proposal #3 --------------------]

# 15.3.6.4

# Resource Mapping of DL Control Channels

[Editor’s note: This section describes the mapping of information bits to the DL control channel physical resource (subcarriers/symbols), coding, modulation etc.]

# 15.3.6.4.1

# Superframe Header

SFBC shall be used as the transmission scheme for SFH in the case of ABS with two Tx antennas. For more than two Tx antennas, SFBC with precoding, which is decoded by the MS without any information on the precoding and antenna configuration, is used as the transmission scheme for SFH.

# 15.3.6.4.1.1

# Primary Superframe Header

Figure 2 shows the physical processing block diagram for the P-SFH.

The P-SFH IE is processed and then mapped to the P-SFH as shown in figure 2

Figure2– Mapping of P-SFH IE to P-SFH Physical processing block diagram for the P-SFH

The P-SFH IE shall be appended with NCRC, P-SFH bits CRC .The P-SFH IE shall be followed by scramblinged with a cell-specific sequence. The cell-specific sequence is determined indicated by from the A-Ppreamble.

The P-SFH IE resulting sequence of bits shall be encoded by the channel encoder convolutional encoder described in [refer to 15.3.x channel coding section]. A coding rate of 1/2 is used.

The encoded sequences shall be repeated NRep, P-SFH times.

The repeated bit sequences shall be modulated using QPSK.

The modulated symbols shall be mapped to two transmission streams using SFBC for two antennas. The two streams using SFBC may be precoded and mapped to more than two antennas.

Antenna specific symbols at the output of the MIMO encoder/precoder shall be mapped to the resource elements described in 15.3.6.2.1.1

# 15.3.6.4.1.2

# Secondary Superframe Header

Figure 3 shows the physical processing block diagram for the S-SFH.

The S-SFH IE is processed and mapped to S-SFH as shown in figure 3

Figure 3 – Mapping of S-SFH IE to S-SFHPhysical processing block diagram for the S-SFH

The S-SFH IE shall be appended with 16-bit CRC followed by The S-SFH IE shall be scramblinged with a cell-specific sequence. The cell-specific sequence is determined from the by A-Preamble.

The resulting sequence of bits shall be encoded by the channel encoder. The channel encoder for S-SFH is described in [refer to 15.3.x channel coding section]. A code rate of 1/2 or 1/3 is used.

The encoded sequences shall be repeated NRep, S-SFH times.

The repeated bit sequences shall be modulated using QPSK.

The modulated symbols shall be mapped to two transmission streams using SFBC for two antennas. The two stream using SFBC may be precoded and mapped to more than two antennas.

Antenna specific symbols at the output of the MIMO encoder/precoder shall be mapped to the resource elements described in 15.3.6.2.1.2.

--------------------------------------  Text End  ---------------------------------------------------