| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control – Additional Detail for UL Sounding in Proposals #3 and #8 |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Fred Vook, Bishwarup Mondal, Fan Wang, Mark Cudak Motorola | E-mail: fred.vook@motorola.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | TGm UL Control DG on UL Control Draft Amendment text |  |
| Abstract | The contribution proposes additional text for supporting UL Sounding in the draft 802.16m amendment. The proposed text is presented in the form of modifications to the harmonized output of the Drafting Group on the PHY Structure for UL Control (C802.16m-09/0386). |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control

Jong-Kae (JK) Fwu,

TGm UL Control Drafting Group Chair

Jinyoung Chun, Roshni Srinivasan, Yan-Xiu Zheng, Fan Wang, Fred Vook, Bishwarup Mondal, Sophie Vrzic, Adrian Boariu, Pei-Kai Liao, Sukwoo Lee, Hwasun Yoo, Rajni Agarwal, Dong Li, JungNam Yun, Eldad Zeira

TGm UL Control Drafting Group

Introduction

This contribution provides amendment text for the UL Control Structure in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The text was developed by UL Control Drafting group as baseline text from the Draft IEEE 802.16m System Description Document (SDD) [2].

References

- IEEE 802.16m Amendment Working Document, IEEE 802.16m-08/050

- IEEE 802.16m System Description Document (SDD) [Draft], IEEE 802.16m-08/003r6.

Contribution List

| Contribution# | Contribution Title | Affiliate/Participant |
| --- | --- | --- |
| C80216m-09_0198.doc | Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control | Intel/Roshni |
| C80216m-09_0222.doc [223, 224, 225, 046] | Bandwidth Request Channel Physical Layer Design | ITRI/Yan-Xiu Zheng |
| C80216m-09_0238.doc [239, 240, 241, 243] | Proposed Text for UL Sounding Channel in the UL Control Structure Section for the IEEE 802.16m Amendment | Motorola/ Fan Wang |
| C80216m-09_0248.doc C80216m-09_0294.doc | Proposed Text on BW Request for the IEEE 802.16m Amendment, | Nortel/ Sophie Vrzic |
| C80216m-09_0253r1.doc C80216m-09_0254.pdf | Ranging Section for the IEEE 802.16m Amendment, etc | NSN/ Adrian Boariu |
| C80216m-09_0271.doc | Uplink Control Structure for IEEE 802.16m Amendment | MediaTek/ Pei-Kai Liao |
| C80216m-09_0289.doc [290, 291, 292, 293,335r1] | Proposed Text of UL Fast Feedback Channel Sections for the IEEE 802.16m Amendment | LGE/ Sukwoo Lee |
| C80216m-09_0346.pdf | Proposed Text of UL Control Channel for IEEE 802.16m Amendment | Samsung/ Hwasun Yoo |
| C80216m-09_0348.pdf | UL Fast Feedback (FFB) Channel Signalling for 802.16m Amendment | Fujitsu/Rajni Agarwal |
| C80216m-09_0349.pdf | Proposal of sounding channel in 16m (amendment) | Alcatel Shanghai Bell/Dong Li |
| New proposal | JungNam Yun | PosData/ JungNam Yun |
| C80216m-09_0037r1 | Tail Biting Convolutional Codes for the Secondary Fast Feedback Channel with Performance and Complexity Analysis | Ericsson/Jason Chen |
| New proposal | Bandwidth Requst channel structure | InterDigital/ Eldad Zeira |

Proposed Amendment Text

The following text is proposed for inclusion in Section 15 of the IEEE 802.16m Amendment Working Document [1].

[  ------------------- Recommended AWD Text Proposal #1 --------------------]

[DG Comment #1: Accept the recommended text proposal #1 on abbreviations]

4. Abbreviations and acronyms

[Insert the following abbreviations:]

PFBCH	UL primary fast feedback channel

SFBCH	UL secondary fast feedback channel

FMT	 	UL feedback mini-tile

RFMT	 	Reordered UL feedback mini-tile

HMT		UL HARQ mini-tiles

RHMT	 	Reordered UL HARQ mini-tile

RCP 		ranging cyclic prefix

RP		ranging preamble

[  ------------------- Recommended AWD Text Proposal #2 --------------------]

[DG Comment #2: Accept the recommended text proposal #2 on PHY Structure for UL Control Support]

[Editor’s note: The following section (15.3.6.3.4) is the proposed text to be included in UL PHY section which is related to UL Control related PHY Structure support]

[Editor’s note: the following sections are to be inserted in AWD after section 15.3.8.3.3 Tile permutation]

# Resource Allocation and Tile Permutation for Control Channels

The distributed LRUs in each of uplink frequency partition may be further divided into data, bandwidth request and feedback channels. The feedback channels can be used for both HARQ ACK/NAK and fast feedback. The allocation order of data channels and UL control channels are TBD.

# Bandwidth Request Channels

The number of bandwidth request channels in frequency partition FPi, LBWR,FPi, is indicated by the (TBD)-bit field UL_BWREQ_SIZE in the S-SFH (TBD) in the unit of LRUs.

LBWR,FPi=UL_BWREQ_SIZE							Eqn. UL-

Bandwidth request channels are not necessarily present in all subframes and the allocation can differ from subframe to next.

In MZone, the bandwidth request channels are of same size as LRUs, i.e. three 6-by-6 tiles. In LZone with PUSC, the bandwidth request channels consist of three 4-by-6 tiles. The bandwidth request channels use LRUs constructed from the tile permutation specified in Section 15.3.6.3.3.

# Feedback Channels

Let UL_FEEDBACK_SIZE distributed LRUs in frequency partition FPi be reserved for feedback channels in the units of LRU. The number of feedback channels in frequency partition FPi, is LFB,FPi, and.

LFB,FPi=Nfb×UL_FEEDBACK_SIZE							Eqn. UL-

where

Nfb is 3 in MZone and 4 in LZone with PUSC.

The feedback channels are formed by 3 permuted 2-by-6 mini-tiles.  The mini-tile reordering process applied to each distributed LRU is described below and illustrated in Figure UL- .

- The uplink tiles in the distributed LRUs reserved for feedback channels are divided into 2-by-6 feedback mini-tiles (FMTs). The FMTs so obtained are numbered from 0 to 3LFB,FPi−1.

- A mini-tile reordering is applied to the available 2-by-6 FMTs as specified by    Eqn. UL-  and Eqn. UL-4 to obtain the reordered FMTs (RFMTs).

- Each group of three consecutive RFMTs forms a feedback channel.

The closed form expressions for the FMT reordering function used in step 2 above are as

Eqn. UL-  in MZone and Eqn. UL-  in the LZone with PUSC:

MiniTile(s,n)=9∗floor(s3)+mod(s,3)+3∗n			Eqn. UL-

MiniTile(s,n)=6∗floor(s2)+mod(s,2)+2∗n				Eqn. UL-

Where

MiniTile(s,n) is the n-th mini-tile of the s-th feedback channel.

n is the mini-tile index in a feedback channel. n can take a value 0, 1 or 2.

s is the feedback channel index.  s can take an integer value in the range 0 to LFB,FPi-1.

# HARQ Feedback Channels

Each feedback channel constructed according to Section 15.3.8.3.4.2 can be used to transmit six HARQ feedback channels. The number of HARQ feedback channels is denoted by LHFB,FPi.

A HARQ feedback channel is formed by three reordered 2-by-2 HARQ mini-tiles (RHMT). The HMTs reordering process and the construction of HARQ feedback channel are described below and illustrated in Figure UL-1.

- Each 2x6 RFMT is divided into three consequitivly indexed 2-by-2 HMTs. The HMTs so obtained are numbered from 0 to 3LHFB,FPi−1.]

- A HMT reordering is applied to the HMTs as specified by Eqn. UL-5 to obtain the reorderd HMTs (RHMTs).

- Each group of three consecutive RHMTs forms a HARQ feedback channel.

The closed form expression for the HMT reordering function used in step 2 above is as  Eqn. UL- .

HMT(k,m)=9×floor(k'3)+mod(k'+m,3)+3×m Eqn. UL-

where

HMT(k,m)is the m-th HMT of the k-th HARQ feedback channel.

m is the HMT index in a HARQ feedback channel. m can take a value 0, 1 or 2.

k is the HARQ feedback channel index. k can take an integer value in the range 0 to LHFB,FPi -1.

k'=k/2

Figure UL- , an example of UL resource allocation for UL control channels. The displayed order of data channels and control channels is for illustrative purpose only.

# Fast Feedback Channels

A fast feedback channel consists of one feedback channel.

[  ------------------- Recommended AWD Text Proposal #3 --------------------]

[DG Comment #3: Accept the recommended text proposal #3 on Physical UL Control Channel (FFB, HARQ, Sounding)]

# Uplink Control Channel

# Physical Uplink Control Channel

# Fast Feedback Control Channel

The DRUs are permuted by UL tile permutation as described in Section 15.3.5.4 to form distributed LRUs for for both data and control resource/channel. A UL feedback mini-tile (FMT) is defined as 2 contiguous subcarriers by 6 OFDM symbols. The UL feedback control channels are formed by applying the UL mini-tile permutation to the LRUs allocated to the control resource. The fast feedback channels are comprised of 3 RFMTs. The details of feedback mini-tile permutation and the subchannelization of Fast feedback are described in section 15.3.8.3.4.2.

# Primary Fast Feedback Control Channel

The primary fast feedback channel is comprised of 3 RFMTs. The construction process of primary fast feedback channels is described in section 15.3.8.3.4.2.

# Secondary Fast Feedback Control Channel

The secondary fast feedback channel has the same physical control channel structure as the primary fast feedback channel. The secondary fast feedback channels are comprised of 3 RFMTs. The construction process of secondary fast feedback is described in section 15.3.8.3.4.2.

# HARQ Feedback Control Channel

Each UL HARQ feedback resource consists of three distributed UL reordered feedback mini-tiles (RFMTs), where the UL FMT is defined as 2 contiguous subcarriers by 6 OFDM symbols. The procedures for allocation of resources for transmission of UL control information and the formation of control channels for such transmission are described in section 15.3.8.3.4.2. A total resource of three distributed 2x6 RFMTs supports 6 UL HARQ feedback channels. The 2x6 RFMTs are further divided into UL HARQ mini-tiles (HMT). A UL HARQ mini-tile has a structure of 2 subcarriers by 2 OFDM symbols.

# Sounding Channel

Uplink channel sounding provides the means for the ABS to determine UL channel response for the purpose of UL closed-loop MIMO transmission and UL scheduling. In TDD systems, the ABS can also use the estimated UL channel response to perform DL closed-loop transmission to improve system throughput, coverage and link reliability. In this case ABS can translate the measured UL channel response to an estimated DL channel response when the transmitter and receiver hardware of ABS and AMS are appropriately calibrated.

# Sounding PHY structure

Normal Sounding Channel:  The sounding signal occupies a single OFDMA symbol in the UL sub-frame. The sounding symbol in the UL sub-frame is located in the [first] symbol. Each UL sub-frame can contain only one sounding symbol. Multiple UL subframes in a 5-ms radio frame can be used for sounding. The number of subcarriers for the sounding in a PRU is 18 adjacent subcarriers.

Distributed Sounding Channel:  To improve the coverage performance of the sounding channel, the sounding channel can alternatively be configured to be distributed over several OFDM symbols within a subframe as shown in Figure UL-X.  For a distributed sounding channel, the subframe is divided into 6×Nsym sounding tiles, where Nsym is the number of symbols in the subframe.  Each 6×Nsym tile has a sounding channel format shown in Figure UL-X, where the sounding channel in a subframe either employs pattern A or pattern B for the sounding tiles.  The PRUs for the purposes of UL Sounding consist of three consecutive sounding tiles in frequency.  The subframe that contains the distributed sounding channel is also used for UL data transmissions.

Coexistence of Distributed Sounding Channel with UL Data transmissions:  For AMSs assigned to transmit on a sounding tile, the AMS transmits sounding only on the indicated sounding pilots; the blank subcarriers/symbols are not transmitted by the sounding AMS.  AMSs assigned to transmit data on subframes that include a distributed sounding channel do not transmit on the subcarriers/symbols that correspond to the sounding pilots.  For SFBC data transmissions on UL PRUs that coexist with a sounding channel, the mapping of the SFBC-encoded data symbols to the subcarriers of the tile is [TBD].

Figure UL-X – Distributed Sounding Channel Design.  Pattern A (left) and Pattern B (right).

[  ------------------- Recommended AWD Text Proposal #4 --------------------]

[DG Comment #4: Accept the recommended text proposal #4 on Physical UL Control Channel (Ranging, BW REQ)]

# Ranging Channel

The UL ranging channel is used for UL synchronization. The UL ranging channel can be further classified into ranging channel for non-synchronized mobile stations and synchronized mobiles stations. The ranging channel for synchronized AMSs is used for periodic ranging. The ranging channel for non-synchronized AMSs is used for initial access and handover.

# Ranging Channel Structure for Non-synchronized AMSs

The ranging channel for non-synchronized AMSs is used for initial network entry and association and for ranging against a target BS during handover.

A physical ranging channel for non-synchronized AMSs consists of the ranging preamble (RP) with length of TRP depending on the ranging subcarrier spacing ∆fRP, and the ranging cyclic prefix (RCP) with length of TRCP in the time domain.

A ranging channel occupies a localized bandwidth corresponding to the [1 or 2] subbands.

Power control operation described in subclause [TBD] applies to ranging signal transmission.

Figure UL-  illustrates the ranging channel structures in the time domain.

(a) Structure 1

(b) Structure 2

- Structure 3

Figure UL- , The ranging channel structures in the time domain.

Table UL-  shows [4] ranging channel formats and parameters.

Table UL- , Ranging Channel Formats and Parameters.

| Format No. | Ranging Channel Format | TRCP | TRP | ∆fRP |
| --- | --- | --- | --- | --- |
| 0 | Structure 1 |  |  |  |
| 1 | Structure 2 |  |  |  |
| 2 | Structure 3 |  |  |  |
| … | … |  |  |  |

In the ranging channel Format 0, the repeated RCPs and RPs are used as a single time ranging opportunity within a subframe in Figure UL- . Format 1 consists of a single RCP and repeated RPs within a subframe. Format 2 consists of a single RCP and RP.

For the ranging opportunity of the non-synchronized AMS, each AMS randomly chooses one of ranging preamble sequences from the available ranging sequence set in a cell defined in TBD Ranging preamble codes.

# Ranging Channel for Synchronized AMSs

The ranging channel for synchronized AMSs is used for periodic ranging. The transmission of ranging channel for synchronized AMSs shall be sent only by the AMSs that have already synchronized to the system.

Power control operation described in subclause [TBD] applies to ranging signal transmission.

# Bandwidth request Channel

In the LZone with PUSC, a BW REQ tile is defined as 4 contiguous subcarriers by 6 OFDM symbols. The number of BW REQ tiles per BW REQ channel is 3 and each BW REQ tile carries preamble only.

In the MZone, a BW REQ tile is defined as 6 contiguous subcarriers by 6 OFDM symbols. Each BW REQ channel consists of 3 distributed BW REQ tiles. Each BW REQ tile carries a BW REQ access sequence and the BW REQ message.

[  ------------------- Recommended AWD Text Proposal #5 --------------------]

[DG Comment #5: Accept the recommended text proposal #5 on UL Control Channel (PFBCH)]

# Uplink Control Channels Physical Resource Mapping

# Fast Feedback Control Channel

There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH).

# Primary Fast Feedback Control Channel

The primary fast feedback channels are comprised of 3 distributed FMTs. Figure UL-3 illustrates the mapping of the PFBCH.

Figure UL- , PFBCH comprised of three distributed 2x6 UL FMTs.

Figure UL- , Mapping of information in the PFBCH.

The process of composing the PFBCH is illustrated in Figure UL- . The l PFBCH payload bits are used to generate PFBCH sequence according to Table UL- . The resulting bit sequence is modulated [, repeated] and mapped to uplink PFBCH symbol s[k]. The mapping of primary fast feedback channel symbol s[k] to the UL FMTs is given by Eqn. UL- . This set of sequence can carrier up to 6 information bits.

Ci,j=s[k],fori=0,1,2,0≤j≤,11,and0≤k≤,11, Eqn. UL-

Table UL- , Sequences for PFBCH.

| Index | Sequence | Index | Sequence |
| --- | --- | --- | --- |
| 0 |  | 32 |  |
| 1 |  | 33 |  |
| 2 |  | 34 |  |
| … | … | … | … |
| 30 |  | 62 |  |
| 31 |  | 63 |  |

[  ------------------- Recommended AWD Text Proposal #6 --------------------]

[DG Comment #6: Accept the recommended text proposal #6 on UL Control Channel (SFBCH)]

# Secondary Fast Feedback Control Channel

The SFBCH is comprised of 3 distributed FMTs with [2] [4] pilots allocated in each FMT. Pilot sequence is TBD.

The SFBCH symbol generation procedure is shown in Figure UL- . First, the SFBCH payload information bits a0a1a2⋯al−1 are encoded to N bits b0b1b2⋯bN−1using the linear block codes described in Section 15.3.9.2.1.2.1.

For l≤12, information bits a0a1a2⋯al−1 are encoded using the linear block code (N,l).

For 12<l≤24, information bits a0a1a2⋯al−1are split into 2 parts: Part A consisting of a0a1a2⋯al2−1, and Part B consisting of al2al2+1al2+2⋯al−1. Part A is encoded to N2bits b0b1b2⋯bN2−1using linear block code (N2,l2) and Part B is encoded to N2bits bN2bN2+1bN2+2⋯bN−1 using a linear block code(N2,l−l2). The coded sequence b0b1b2⋯bNis then modulated to N2symbols c0c1c2⋯cN2−1using QPSK. The value of N is TBD.  The modulated symbols c0c1c2⋯cN2−1 and pilot sequence (TBD) are combined to form sequence d0d1d2⋯d35 and then mapped to the data subcarriers of the SFBCH FMTs as shown in Figure UL- .

Figure UL- , Mapping of information in the SFBCH.

Figure UL- , SFBCH comprising of three distributed 2x6 UL FMTs.

# Channel coding for Secondary Fast Feedback Control Channel

The k (7≤K≤12) information bits in the SFBCH shall be encoded using linear block codes with codeword length. Let the K information bits be denoted by a0,a1,…,aK−1and the N bits codeword is denoted byb0,b1,…,bN−1. The codeword is obtained as a linear combination of the N basis sequences denoted asvi,n where n=0,⋯,K−1, i=0,⋯,N−1 in Table UL- .

Table UL- , Basis sequences.

| i | vi,0 | vi,1 | vi,2 | vi,3 | vi,4 | vi,5 | vi,6 | vi,7 | vi,8 | vi,9 | vi,10 | vi,11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |
| N-1 |  |  |  |  |  |  |  |  |  |  |  |  |

[  ------------------- Recommended AWD Text Proposal #7 --------------------]

[DG Comment #7: Accept the recommended text proposal #7 on UL Control Channel (HARQ)]

# HARQ Feedback Control Channel

The HARQ feedback control channel resource of 3 distributed FMTs shall be further divided into 9 HARQ mini-tiles (HMTs), each having a structure of 2 subcarrier ×2 OFDM symbols. Each pair of HARQ feedback channels are allocated 3 HMTs, identified by similar patterns in the structure shown in Figure UL- . The orthogonal sequence (Ci,0, C i,1, C i,2, C i,3, where i=0,1 and 2) as shown in Table UL- 4 is mapped to each HMT to form HARQ feedback channels, where and i denotes HMT index. Each group of 3 RFMTs can therefore support 6 HARQ feedback channels.

When each channel carries one bit of HARQ feedback, two sequences are used to signal each ACK or NACK feedback. In one unit, four sequences are used for two HARQ channels, 1st and 2nd HARQ feedback channel. The support and details of two bit HARQ feedback scenatios are TBD. The sequence and mapping of the HARQ feedback are show in Table UL- .

Figure UL- , 2x2 HMT structure.

Table UL- , Orthogonal sequences for UL HARQ Feedback Channel.

| Sequence index | Orthogonal sequence | 1-bit Feedabck |
| --- | --- | --- |
| 0 | [+1 +1 +1 +1] +1] | Even numbered channel ACK |
| 1 | [+1 -1 +1 -1] | Even numbered channel NACK |
| 2 | [+1 +1 -1 -1] | Odd numbered channel ACK |
| 3 | [+1 -1 -1 +1] | Odd numbered channel NACK |

[  ------------------- Recommended AWD Text Proposal #8 --------------------]

[DG Comment #8: Accept the recommended text proposal #8 on UL Control Channel (Sounding)]

# Sounding Channel

Determining the sounding waveform to be sent by an AMS consists of four steps:  First, an AMS is assigned to sound within the sounding channel within a specified subframe.  Second, in the sounding channel within the assigned subframe, the basic unit of sounding is the PRU, and the AMS is assigned a set of PRUs to be occupied during sounding as defined in Section 15.3.9.2.3.2.  Third, within the occupied PRUs, one or more AMSs may be multiplexed to sound within the PRU, and the AMS is assigned the relevant multiplexing parameters and sequence to use within the PRU as defined in Section 15.3.9.2.3.3 and Section 15.3.9.2.3.4.  The methodology for informing the AMS of the sounding parameters is described in Section 15.3.9.2.3.1.

# Sounding Signaling Methodology

Sounding Request:  The Sounding Request commands a MS to transmit a sounding waveform on one or more sounding channels.  Details are TBD.  The Sounding Request provides the sounding parameters necessary for the AMS to determine the exact sounding waveform.

Sounding Request Location:  The Sounding Request can be contained in an independent control message in the user-specific-control-header.  The Sounding Request can also be piggybacked in a downlink data allocation assignment.  Details are TBD.

Support for Persistent Sounding:  The sounding request can set up a persistent sounding response in which the MS will periodically transmit its sounding response at some defined periodicity interval.  The sounding request can also terminate the persistent sounding response.  The methodology used to setup and tear down persistent sounding is patterned after the methodology used for persistent data allocations.  Details are TBD.

# Sounding Allocation Methodology

The PRUs to be occupied within the sounding channel is determined via one of three methodologies:

Downlink Allocation Matched:  In the “Downlink Allocation Matched” methodology, the PRUs to be included in a sounding allocation are defined in the same way that the PRUs in a downlink data allocation are defined.  The Downlink-Allocation-Matched methodology may be used to enable the MS to transmit the UL sounding waveform on the same physical frequency region that the MS will receive data on.

Uplink Allocation Matched:  In the “Uplink Allocation Matched” methodology, the SRUs that belong to a sounding allocation are defined in the same way that the PRUs of an uplink allocation are defined.  The Uplink-Allocation-Matched methodology may be used to enable the MS to transmit the UL sounding waveform on the same PRUs that the MS will transmit data on, e.g., in support of codebook-feedback based closed-loop MIMO transmissions on the UL.

Wideband Allocation Mode:  In the wideband allocation mode, the sounding allocation consists of all PRUs in the Sounding Symbol.  A decimation rate and decimation offset is specified in the sounding request, where a decimation rate of N means every Nth subcarrier is occupied by the sounding response starting from the subcarrier corresponding to the decimation offset.

# Sounding sequence

Sector-specific sounding sequences shall be used.  The sequence design is TBD.

# Multiplexing for multi-antenna and multi-AMS

The uplink sounding channels of multiple AMS and multiple antennas per AMS can be multiplexed through [Option 1: decimation separation or cyclic shift separation] in each sounding allocation. Also, in case of multiple UL subframes for sounding, time division separation can be applied by assigning different AMS to different UL subframe. For cyclic shift separation each AMS occupies all subcarriers within sounding allocation and uses the different sounding waveform. For frequency decimation separation each AMS uses decimated subcarrier subset from the sounding allocation set with different frequency offset.

[  ------------------- Recommended AWD Text Proposal #9 --------------------]

[DG Comment #9: Accept the recommended text proposal #9 on UL Control Channel (Ranging)]

# Ranging Channel

# Ranging Channel for Non-synchronized AMSs

# Ranging Preamble Codes

The ranging preamble codes are classified into initial ranging and handover ranging preamble codes. The initial ranging preamble codes shall be used for initial network entry and association. Handover ranging preamble codes shall be used for ranging against a target ABS during handover. For a ranging code opportunity, each AMS randomly chooses one of the ranging preamble codes from the available ranging preamble codes set in a cell.

[The Zadoff-Chu sequences with cyclic shifts are used for the ranging preamble codes.]

# Ranging channel configurations

[Note: Based on the decision of the ranging channel structures, this subclause describes the ranging opportunities and/or corresponding control signaling.]

# Ranging signal transmission

Eqn. UL-  specifies the transmitted signal voltage to the antenna, as a function of time, during ranging channel format.

s(t)=franging() 			Eqn. UL-

where

franging()is TBD function

# Ranging Channel for Synchronized AMS

# Ranging Codes

# Ranging channel configurations

[  ------------------- Recommended AWD Text Proposal #10 --------------------]

[DG Comment #10: Accept the recommended text proposal #10 on UL Control Channel (BW REQ)]

# Bandwidth request Channel

Contention based random access is used to transmit bandwidth request information on this control channel.The bandwidth request (BW REQ) channel contains resources for the AMS to send a BW REQ access sequence and an optional quick access message. Prioritized bandwidth requests are supported on this channel. The mechanism for such prioritization is TBD.

The random access based bandwidth request procedure is described in Figure UL- . A 5-step regular procedure (step 1 to 5) or an optional 3-step quick access procedure (step 1, 4 and 5) may be supported concurrently. Step 2 and 3 are used only in 5-step regular procedure. In step 1, AMS sends a bandwidth request indicator and a message for quick access. The 5-step regular procedure is used independently or as a fallback mode for the 3-step bandwidth request quick access procedure. The AMS may piggyback additional BW REQ information along with user data during uplink transmission (step 5).  Following Step 1 and Step 3, ABS may acknowledge the reception of bandwidth request. If AMS does not receive any acknowledgement or UL grant, it waits until the expiration of a pre-defined period and restarts the bandwidth request. The pre-defined period may be differentiated by factors such as QoS parameters (e.g. scheduling type, priority, etc). In case BW is granted immediately, there is no need for ABS to send explicit ACK.

Figure UL- , Bandwidth Request Procedure.

As shown in Figure UL- , in the LZone with PUSC, one BW REQ tile consists of 4 contiguous subcarriers by 6 OFDM symbols. The total 72 subcarriers of one LZone BRCH are indexed firstly in frequency, secondly in OFDM symbol and lastly in tile as in Figure UL- .

In the MZone, the total 108 subcarriers of one MZone BW REQ channel are indexed one tile at a time, firstly in frequency, then in OFDMA symbol as in Figure UL- .

Figure UL- , 4x6 BW REQ tile structure.

Figure UL- , 6x6 BW REQ Tile Structure in the Advance Air Interface.

[  ------------------- Recommended AWD Text Proposal #11 --------------------]

[DG Comment #11: Accept the recommended text proposal #11 on UL Control Information Content (PFBCH)]

# Uplink Control Information Content

The UL control channels carry multiple types of control information to support air interface procedures. Information carried in the control channels is classified into the following categories: (1) Channel quality feedback, (2) MIMO feedback, (3) HARQ feedback (ACK/NACK), (4) Uplink synchronization signals, (5) Bandwidth requests and (6) E-MBS feedback.

# Fast Feedback Control Channel

The UL fast feedback channel shall carry channel quality feedback and MIMO feedback. There are two types of UL fast feedback control channels: primary fast feedback channel (PFBCH) and secondary fast feedback channels (SFBCH). The UL fast feedback channel starts at a pre-determined location, with the size defined in a DL broadcast control message. Fast feedback allocations to an AMS can be periodic and the allocations are configurable.

# Primary Fast Feedback Control Channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback and MIMO feedback.

[  ------------------- Recommended AWD Text Proposal #12 --------------------]

[DG Comment #12: Accept the recommended text proposal #12 on UL Control Information Content (SFBCH)]

# Secondary Fast Feedback Control Channel

The UL SFBCH carries narrowband CQI and MIMO feedback information. The number of information bits carried in the SFBCH ranges from 7 to 24. The number of bits carries in the fast feedback channel can be adaptive.

[  ------------------- Recommended AWD Text Proposal #13 --------------------]

[DG Comment #13: Accept the recommended text proposal #13 on UL Control Information Content (HARQ Feedback & BW REQ)]

# HARQ Feedback Control Channel

# Bandwidth request Channel