| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional material related to the "Recommended AWD Text Proposal #6" in "C802.16m-09/0558.doc" (S-SFH IE) |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Mihyun Lee, Hyunjeong Kang, Rakesh Taori, Heewon Kang, Hokyu Choi Samsung Electronics | mihyun.mac.lee@samsung.com |
| Re: | Call for reply comment on IEEE 802.16m-09/0558r1, “Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment.” Specific topic: “The recommended AWD text proposal #6 |  |
| Abstract | The contribution proposes the text of DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional material related to the "Recommended AWD Text Proposal #6" in "C802.16m-09/0558.doc" (S-SFH IE)

Mihyun Lee, Hyunjeong Kang and Rakesh Taori, Heewon Kang, Hokyu Choi

Samsung Electronics

- Introduction

As suggested in comment 139 in 80216m-09_0014, this contribution addresses additional material related to the "Recommended AWD Text Proposal #6" in "C80216m-09_0558r1.doc" or its latest revision.

P-SFH is kept as suggested in C80216m-09_0558r1.doc. Main changes with respect to the latest version of C80216m-09_0558.doc are as follows,

- Reorganization of subpacket according to two functions

  - Network Re-entry carried in S-SFH subpacket 2 IE.

  - Network Initial entry carried in S-SFH subpacket 3IE.

- S-SFH subpacket 5 in C80216m-09_0558r1.doc, which primarily carries paging information can be uncoupled from S-SFH. Paging information, if present in a superframe, can be located right after the S-SFH.

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity.

S-SFH SP1 IE presented in Table 1 indicate essential information needed for MS to select and access the network.

Table 1 S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSBs of superframe number | TBD | Part of superframe number |
| S-SFH SP scheduling information | TBD | SP scheduling bitmap |
| S-SFH SP change bitmap | TBD | 1 or multiple bits per SP |
| } |  |  |

Point to be considered during discussion: If fixed periodicity and offset are considered, it may be possible to remove above S-SFH SP1 IE.

S-SFH SP2 contains information for network re-entry, see Table 2

Table 2 S-SFH SP2 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 2 | Sector ID BS ID Periodicity of A-MAP Sub-frame configuration (DL/UL ratio) DL permutation configuration (CRU, DRU partitioning and signaling related to that) UL permutation configuration (CRU, DRU partitioning and signaling related to that) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna MAC protocol revision Downlink burst profile BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Uplink_Burst_Profile UL initial transmit timing Initial ranging channel information (initial ranging region location) Initial ranging channel format HO ranging codes HO ranging backoff start HO ranging backoff end |  |

S-SFH SP3 contains information for initial network entry and network discovery, see Table 3

Table 3 S-SFH SP3 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet3 | Sub-frame configuration (DL/UL ratio) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna BS Restart count MAC protocol revision Downlink burst profile BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Uplink_Burst_Profile Initial ranging channel information (initial ranging region location) Initial ranging channel format Initial ranging codes Initial ranging backoff start Initial ranging backoff end Minimum level of power offset adjustment Maximum level of power offset adjustment Duplex mode (TDD, FDD, HFDD) TTG RTG NSP IDs Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end HARQ Ack delay Fast Feedback Region HARQ Ack Region Default RSSI and CINR averaging parameter MS Transmit Power Limitation Level UL Noise and Interference Level Tx Power report Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end |  |

S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 4.

Table 4 S-SFH SP4 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 4 | Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end Sounding Region Fast Feedback Region HARQ Ack Region HARQ Ack delay |  |

S-SFH SP5 is transmitted in paging listening interval. It contains information for idle mode AMSs, see Table 5

Table 5: S-SFH SP5 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 5 | N_PGID Paging Indicator Usage Flag PGID List If( Paging Indicator Usage Flag ==1) ( Paging Indicator Bitmap } |  |

-------------------------------  Text End  ---------------------------------------------------