| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on the Burst Partition for the IEEE 802.16m AWD |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Sung-Eun Park, Chiwoo Lim, Seunghoon Choi, Songnam Hong, Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics Co., Ltd. | +82-31-279-5249 se.park@samsung.com |
| Re: | “802.16m amendment working document”: IEEE 802.16m-09/0012, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “Channel Coding and HARQ”. |  |
| Abstract | The contribution proposes the text of burst partition to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on the Burst Partition for the IEEE 802.16m AWD

Sung-Eun Park, Chiwoo Lim, Seunghoon Choi, Songnam Hong,

Jaeweon Cho, Jaehee Cho, Heewon Kang, Hokyu Choi

Samsung Electronics Co., Ltd.

- Introduction

The contribution proposes the text of the burst partition to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

- Burst partition

The burst is partitioned into a number of smaller blocks when the burst size including burst CRC exceeds the maximum FEC block size.

The burst partition that are included in the Channel coding and HARQ drafting group proposed text [5] is as follows.

NFB=argmaxX∈NFBX≤(NmodRFECNRENSMKFB)

The current burst partition is very simple rule, but has several drawbacks. Firstly, the current burst partition sends the burst that is smaller than the nominal size indicated by the signaling of MCS level and the resource allocation. The difference between the nominal burst size and the real burst size causes a system throughput loss. Secondly, the current burst partition rule allows the undesirable burst partition that causes the loss of channel coding gain and the loss of throughput by unnecessary adding of FEC block CRC.

In this contribution, we propose to replace the current burst partition rule by following equation.

NFB=argminX∈NFBX≥(NmodRFECNRENSMKFB)

The proposed burst partition rule can solve all the problems and still has the simplest form.

- Examples of the burst partition

The examples of burst partition are provided with various nominal burst sizes in Table 1.

In the examples, k means the nominal burst size depending on the signaling of MCS level and the resource allocation as follows.

k = Nmod •RFEC•NRE•NSM

The parameter k’ means the transmittable burst size depending on the burst partition rule.

In the proposed scheme, the difference between k’ and k is always positive number. That means that the proposed scheme sends the burst that is larger than the nominal burst size, so that there’s no throughput loss.

Furthermore, the proposed scheme can avoid the undesirable burst partition that happens in the current burst partition rule. (See the yellow background area in Table 1.)

Table 1 ─ Examples of burst partition

| k | DG |  |  |  |  | Proposed |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | KFB | k/ KFB | NEP | k' | k’-k | KFB | k/ KFB | NEP | k' | k’-k |
| 4856 | 2 | 2428 | 2368 | 4736 | -120 | 2 | 2428 | 2432 | 4864 | +8 |
| 5496 | 2 | 2748 | 2624 | 5248 | -240 | 2 | 2748 | 2752 | 5504 | +8 |
| 7256 | 2 | 3628 | 3520 | 7040 | -216 | 2 | 3628 | 3648 | 7296 | +40 |
| 9656 | 3 | 3218.7 | 3200 | 9600 | -56 | 3 | 3218.7 | 3264 | 9792 | +136 |
| 9784 | 3 | 3261.3 | 3200 | 9600 | -184 | 3 | 3261.3 | 3264 | 9792 | +8 |

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0510, “Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment”

- Text proposal for inclusion in the 802.16m amendment

Adopt the proposed text in the chapter 15.x.1.3 Burst partition of the DG text proposal (C80216m-09_0510.doc) with following modification:

Replace

NFB=argmaxX∈NFBX≤(NmodRFECNRENSMKFB)

by

NFB=argminX∈NFBX≥(NmodRFECNRENSMKFB)

Equivalent whole text proposal is as follows.

-------------------------------  Text Start  ---------------------------------------------------

Insert the following subsection at a new section 15:

  - Channel coding and HARQ

      - Burst partition

When the burst size including 16 burst CRC bits exceeds maximum FEC block size, NFB_MAX, the burst is partitioned into KFB FEC blocks. The value of KFB is calculated by following equation.

KFB=Nmod⋅RFEC⋅NRE⋅NSMNFB_MAX

where

Nmod 	is the modulation order of the burst transmission, which is specified by MCS index

RFEC 	is the nominal code rate of the burst transmission, which is specified by MCS index

NRE 		is the number of resource elements allocated for burst transmission. The calculation of NRE shall exclude the resource elements used by pilot channels

NSM		is the spatial multiplexing order of the resource elements allocated for burst transmission. The value of NSM depends on the MIMO transmission mode and is defined in section x.x.x.x.

NFB_MAX	is the maximum FEC block size, which equals to 4800 bits.

The nominal MCS used in a data transmission shall be selected from Table 15.x.1-1.

Table 15.x.1-1. MCS table for downlink and uplink data channel

| MCS index | Modulation | Code rate |
| --- | --- | --- |
| ‘0000’ | QPSK | 31/256 |
| ‘0001’ | QPSK | 48/256 |
| ‘0010’ | QPSK | 71/256 |
| ‘0011’ | QPSK | 101/256 |
| ‘0100’ | QPSK | 135/256 |
| ‘0101’ | QPSK | 171/256 |
| ‘0110’ | 16QAM | 102/256 |
| ‘0111’ | 16QAM | 128/256 |
| ‘1000’ | 16QAM | 155/256 |
| ‘1001’ | 16QAM | 184/256 |
| ‘1010’ | 64QAM | 135/256 |
| ‘1011’ | 64QAM | 157/256 |
| ‘1100’ | 64QAM | 181/256 |
| ‘1101’ | 64QAM | 205/256 |
| ‘1110’ | 64QAM | 225/256 |
| ‘1111’ | 64QAM | 237/256 |

The size of each FEC block is NFB, which is calculated by the following equation.

NFB=argminX∈NFBX≥(NmodRFECNRENSMKFB)

where {NFB} 	is a set of NFB shown in Table 15.x.1-2.

Table 15.x.1-2. FEC block size table for downlink and uplink data channels

| Index | NFB | Index | NFB | Index | NFB | Index | NFB | Index | NFB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 48 | 30 | 328 | 60 | 720 | 90 | 1424 | 120 | 2752 |
| 1 | 64 | 31 | 344 | 61 | 736 | 91 | 1448 | 121 | 2816 |
| 2 | 72 | 32 | 352 | 62 | 752 | 92 | 1480 | 122 | 2880 |
| 3 | 80 | 33 | 360 | 63 | 768 | 93 | 1504 | 123 | 2944 |
| 4 | 88 | 34 | 368 | 64 | 776 | 94 | 1536 | 124 | 3008 |
| 5 | 96 | 35 | 376 | 65 | 800 | 95 | 1560 | 125 | 3072 |
| 6 | 104 | 36 | 384 | 66 | 824 | 96 | 1600 | 126 | 3200 |
| 7 | 120 | 37 | 400 | 67 | 848 | 97 | 1640 | 127 | 3264 |
| 8 | 128 | 38 | 416 | 68 | 872 | 98 | 1672 | 128 | 3328 |
| 9 | 136 | 39 | 432 | 69 | 888 | 99 | 1712 | 129 | 3392 |
| 10 | 144 | 40 | 440 | 70 | 912 | 100 | 1752 | 130 | 3456 |
| 11 | 152 | 41 | 456 | 71 | 936 | 101 | 1784 | 131 | 3520 |
| 12 | 160 | 42 | 472 | 72 | 960 | 102 | 1824 | 132 | 3648 |
| 13 | 176 | 43 | 480 | 73 | 984 | 103 | 1864 | 133 | 3712 |
| 14 | 184 | 44 | 496 | 74 | 1000 | 104 | 1896 | 134 | 3776 |
| 15 | 192 | 45 | 512 | 75 | 1024 | 105 | 1920 | 135 | 3840 |
| 16 | 200 | 46 | 528 | 76 | 1048 | 106 | 1952 | 136 | 3904 |
| 17 | 208 | 47 | 544 | 77 | 1072 | 107 | 2000 | 137 | 3968 |
| 18 | 216 | 48 | 552 | 78 | 1096 | 108 | 2048 | 138 | 4096 |
| 19 | 232 | 49 | 568 | 79 | 1112 | 109 | 2096 | 139 | 4160 |
| 20 | 240 | 50 | 584 | 80 | 1136 | 110 | 2144 | 140 | 4224 |
| 21 | 248 | 51 | 600 | 81 | 1160 | 111 | 2192 | 141 | 4288 |
| 22 | 256 | 52 | 608 | 82 | 1184 | 112 | 2232 | 142 | 4352 |
| 23 | 264 | 53 | 624 | 83 | 1216 | 113 | 2280 | 143 | 4416 |
| 24 | 272 | 54 | 640 | 84 | 1248 | 114 | 2328 | 144 | 4544 |
| 25 | 288 | 55 | 656 | 85 | 1280 | 115 | 2368 | 145 | 4608 |
| 26 | 296 | 56 | 664 | 86 | 1312 | 116 | 2432 | 146 | 4672 |
| 27 | 304 | 57 | 680 | 87 | 1336 | 117 | 2496 | 147 | 4736 |
| 28 | 312 | 58 | 696 | 88 | 1368 | 118 | 2560 | 148 | 4800 |
| 29 | 320 | 59 | 712 | 89 | 1392 | 119 | 2624 |  |  |

The burst size NDB including burst CRC and FEC block CRC is defined by the following equation:

NDB = KFB × NFB

The payload size excluding burst CRC and FEC block CRC is defined by the following equation:

NPL = KFB × (NFB – IMFB×NFB-CRC) – NDB-CRC

where,

IMFB		equals 0 when KFB =1, 1 when KFB >1,
NFB-CRC	equals 16, which is the size of FEC block CRC,
NDB-CRC	equals 16, which is the size of burst CRC.

The burst partition block generates KFB FEC blocks, with each FEC block processed by the FEC block CRC encoding block as described in 15.x.1.4.

-------------------------------  Text End  ---------------------------------------------------