| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Resource Allocation in the SFH Subframe for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Jinsoo Choi, HanGyu Cho, Sungho Moon, Jin Sam Kwak LG Electronics Zexian Li Nokia Xiaoyi Wang, Xin Qi Nokia Siemens Networks | {emptylie, hgcho, msungho, samji}@lge.com zexian.li@nokia.com {xiaoyi.wang, xin.qi }@nsn.com |
| Re: | Call for reply comment on IEEE 802.16m-09/0558r1, “Proposed text from DL PHY Ctrl Drafting group for IEEE 802.16m Amendment.” Specific topic: “The recommended AWD text proposal #1” |  |
| Abstract | The contribution provides the proposed text of SFH resource allocation to be included in IEEE 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Resource Allocation in the SFH Subframe
for IEEE 802.16m Amendment

Jinsoo Choi, HanGyu Cho, Sungho Moon, Jin Sam Kwak

LG Electronics

Zexian Li

Nokia

Xiaoyi Wang, Xin Qi

Nokia Siemens Networks

- Introduction

This contribution provides the proposed text on resource allocation in the subframe that contains Superframe Header (SFH) to be included in the DL control section (related to recommended AWD text proposal #1 in C802.16m-09_0558 or its latest version) of the 802.16m amendment working document (AWD).

- Rationale of the proposal

In the first subframe of every superframe containing SFH (called SFH subframe throughout this contribution), resource allocation is different from other subframes. It is because the total system bandwidth consists of the bandwidth for SFH which is the physical 5MHz minimum bandwidth and the remaining system bandwidth when the system bandwidth is more than 5MHz.

This proposal provides resource allocation rule in the SFH subframe considering both the case of 5MHz system bandwidth and the case of more than 5MHz system bandwidth.

The design criteria is as follows:

- For the SFH frequency band (5MHz)

- The only distributed LRUs are assigned for providing the sufficient diversity gain and avoiding the unnecessary overhead.

- The subband partitioning process is omitted.

- The subcarrier permutation is done as defined in 15.3.5.

- The LRUs are assigned in the order of P-SFH, S-SFH and data/control (if any).

- For the remaining frequency band in the case of more than 5MHz system bandwidth

- Basically, subchannelization is compliant to the rules defined in 15.3.5.

- It is desirable to be able to support both CRU and DRU.

- It is reasonable to assume that subbands are assigned to CRU and minibands are assigned to DRU for minimizing signaling overhead.

- Based on the above, the only signaling necessary for resource allocation on this frequency band is the number of subbands assigned (called Subband Allocation Count (SAC) in the normal subframes). We denote the number of subbands assigned for the remaining frequency band in SFH subframe by SACSFH. The SACSFH is [4]-bit and signaled in the S-SFH. [Note: Considering at most 18 subbands in the remaining 15MHz bandwidth for 20MHz system bandwidth, 4 bits or less seems enough.]

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- 4. Abbreviations and acronyms

[Insert the following abbreviations and acronymns (Section 4):]

A-Preamble	advanced preamble

PA-Preamble	primary advanced preamble

SA-Preamble	secondary advanced preamble

P-SFH		primary superframe header

S-SFH 		secondary superframe header

A-MAP		advanced MAP

GRA		group resource allocation

PA		persistent allocation

IE		information element

# DL Control Structure

# Advanced Preamble

# DL Control Channels

DL control channels convey information essential for system operation. Information on DL control channels is transmitted hierarchically over different time scales from the superframe level to the subframe level.

In mixed mode operation (WirelessMAN-OFDMA/Advanced Air interface), an AMS can access the system without decoding WirelessMAN-OFDMA FCH and MAP messages.

# Superframe Header

The Superframe Header (SFH) carries essential system parameters and system configuration information. The SFH is located in the first subframe within a superframe. SFH occupies no more BW than 5 MHz. If SFH occupies narrower BW than system BW, SFH is FDM with data and control within the same subframe.

The SFH are TDM with A-Preamble.

The PHY structure for resource allocation of the SFH is described in 15.3.5. The SFH is transmitted within a predefined frequency partition called SFH frequency partition. The PRUs of the SFH frequency partition are selected within 5MHz physical bandwidth. The PRU in the whole SFH frequency partition uses the 2 stream pilot pattern defined in 15. 3. 5. The whole frequency partition is mapped to be distributed LRUs.

The SFH is divided into two parts: Primary Superframe Header (P-SFH) and Secondary Superframe Header (S-SFH).

Table 1 inludes the parameters and values for resource allocation of the SFH.

Table 1: Parameters and values for resource allocation of SFH

| Parameters | Description | Value |
| --- | --- | --- |
| NDLRU, SFH | The number of distributed LRUs which are occupied by SFH in the SFH frequency partition Note that NDLRU,SFH = NDLRU,P-SFH + NDLRU,S-SFH | TBD (<= 24 (i.e. 5 MHz)) |
| N DLRU, P-SFH | The number of distributed LRUs which are occupied by P-SFH | Fixed (value is TBD) |
| N DLRU, S-SFH | The number of distributed LRUs which are occupied by S-SFH | Variable (maximum value is TBD) |

If N DLRU, SFH is less than 24, the other DLRUs distributed LRUs of the SFH frequency partition are allocated for data or other control transmission.

The subcarrier to resource unit mapping of the subframe containing the SFH is explained as follows:

1. Permutation for P/S-SFH bandwidth (regardless of the system BW)

- The PRUs within the 5MHz physical bandwidth are selected for resource allocation of the SFH (or data/control)

- The subband partitioning process is omitted in this process.

- All the PRUs are assigned to DRUs. The subcarrier permutation defined in 15.3.5 is applied to the DRUs to form distributed LRUs.

- Distributed LRUs are assigned in the order of P-SFH and S-SFH (and data/control if there is remaining LRUs) as represented in Figure xxx for the case of 10MHz system bandwidth.

2. Permutation for the remaining system BW (in cases of more than 5MHz system bandwidth) based on the rule defined in 15.3.5.

- There is no frequency partitioning for the remaining frequency band.

- Subband partitioning is applied by using the [4]-bit SACSFH field signaled in the S-SFH.

- After subband partitioning, all the subbands are assigned to CRUs.

- Miniband permutation is applied for all the minibands.

- After miniband permutation, all the minibands are assigned to DRUs and subcarrier permutation is performed on the DRUs as defined in 15.3.5 .

- The distributed and localized LRUs are assigned to the data/control channels.

In case of 10MHz system bandwidth, the subcarrier to resource unit mapping of the subframe containing the SFH is illustrated in Fig. xxx.

Figure xxx Subcarrier to resource unit mapping in the SFH subframe

-------------------------------  Text End  ---------------------------------------------------