| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for Primary Fast Feedback Channel |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Hwasun Yoo, Sangheon Kim, Jaehee Cho, Heewon Kang, Hokyu Choi Samsung Electronics | hwasun.yoo@samsung.com |
| Re: | C80216m-09_0386, “Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control” |  |
| Abstract | This contribution proposes the design of primary feedback channel to the UL Ctrl DG text. |  |
| Purpose | For member’s review and adoption into P802.16m Amendment Working Document |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Primary Fast Feedback Channel

Hwasun Yoo, Sangheon Kim, Jaehee Cho, Heewon Kang, Hokyu Choi

Samsung Electronics

Introduction

This contribution provides amendment text on the detail design of uplink primary fast feedback channel. During UL control drafting group discussion, several sequences are proposed for modulating primary fast feedback channel. The performance comparison results are provided in C80216m-09_0xxx.

Suggested Modification on UL Control DG harmonized text (C80216m-09_0386 or its latest version)

[Deleted the text in red and replace with the proposed text in blue]

[-------------------AWD Text Proposal #5 of UL Ctrl DG  --------------------]

# Primary Fast Feedback Control Channel

The primary fast feedback channels are comprised of 3 distributed FMTs. Figure UL-3 illustrates the mapping of the PFBCH.

Figure UL- , PFBCH comprised of three distributed 2x6 UL FMTs.

Figure UL- , Mapping of information in the PFBCH.

The process of composing the PFBCH is illustrated in Figure UL- . The l PFBCH payload bits are used to generate PFBCH sequence according to Error: Reference source not found. The resulting bit sequence is modulated [, repeated] and mapped to uplink PFBCH symbol s[k]. The mapping of primary fast feedback channel symbol s[k] to the UL FMTs is given by Error: Reference source not found. This set of sequence can carrier up to 6 information bits.

Ci,j=s[k],fori=0,1,2,0≤j≤,11,and0≤k≤,11, Eqn. UL-

Ci,j=s[ Ki[j] ],fori=0,1,2,0≤j≤11, 		Eqn. UL-6

where Ki[j] denotes the jth element of Kiand is defined as

K0=0,1,2,3,4,5,6,7,8,9,10,11

K1=9,10,11,3,4,5,0,1,2,6,7,8

K2=3,4,5,6,7,8,9,10,11,0,1,2

Table UL- , Sequences for PFBCH.

| Index | Sequence | Index | Sequence |
| --- | --- | --- | --- |
| 0 |  | 32 |  |
| 1 |  | 33 |  |
| 2 |  | 34 |  |
| … | … | … | … |
| 30 |  | 62 |  |
| 31 |  | 63 |  |

Table UL- , Sequences for PFBCH

| Index | Sequence | Index | Sequence |
| --- | --- | --- | --- |
| 0 | 111111111111 | 32 | 101011001001 |
| 1 | 101111010110 | 33 | 111011100000 |
| 2 | 011010111101 | 34 | 001110001011 |
| 3 | 001010010100 | 35 | 011110100010 |
| 4 | 101010101010 | 36 | 100111111010 |
| 5 | 111010000011 | 37 | 110111010011 |
| 6 | 001111101000 | 38 | 000010111000 |
| 7 | 011111000001 | 39 | 010010010001 |
| 8 | 110011001100 | 40 | 111110011100 |
| 9 | 100011100101 | 41 | 101110110101 |
| 10 | 010110001110 | 42 | 011011011110 |
| 11 | 000110100111 | 43 | 001011110111 |
| 12 | 100110011001 | 44 | 101010011111 |
| 13 | 110110110000 | 45 | 111010110110 |
| 14 | 000011011011 | 46 | 001111011101 |
| 15 | 010011110010 | 47 | 011111110100 |
| 16 | 101011111100 | 48 | 111111001010 |
| 17 | 111011010101 | 49 | 101111100011 |
| 18 | 001110111110 | 50 | 011010001000 |
| 19 | 011110010111 | 51 | 001010100001 |
| 20 | 111110101001 | 52 | 110010101111 |
| 21 | 101110000000 | 53 | 100010000110 |
| 22 | 011011101011 | 54 | 010111101101 |
| 23 | 001011000010 | 55 | 000111000100 |
| 24 | 100111001111 | 56 | 100110101100 |
| 25 | 110111100110 | 57 | 110110000101 |
| 26 | 000010001101 | 58 | 000011101110 |
| 27 | 010010100100 | 59 | 010011000111 |
| 28 | 110010011010 | 60 | 110011111001 |
| 29 | 100010110011 | 61 | 100011010000 |
| 30 | 010111011000 | 62 | 010110111011 |
| 31 | 000111110001 | 63 | 000110010010 |