| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text for Secondary Fast Feedback Channel |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Sangheon Kim, Hwasun Yoo, Jaeehee Cho, Heewon Kang, and Hokyu Choi Samsung Electronics | E-mail: sangheon.kim@samsung.com |
| Re: | Call for reply comments on Project 802.16m Amendment Working Document (AWD). |  |
| Abstract | The contribution proposes the text of secondary fast feedback channel in the UL Control section of the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text for Secondary Fast Feedback

Sangheon Kim, Hwasun Yoo, Jaehee Cho, Heewon Kang, and Hokyu Choi

Samsung Electronics

- Introduction

This contribution proposes amendment text of UL control for secondary fast feedback channel to be included in the IEEE 802.16m Amendment Working Document , subclause 15.3.9.3. This contribution is additional material related to the “Recommended AWD Text Proposal # 6” in [2].

- References

[1] IEEE 802.16m-09/0010, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-01-29.

[2] IEEE 802.16m-09/386, “Proposed Text for the Draft P802.16m Amendment on the PHY Structure for UL Control”, 2009-03-02.

- Proposed AWD Text

 Proposed Text for DG proposal #6: UL Control Content (SFBCH) 

Add the text and table marked in blue.

--------------------------------------------- Start of Text for DG Proposal #11 ------------------------------------------

# Secondary Fast Feedback Control Channel

The SFBCH is comprised of 3 distributed FMTs with [2] [4] pilots allocated in each FMT. Pilot sequence is TBD.

The SFBCH symbol generation procedure is shown in Figure UL- . First, the SFBCH payload information bits a0a1a2⋯al−1 are encoded to N bits b0b1b2⋯bN−1using the linear block codes described in Section 15.3.9.2.1.1.1.

For l≤12, information bits a0a1a2⋯al−1 are encoded using the linear block code (N,l).

For 12<l≤24, information bits a0a1a2⋯al−1are split into 2 parts: Part A consisting of a0a1a2⋯al2−1, and Part B consisting of al2al2+1al2+2⋯al−1. Part A is encoded to N2bits b0b1b2⋯bN2−1using linear block code (N2,l2) and Part B is encoded to N2bits bN2bN2+1bN2+2⋯bN−1 using a linear block code(N2,l−l2). The coded sequence b0b1b2⋯bNis then modulated to N2symbols c0c1c2⋯cN2−1using QPSK. The value of N is TBD 60. The modulated symbols c0c1c2⋯cN2−1 and pilot sequence p0p1p2⋯p5(TBD) are combined to form sequence d0d1d2⋯d35 and then mapped to the data subcarriers of the SFBCH FMTs as shown in Figure UL- . according to Eqn. UL- xx.

di=pi7for  mod(i,7)=0ci−i7−1'for  mod(i,7)≠0  , 0≤i≤35 			Eqn. UL- xx

where symbols c0'c1'c2'⋯cN2−1' denote the reordered symbols of c0c1c2⋯cN2−1, in order to be allocated to data subcarriers by frequency-first order. Then, sequence d0d1d2⋯d35 are mapped to the data subcarriers of the SFBCH FMTs as shown in Figure UL-6.

Figure UL- , Mapping of information in the SFBCH.

Figure UL- , SFBCH comprising of three distributed 2x6 UL FMTs.

# Channel coding for Secondary Fast Feedback Control Channel

The k (7≤K≤12) information bits in the SFBCH shall be encoded using linear block codes with codeword length. Let the K information bits be denoted by a0,a1,…,aK−1and the N bits codeword is denoted byb0,b1,…,bN−1. The codeword is obtained as a linear combination of the N basis sequences denoted asvi,n where n=0,⋯,K−1, i=0,⋯,N−1 in Error: Reference source not found.

For the length of 7≤l≤12, information bits in the SFBCH shall be encoded using linear block codes with codeword length . Let the l information bits be denoted by a0,a1,…,al−1and the N bits codeword is denoted byb0,b1,…,bN−1. The codeword is obtained as a linear combination of the l basis sequences of length  denoted asvi,n where n=0,⋯,l−1, i=0,⋯,N−1 in Table UL-3, and is given by Eqn. UL-xxx.

bi=n=0l−1an×vi,nmod2 			Eqn. UL- xxx

For the length of 13<l≤24, the first part of information bits a0a1a2⋯al2−1 and the second part of information bits al2al2+1al2+2⋯al−1 shall be encoded separately using linear block codes with codeword length N2 into b0b1b2⋯bN2−1  and bN2bN2+1bN2+2⋯bN−1 as given by Eqn. UL-xxxx.

bi=n=0l2−1an×vi,nmod2for  i=0, ..., N2−1n=l2l−1an×vi−N2,n−l2mod2  for  i=N2, ..., N−1	 Eqn. UL- xxxx

Table UL-3, Basis sequences.

| i | vi,0 | vi,1 | vi,2 | vi,3 | vi,4 | vi,5 | vi,6 | vi,7 | vi,8 | vi,9 | vi,10 | vi,11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 2 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 3 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 4 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 |
| 5 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 6 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 7 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |
| 8 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 9 | 1 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |
| 10 | 1 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| 11 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 12 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 13 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| 14 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 15 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |
| 16 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 17 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 18 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 19 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 20 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 |
| 21 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 22 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 23 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 24 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 |
| 25 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 26 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 27 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| 28 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 29 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| 30 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| 31 | 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 |
| 32 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 33 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |
| 34 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 35 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 36 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 |
| 37 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 38 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 39 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 |
| 40 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 |
| 41 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |
| 42 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 43 | 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 |
| 44 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| 45 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 46 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 |
| 47 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 |
| 48 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 49 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| 50 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 51 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 |
| 52 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 53 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| 54 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 55 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 56 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 57 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 58 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 |
| 59 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |

--------------------------------------------- End of Text for DG Proposal #6 -------------------------------------------