| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Constellation Re-arrangement for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Voice: +65-65505304 E-mail: lei.huang@sg.panasonic.com |
| Re: | 802.16m amendment working document Target topic: “Channel Coding and HARQ” |  |
| Abstract | 802.16m amendment text proposal on Constellation Re-arrangement |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Constellation Re-arrangement For IEEE 802.16m Amendment

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

According to IEEE 802.16m SDD [1], Constellation re-arrangement (CoRe) is supported in 802.16m, wherein the CoRe can be expressed by a bit-level interleaver with a tone. However, the CoRe details are not described in [2]. This contribution proposes details for the CoRe scheme.

- Constellation Re-arrangement

Constellation re-arrangement only applies to 16 QAM and 64 QAM modulated bits. In case of QPSK it is transparent.

Table 1 and Table 2 describe the operations that produce the different CoRe-versions for 16 QAM and 64 QAM, respectively, so that the four bits in a 16 QAM symbol (the six bits in a 64 QAM) are of the same resilience. In other words, the two bits of higher quality at CoRe-version 1 are of lower quality at CoRe-version 2 while the two bits of lower quality at CoRe-version 1 are of higher quality at CoRe-version 2. Note that for the CoRe-version 1, each M bits (M = 4, 6) shall be mapped to the constellation bits bM-1– b0 in MSB-first order (i.e., the first bit shall be mapped to the higher index bit in the constellation).

Table 1: Constellation re-arrangement for 16 QAM

| BitRe-versions | Output bit sequence | Operation |
| --- | --- | --- |
| 1 |  | - None |
| 2 |  | - 1-bit circular shift for the in-phase and orthogonal components individually |

Table 2: Constellation re-arrangement for 64 QAM

| BitRe-versions | Output bit sequence | Operation |
| --- | --- | --- |
| 1 |  | - None |
| 2 |  | - 1-bit circular shift for the in-phase and orthogonal components individually |

- Simulation Results

As for the detailed simulation results, please refer to [3]. Note that 4 CoRe-versions are used for 16 QAM and 6 CoRe-versions are used for 64 QAM in [3], and but only 2 CoRe-versions are suggested for 16 QAM and 64 QAM in this contribution. This is because the tradeoff between CoRe gain and signaling overhead in IR HARQ is taken into account.

Reference

[1] IEEE 802.16m-08/003r7, IEEE 802.16m System Description Document [draft]

[2] IEEE C802.16m-09/0510, Proposed Text of Channel Coding and HARQ for the IEEE 802.16m Amendment

[3] IEEE 802.16m-08/771r1, Enhanced HARQ scheme with Signal Constellation Rearrangement

# Suggested Amendment Text

----------------------------------------------------------------------------------

# 15.X.2.

# HARQ

# 15.X.2.1 IR HARQ

# 15.X.2.2 Constellation Re-arrangement

Constellation re-arrangement (Co-Re) is supported in 802.16m. Constellation re-arrangement only applies to 16 QAM and 64 QAM modulated bits. In case of QPSK it is transparent.

Table 1 and Table 2 describe the operations that produce the different CoRe-versions for 16 QAM and 64 QAM, respectively, so that the four bits in a 16 QAM symbol (the six bits in a 64 QAM) are of the same resilience. In other words, the two bits of higher quality at CoRe-version 1 are of lower quality at CoRe-version 2 while the two bits of lower quality at CoRe-version 1 are of higher quality at CoRe-version 2. Note that for the CoRe-version 1, each M bits (M = 4, 6) shall be mapped to the constellation bits bM-1– b0 in MSB-first order (i.e., the first bit shall be mapped to the higher index bit in the constellation).

Table xxx: Constellation re-arrangement for 16 QAM

| BitRe-versions | Output bit sequence | Operation |
| --- | --- | --- |
| 1 |  | - None |
| 2 |  | - 1-bit circular shift for the in-phase and orthogonal components individually |

Table yyy: Constellation re-arrangement for 64 QAM

| BitRe-versions | Output bit sequence | Operation |
| --- | --- | --- |
| 1 |  | - None |
| 2 |  | - 1-bit circular shift for the in-phase and orthogonal components individually |

----------------------------------------------------------------------------------