| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Downlink HARQ Parameter Signaling for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-03-09 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Voice: +65-65505304 E-mail: lei.huang@sg.panasonic.com |
| Re: | 802.16m amendment working document Target topic: Channel Coding and HARQ |  |
| Abstract | 802.16m amendment text proposal on HARQ parameter signaling |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Downlink HARQ Parameter Signaling for IEEE 802.16m Amendment

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

For IEEE 802.16m, hybrid ARQ (HARQ) has been adopted to be mandatory for both downlink (DL) and uplink (UL) unicast data traffic at both base station (BS) and mobile station (MS). This contribution addresses the parameter signaling for DL HARQ.

- DL HARQ Parameter Signaling

According to the proposed amendment text prepared by DL PHY Ctrl Drafting Group [1], an assignment A-MAP IE includes the following information:

  - MCS

  - MM

  - Resource allocation

  - AI_SN

  - ACID

  - SPID

  - CRV, etc.

MCS determines the Modulation and Coding Scheme;

MM determines the MIMO Mode;

Resource allocation determines the start, length and number of subframes spanned the allocation;

AI_SN stands for HARQ Identifier Sequence Number.

ACID stands for HARQ Channel Identifier, which is used to identify HARQ channels.

SPID stands for Subpacket Identifier, which is used to identify the starting position of the bit selection in circular buffer for each subpacket.

CRV stands for Constellation Re-arrangement Version.

- Related Discussions in Channel Coding & HARQ Drafting Group

In Channel Coding & HARQ Drafting Group, it has been suggested that some kind of joint coding for SPID, MCS, CRV, or AI_SN can be used to reduce A-MAP signaling overhead. For example, a MCS signaling scheme has been proposed in [2] to reduce A-MAP overhead, where MCS index in MCS table have different information for initial and retransmission.

- Initial transmission: MCS index indicates modulation and code rate.

- Retransmission: MCS index indicates modulation, SPID and CRV.

Note that this scheme relies on successful reception of A-MAP of the first transmission. However, relying on high reliability of A-MAP is problematic, especially with separately encoded A-MAP IEs where each IE has link adaptation to its destination.

Someone may argue that the initial MAP loss problem can be solved by DTX detection. However, DTX detection is an implementation issue. “Implementation” means that it is not mandatory to implement a DTX detection capability at the BS. Therefore, the system should work efficiently also w/o DTX detection capability. In addition, DTX detection is not always reliable.

- Our Proposal For DL HARQ Parameter Signaling

In our proposal, the MCS is signaled separately from SPID and CRV. Furthermore, our proposal does not rely on successful reception of A-MAP of the first transmission. In other words, our proposal has no initial MAP loss problem.

In our proposal, a 2-bit SPID and a 1-bit CRV are used to support 4 starting positions and 2 CoRe versions. There are two alternatives for new data indication, i.e., explicit indication with AI_SN field and indication with SPID = 0. In the former, the AI_SN is toggled between “0” and “1” on successfully transmitting each encoder packet with the same HARQ channel. In the latter, SPID = 0 is used to indicate transmission of new data, and SPID = 0 cannot be used for retransmissions. However, in case of jointly signaling of SPID and CRV, only explicit indication with AI_SN field is feasible.

We propose two options for DL HARQ parameter signaling as shown in Table 1.

Table 1: DL HARQ parameter signaling

| Options | ACID | AI_SN | SPID | CRV | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | 4 bits | NA | 2 bits | 1 bit | a) SPID = 0 implies indication of new data, and SPID = 0 cannot be used for retransmissions b) 2 CoRe-versions and 4 starting positions c) Total 7 bit signaling overhead |
| 2 | 4 bits | 1 bit | 2 bits | 1 bit | a) 2 BitRe-versions and 4 starting positions b) Total 8 bit signaling overhead c) SPID = 0 can be used for (re)transmissions |

Reference

[1] IEEE 802.16m-09.0558, Proposed text from DL PHY Ctrl Drafting Group for IEEE 802.16m Amendment

[2] IEEE 802.16m-09/0554, MCS Signaling for Reducing MAP Overhead

# Suggested Amendment Text

----------------------------------------------------------------------------------

# Advanced Air Interface

# Medium Access Control

# MAC Support for HARQ

# 15.2.X.Y DL HARQ parameter signaling

For DL HARQ, the parameters for each subpacket should be signaled independent of the subpacket itself. Three options for DL HARQ parameter signaling are shown in Table yyy.

Table 1: DL HARQ parameter signaling

| Options | ACID | AI_SN | SPID | CRV | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | 4 bits | NA | 2 bits | 1 bit | a) SPID = 0 implies indication of new data, and SPID = 0 cannot be used for retransmissions b) 2 CoRe-versions and 4 starting positions c) Total 7 bit signaling overhead |
| 2 | 4 bits | 1 bit | 2 bits | 1 bit | a) 2 BitRe-versions and 4 starting positions b) Total 8 bit signaling overhead c) SPID = 0 can be used for (re)transmissions |

ACID

The HARQ channel identifier, which is used to identify HARQ channels. Each connection can have multiple HARQ channels, each of which may have an encoder packet transaction pending.

SPID

The HARQ subpacket identifier, which is used to identify the starting position of the bit selection in circular buffer for each subpacket.

CRV

The constellation re-arrangement version, which is used to identify the constellation re-arrangement version for each subpacket.

AI_SN

The HARQ identifier sequence number. This is toggled between 0 and 1 on successfully transmitting each encoder packet with the same HARQ channel.

-----------------------------------------------------------------------------------------------------------------