| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for S-SFH IE Subpacketization |  |
| Date Submitted | 2009-03-10 |  |
| Source(s) | Yerang Hur Kyounghwan Lee POSDATA | Voice: +1-408-986-1140 E-mail:{yehur, khlee}@posdata-usa.com |
| Re: | Call for reply comment on IEEE 802.16m-09/0558r1 |  |
| Abstract | This contribution proposes modification of S-SFH SP1 IE contents and rationale to classify S-SFH SP IEs . |  |
| Purpose | To discuss and adopt the proposed text in the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal for S-SFH IE Subpacketization

Yerang Hur and Kyounghwan Lee

Posdata

# Introduction

This contribution proposes modification of S-SFH SP1 IE contents and rationale to categorize S-SFH SP IEs provided in C802.16m-09/558r1.

# Outline of Proposal

This contribution proposes that we agree on the followings first before nailing down the details of S-SFH IE.

- S-SFH SP1 IE format including Additional Broadcast Information Indicator in S-SFH SP1 IE.

- The number of S-SFH SP IEs.

# Proposed Text

-----------------------------------------  Start of the Text---------------------------------------------------

# S-SFH IE

The S-SFH IE is mapped to the S-SFH. Essential system parameters and system configuration information belonging to the S-SFH are categorized into multiple S-SFH IEs. The S-SFH IEs are transmitted in different timing and periodicity.

S-SFH SP1 IE presented in Table 1 indicates essential information needed for MS to select and access the network.

Table 1 S-SFH SP1 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| MSBs of superframe number | TBD | Part of superframe number |
| S-SFH SP scheduling information | TBD | SP scheduling bitmap |
| S-SFH SP change bitmap | TBD | 1 or multiple bits per SP |
| Additional broadcast message indicator | 1 | Indicates that additional broadcast information is carried in the Broadcast Management message in this Superframe. |
| } |  |  |

S-SFH SP2 contains information for HO or network re-entry from Idle Mode see Table 2

Table 2 S-SFH SP2 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 2 | Sector ID Periodicity of A-MAP Sub-frame configuration (DL/UL ratio) DL permutation configuration (CRU, DRU partitioning and signaling related to that) UL permutation configuration (CRU, DRU partitioning and signaling related to that) FFR partitioning info for DL region (static) FFR partitioning info for UL region (static) FFR UL target IoT per partition # tx antenna MAC protocol revision Downlink burst profile BS EIRP UL carrier frequency UL bandwidth UL A-MAP relevance Uplink_Burst_Profile |  |

S-SFH SP3 contains information for initial network entry and network discovery, see Table 3

Table 3 S-SFH SP3 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 3 | Initial ranging channel information (initial ranging region location) Initial ranging channel format Initial ranging codes Initial ranging backoff start Initial ranging backoff end Minimum level of power offset adjustment Maximum level of power offset adjustment Duplex mode (TDD, FDD, HFDD) TTG RTG NSP IDs MSB bytes of BSID |  |

S-SFH SP4 contains information for maintaining communication with the ABS (e.g., periodic ranging, bandwidth request, sounding, HARQ parameters, and fast feedback parameters), see Table 4.

Table 4 S-SFH SP4 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 4 | Periodic ranging channel information (periodic ranging region location) Periodic ranging codes Periodic ranging backoff start Periodic ranging backoff end Bandwidth request channel information (bandwidth request region location) Bandwidth request backoff start Bandwidth request backoff end Sounding Region Fast Feedback Region HARQ Ack Region HARQ Ack delay |  |

S-SFH SP54 is transmitted in paging listening interval. It contains paging information for idle mode AMSs, see Table  Table7 6. If the paging indicator flag associated with its own PGID is set then AMS will subsequently decode the full paging message at the pre-determined location; otherwise AMS will return to Paging Unavailable Interval.

Table7 6: S-SFH SP54 IE

| Channel | Contents | Size (bits) |
| --- | --- | --- |
| S-SFH Sub-packet 5 | N_PGID Paging Indicator Usage Flag PGID List If( Paging Indicator Usage Flag ==1) ( Paging Indicator Bitmap } |  |

-----------------------------------------  End of the Text---------------------------------------------------

- References

[1] IEEE 802.16m DL Ctrl DG, IEEE C802.16m-09/0588r1, March 2009.