| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized AWD Text on the MAC Procedure for Group Resource Allocation |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | Roshni Srinivasan, Shweta Shrivastava, Rath Vannithamby, Intel Corporation Jason Junsung Lim Samsung Electronics Co. Ltd. Ming-Hung Tao ITRI Yanfeng Guan ZTE Corporation Xiaoyi Wang, Shashikant Maheshwari NSN Kiseon Ryu LG Electronics Yi-Shen Chen, MediaTek | E-mail: roshni.m.srinivasan @intel.com junsung.lim@samsung.com mhtao@itri.org.tw guan.yanfeng@zte.com.cn xiaoyi.wang@nsn.com; shashi.maheshwari@nsn.com ksryu@lge.com yihshen.chen@mediatek.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | 802.16m amendment working document |  |
| Abstract | Group Resource Allocation Mechanism in IEEE 802.16m |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized AWD Text on the MAC Procedure for Group Resource Allocation

Roshni Srinivasan, Shweta Shrivastava, Rath Vannithamby, Intel Corporation;

Jason Junsung Lim, Samsung Electronics Co. Ltd.; Ming-Hung Tao, ITRI;

Yanfeng Guan, ZTE Corporation; Xiaoyi Wang, Shashikant Maheshwari, NSN; Zexian Li, Nokia;

Kiseon Ryu, LG Electronics; Yih-Shen Chen, MediaTek

Introduction

This contribution provides a template for developing text to describe the MAC procedure for Group Resource Allocation for the IEEE 802.16m Amendment Working Document.

Insert the following text in Section 15.2.x (subsection under MAC) of the IEEE 802.16m Amendment Working Document.

Begin proposed text

15.2.x	Group Resource Allocation

Group Resource Allocation mechanism allocates resources to multiple users as a group in order to save control overhead. The mechanism takes advantage of common traffic characteristics and grouping is done based on some common parameters such as MCS (modulation and coding scheme) and resource size, which further saves overhead.

## 15.2.x.1	Grouping Mechanism

Users are assigned to groups based on the combination of MCS used and the resource allocation size (number of LRUs) required. A set of n-bit codes can be used to represent the different combinations of MCSs and resource sizes that are used by a group. These codes are included in a bitmap as part of the group’s resource allocation information.

## 15.2.x.2	Group Configuration

{ Editor’s Notes: This section is a placeholder for description of MCS/Resource configurations that are used to define a group}

## 15.2.x.3	Group Management

## 15.2.x.3.1	Addition of AMS to a Group

## 15.2.x.3.2	Deletion of AMS from a Group

## 15.2.x.3.3	Bitmaps in Group Resource Allocation

GRA makes use of bitmaps to signal resource allocation information for users within a group. These bitmaps are sent in the Group Resource Allocation IE. The first bitmap is the User Bitmap which uses 1 bit per user to signal which users are scheduled in the frame.

The second bitmap is the Resource Allocation bitmap which uses n bits per user to signal the MCS and Resource Size for users that are scheduled in the frame. An example of bitmaps is shown in Figure xxx.

Figure xxx: User Bitmap and Resource Allocation Bitmap

## 15.2.x.4	HARQ Operation for Group Resource Allocation

## 15.2.x.5	Error Handling Procedure

End proposed text