| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Revisions to MIMO DG Working Document (C80216m-09_0513r1_working-document-V01) |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | David Mazzarese, Bruno Clerckx , Jiann-An Tsai, Jerry Pi, Heewon Kang, Hokyu Choi Samsung Electronics Wookbong Lee and Bin-Chul Ihm LG Electronics Alexei Davydov, Guangjie Li, Yang-seok, Gregory Morozov Intel Corporation | d.mazzarese@samsung.com wbong@lge.com alexei.davydov@intel.com |
| Re: | TGm MIMO DG on MIMO Draft Amendment text |  |
| Abstract | The contribution proposes the revisions to the text of MIMO section to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Revisions to MIMO DG Working Document (C80216m-09_0513r1_working-document-V01)

David Mazzarese, Bruno Clerckx , Jiann-An Tsai, Jerry Pi, Heewon Kang, Hokyu Choi

Samsung Electronics

Wookbong Lee and Bin-Chul Ihm

LG Electronics

Alexei Davydov, Guangjie Li, Yang-seok, Gregory Morozov

Intel Corporation

- Introduction

This contribution proposes revisions to the MIMO DG Working Document [1].

The proposed revisions are highlighted in blue.

- References

[1] IEEE C80216m-09_0513r1_working-document-V01, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-03-10.

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #1 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #1 on definitions and abbreviations]

- Definitions

Insert the following at the end of section 3:

3.xx layer: An information path fed to the MIMO encoder as an input

3.xx stream: Each information path encoded by the MIMO encoder that is passed to the precoder

3.xx rank: For the spatial multiplexing modes in SU-MIMO, the number of streams to be used for the user allocated to the Resource Unit (RU)

3.xx Rate: The number of QAM symbols signaled per array channel use.

3.xx Horizontal encoding: Indicates transmitting multiple separately FEC-encoded layers over multiple antennas. The number of encoded layers may be more than 1

3.xx Vertical encoding: Indicates transmitting a single FEC-encoded layer over multiple antennas. The number of encoded layers is always 1.

3.xx Resource Unit: A granular unit in frequency and time, described by the number of OFDMA subcarriers and OFDMA symbols

3.xx Single User MIMO: A MIMO transmission scheme in which a single MS is scheduled in one RU

3.xx Multi-User MIMO: A MIMO transmission scheme in which multiple MSs are scheduled in one RU, by virtue of spatial separation of the transmitted signals

# Abbreviations and acronyms

Insert the following at the end of section 4:

CL			Closed-loop

CMI			Codebook Matrix Index

CSM			Collaborative Spatial Multiplexing

DL			Downlink

HE			Horizontal Encoding

MU			Multi-User

OL			Open-loop

PMI			Precoding Matrix Index

RU			Resource Unit

SFBC		Space-Frequency Block Code

STC			Space-Time Coding

SU			Single-User

UL			Uplink

VE			Vertical Encoding

Insert a new section 15:

- Advanced Air Interface

  - Physical layer

### Introduction

### OFDMA symbol description, symbol parameters and transmitted signal

### Frame Structure

### Reserved

### Downlink physical structure

### Downlink physical control

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #2 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: recommended text proposal #2 to change the title of subcluase 15.3.7 to the proposed title]

### Downlink MIMO transmission schemes

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #3 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #3 on DL-MIMO architecture and layer-to-stream mapping]

#### Downlink MIMO architecture and data processing

The architecture of downlink MIMO at the transmitter side is shown in Figure 1.

Figure 1 – Downlink MIMO architecture

The MIMO encoder block maps L (≥1) layers onto Mt (≥L) streams, which are fed to the Precoder block. A layer is defined as a coding and modulation path fed to the MIMO encoder as an input. A stream is defined as an output of the MIMO encoder which is passed to the precoder.

For SU-MIMO, only one user is scheduled in one Resource Unit (RU), and only one FEC block exists at the input of the MIMO encoder (vertical MIMO encoding at transmit side).

For MU-MIMO, multiple users can be scheduled in one RU, and multiple FEC blocks exist at the input of the MIMO encoder (horizontal MIMO encoding at transmit side).

The Precoder block maps stream(s) to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The subcarrier mapping blocks map antenna-specific data to the OFDM symbol.

        - Layer to stream mapping

Layer to stream mapping is performed by the MIMO encoder. The MIMO encoder is a batch processor that operates on M input symbols at a time.

The input to the MIMO encoder is represented by an M×1 vector as specified in equation (1).

s=s1s2⋮sM						(1)

Where,

si is the i-th input symbol within a batch.

Layer to stream mapping of the input symbols is done in the space dimension first.

The output of the MIMO encoder is an Mt×NF MIMO STC matrix as given in equation (2), which serves as the input to the precoder.

x=S(s)						(2)

Where,

Mt is the number of streams

NF is the number of subcarriers occupied by one MIMO block

x is the output of the MIMO encoder

s is the input layer vector

S(s) is an STC matrix

And,

x=x1,1x1,2⋯x1,NFx2,1x2,2⋯x2,NF⋮⋮⋱⋮xMT,1xMT,2⋯xMT,NF					(3)

For SU-MIMO transmissions, the rate is defined as in equation (4).

R=MNF						(4)

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #4 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #4 on DL-MIMO encoding]

          - SFBC encoding

The input to the MIMO encoder is represented 2 × 1 vector.

s=s1s2							(5)

The MIMO encoder generates the SFBC matrix.

x=s1−s2s2s1						(6)

Where

x is 2x2 matrix

The SFBC matrix, x, occupies two consecutive subcarriers.

          - Vertical encoding

The input and the output of MIMO encoder is represented by an M  1 vector.

x=s=s1s2⋮sM						(7)

Where,

si is the i-th input symbol within a batch

For vertical encoding, s1…sM belong to the same layer.

          - Horizontal encoding

The input and output of the MIMO encoder is represented by an M  1 vector.

x=s=s1s2⋮sM						(8)

Where,

si is the i-th input symbol within a batch

For horizontal encoding, s1...sm belong to the different layers.

Horizontal encoding is only used for MU-MIMO mode.

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #5 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #5 on steam-to-antenna mapping and DL-MIMO precoding]

        - Stream to antenna mapping

- Stream to antenna mapping is performed by the precoder. The output of the MIMO encoder is multiplied by an Nt×Mt precoder, W. The output of the precoder is denoted by an Nt×NF matrix, z. The mapping can be defined in equation (9).

z=Wx=z1,1z1,2⋯z1,NFz2,1z2,2⋯z2,NF⋮⋮⋱⋮zNt,1zNt,2⋯zNt,NF				(9)

- Where,

- Nt is the number of transmit antennas

- zj,k is the output symbol to be transmitted via the j-th physical antenna on the k-th subcarrier

          - Non-adaptive precoding

With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the numbers of streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from a subset of size NW precoders of the base codebook for a given rank. The matrix W changes every N1PSC contiguous physical subcarriers according to equation (7-1), and it does not depend on the subframe number. W belongs to the subset of the base codebook specified in Section Error: Reference source not found. The Nt×Mt precoding matrix W(k) applied on subcarrier k is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

- .				(7-1)

          - Adaptive precoding

With adaptive precoding, the precoder W is derived from the feedback of the MS.

For codebook-based precoding (codebook feedback), there are 3 feedback modes: Base mode, adaptive mode and differential mode, which are described in 3.3.1.1.10.4.1.

For TDD sounding-based precoding, the value of W is derived from the MS sounding feedback. The sounding channel is defined in 3.3.1.1.10.5.

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #6 ~~~~~~~~~~~~~~~~~~~~~~~]

[Recommended text proposal #6 on DL MIMO modes and the title of a subclause ‘Mapping to data subcarriers’]

        - Downlink MIMO modes

There are five MIMO transmission modes for unicast DL MIMO transmission as listed in Table 1.

Table 1 – MIMO modes

| Mode index | Description | Reference |
| --- | --- | --- |
| Mode 0 | OL SU-MIMO (SFBC with non-adaptive precoder) |  |
| Mode 1 | OL SU-MIMO (SM with non-adaptive precoder) |  |
| Mode 2 | CL SU-MIMO (SM with adaptive precoder) |  |
| Mode 3 | OL MU-MIMO (SM with non-adaptive precoder) |  |
| Mode 4 | CL MU-MIMO (SM with adaptive precoder) |  |
| Mode 5 -7 | n/a | n/a |

Some parameters for each DL MIMO mode are shown in Table 2.

Table 2 – DL MIMO Parameters

|  | Nt | Rate | Mt | NF | L |
| --- | --- | --- | --- | --- | --- |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | n.a. | 2 | 1 | 2 |
|  | 4 | n.a. | 2 | 1 | 2 |
|  | 4 | n.a. | 3 | 1 | 3 |
|  | 4 | n.a. | 4 | 1 | 4 |
|  | 8 | n.a. | 2 | 1 | 2 |
|  | 8 | n.a. | 3 | 1 | 3 |
|  | 8 | n.a. | 4 | 1 | 4 |

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #7 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #7 on Encoding, precoding and mapping of SU-MIMO]

#### Transmission schemes for data channels

        - Encoding, precoding and mapping of SU-MIMO

          - Encoding of MIMO modes

            - MIMO mode 0

SFBC encoding of section 3.3.1.1.1.1 shall be used with MIMO mode 0.

            - MIMO mode 1

Vertical encoding of section 3.3.1.1.1.2 shall be used with MIMO mode 1. The number of streams is , where Nr is the number of receive antennas and Mt is no more than 8.

            - MIMO mode 2

Vertical encoding of section 3.3.1.1.1.2 shall be used with MIMO mode 2. The number of streams is , where Mt is no more than 8.

          - Precoding of MIMO modes

            - MIMO mode 0

Non-adaptive precoding of section 3.3.1.1.2.1 with Mt=2 streams shall be used with MIMO mode 0.

            - MIMO mode 1

Non-adaptive precoding of section 3.3.1.1.2.1 with Mt streams shall be used with MIMO mode 1.

            - MIMO mode 2

Adaptive precoding of section 3.3.1.1.2.2 shall be used with MIMO mode 2.

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #8 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #8 on Encoding, precoding and mapping of MU-MIMO]

        - Encoding, precoding and mapping of MU-MIMO

Multi-user MIMO schemes are used to enable a resource allocation to communicate data to two or more MSs. Multi-user transmission with one stream per user is supported for MU-MIMO.

MU-MIMO includes the MIMO configuration of 2Tx antennas to support up to 2 MSs, and 4Tx or 8Tx antennas to support up to 4 MSs, with 1 stream per MS.

Both OL MU-MIMO (mode 3) and CL MU-MIMO (mode 4) are supported.

###### Encoding of MIMO mode 3

Horizontal encoding of section 3.3.1.1.1.3 shall be used with MIMO mode 3.

###### Encoding of MIMO mode 4

Horizontal encoding of section 3.3.1.1.1.3 shall be used with MIMO mode 4.

###### Precoding of MIMO modes

            - MIMO mode 3

In OL MU MIMO, the precoder W is predefined and fixed over time. The definition of W is the same as OL SU MIMO (mode 0 and mode 1).

            - MIMO mode 4

In CL MU MIMO, the precoder W is an Nt × M matrix for each subcarrier. It is used to communicate to M MSs simultaneously. The form and derivation of the precoding matrix does not need to known at the MS. The BS determines the precoding matrix based on the feedback received from the MS.

The BS shall construct the precoding matrix W as represented in equation (10).

W(k)=[v1(k)v2(k)⋯vM(k)]				(10)

Where,

vi(k) is the precoding vector for the i-th MS on the k-th subcarrier.

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #9 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Rrecommended text proposal #9 on mapping of data/pilot subcarriers and usage of MIMO modes]

        - Mapping of data subcarriers

###### MIMO mode 0

###### MIMO mode 1, 2

###### MIMO mode 3, 4

        - Mapping of pilot subcarriers

        - Usage of MIMO modes

Table 3 shows permutations supported for each MIMO mode. The definition of DRU, mini-band based CRU, and subband based CRU are in subclause [TBD].

Table 3 – Supported permutation for each DL MIMO mode

| Permutation MIMO Mode | DRU | Mini-band based CRU (diversity allocation) | Mini-band and Subband based CRU (localized allocation) |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes | Yes | No |
| MIMO mode 1 | Yes, with Mt≤2 | Yes | Yes |
| MIMO mode 2 | No | Yes, with Mt=1 | Yes |
| MIMO mode 3 | No | No | Yes |
| MIMO mode 4 | No | Yes | Yes |

All pilots are precoded regardless of number of transmit antennas and allocation type.

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #10 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #10 on MIMO feedback type and signaling]

          - Broadcast information

Some parameters necessary for DL MIMO operation shall be broadcast by the BS. The broadcast information is carried by BCH or in DCD/UCD.

          - Unicast information

Some parameters necessary for DL MIMO operation shall be unicast by the BS to a specific MS. The unicast information is carried by A-MAP IEs or feedback allocation IEs.

        - Feedback mechanisms and operation

          - Downlink post-processing CINR measurement feedback

The reported channel quality indicator has three types: wideband CQI, subband CQI, and average CQI [TBD]

The wideband CQI is one average CQI over whole band.

The subband CQI is one average CQI over the subband.

The average CQI is one average over the best reported subband [TBD].

For MU-MIMO feedback modes, the CQI is calculated at the MS assuming that the interfering users are scheduled by the serving BS using rank-1 precoders orthogonal to each other and orthogonal to the rank-1 precoder represented by the reported PMI.

          - MIMO mode selection feedback

          - MIMO feedback information

Table 4 – MIMO feedback information

|  | Feedback information type | Description | Number of Bits | Parameters |
| --- | --- | --- | --- | --- |
| Long period feedback | Rank information | For MIMO modes 1 and 2 | TBD |  |
|  | Subband selection |  | TBD |  |
|  | Stream index (TBD) | For MIMO mode 3, indicating which streams are preferred. | TBD |  |
|  | Quantized Correlation matrix | For adaptive codebook feedback mode or long term wideband beamforming |  |  |
|  | Time Correlation coefficient information [TBD] | For differential codebook feedback mode |  |  |
|  | PMI report for serving cell | For long-term wideband beamforming |  |  |
|  | PMI report for neighboring cell | For PMI coordination among multiple BSs |  |  |
|  | CQI | Post-processing CINR for a given MIMO feedback mode |  |  |
| Short period feedback | CQI | Post-processing CINR for a given MIMO feedback mode | TBD |  |
|  | PMI report for serving cell | For short-term beamforming with MIMO modes 2 and 4 | TBD |  |
| Event-driven feedback | Preferred MFM (MIMO feedback mode) |  |  |  |

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #11 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #11 on quantized feedback modes and base mode for codebook-based feedback]

          - Quantized MIMO feedback for closed-loop transmit precoding

            - Quantized feedback modes

An MS may feedback a Preferred Matrix Index (PMI) to support DL precoding.

There are three types of codebook feedback modes.

The operation of the codebook feedback modes for the PMI is summarized below:

- The base mode: the PMI feedback from a MS shall represent an entry of the base codebook. It shall be sufficient for the BS to determine a new precoder.

- The adaptive mode: the PMI feedback from a MS shall represent an entry of the transformed base codebook according to long term channel information.

- The differential mode: the PMI feedback from a MS shall represent an entry of the differential codebook or an entry of the base codebook at PMI reset times. The feedback from a MS provides a differential knowledge of the short-term channel information. This feedback represents information that is used along with other feedback information known at the BS for determining a new precoder.

Mobile station shall support the base and adaptive mode and may support the differential mode.

The adaptive and differential feedback modes are applied to the base codebook or to a subset of the base codebook.

            - Base mode for codebook-based feedback

- The base codebook is a unitary codebook. A codebook is a unitary codebook if each of its matrices consists of columns of a unitary matrix.

The MS selects its preferred matrix from the base codebook based on the channel measurements. The MS feedbacks the index of the preferred codeword, and the BS computes the precoder W according to the index. Both BS and MS use the same codebook for correct operation.

- For the base mode, the PMI feedback from a mobile station shall represent an entry of the base codebook, where the base codebooks are defined as follows for two, four, and eight transmit antennas at the BS.

- The notation C(Nt, Mt, NB) denotes the codebook, which consists of 2NB complex, matrices of dimension Nt by Mt, and Mt denotes the number of streams.

- The notation C(Nt, Mt, NB, i) denotes the i-th codebook entry of C(Nt, Mt, NB).

              - Base codebook for two transmit antennas

                - SU-MIMO base codebook

- The codebook for two transmit antenna is constructed using a similar methodology as described in section 8.4.11.15 of WirelessMAN-OFDMA, with the exception that the first codeword v1 is defined as an Nt  by Mt unitary matrix

- v1=1Nt11⋯1ej2πNtej2πNt2j2πNtMt⋮⋮⋱ej2πNtNt−1ej2πNt2Nt−1ej2πNtMtNt−1			(11)

- All codeword matrices vi for i = 2,3…,2NB in V(Nt, Mt, NB) codebook can be derived from the first codeword matrix v1 using the following equations

- vi=HsQi−1HHsv1,				(12)

- vi=vi⋅e−jϕ1,					(13)

- where phase φ1 is a phase of the first entry of the first column of codeword vi. Operation H generates unitary matrix as follows

- H(v)=I, v=e1I−2wwHwHw, otherwise ,			(14)

- where w=v−e1, e1=1,0,...,0T.

- Matrix Qi is defined as following

- Qi=diage2π2Lu1i,e2π2Lu2i,...,e2π2LuNti,			(15)

- The parameters for generation of the codebooks for two transmit antenna Nt = 2 and number of streams Mt =1, 2 are listed in Table 5.

Table 5 – Generating parameters for two transmit antenna codebook

| Nt | Mt | NB | L | u=u1,u2,...,uNt in Qi(u) | s in H(s) |
| --- | --- | --- | --- | --- | --- |
| 2 | 1 | 3 | 3 | [1, 2] | [1, 0] |
| 2 | 2 | 2 | 3 |  |  |

- The indexes from 4 to 7 are not used in 3-bits downlink PMI feedback for Mt =2 codebook.

                - MU-MIMO base codebook

The base codebook for MU-MIMO is the same as the rank 1 base codebook for SU-MIMO, defined in 3.3.1.1.10.4.2.1.1.

              - Base codebook for four transmit antennas

                - SU-MIMO base codebook

The base codebooks of SU-MIMO with four transmit antennas consist of rank-1 codebook C(4,1,6), rank-2 codebook C(4,2,6), rank-3 codebook C(4,3,4) and rank-4 codebook C(4,4,3). Table 6, Table 7, Table 8 and Table 9 are included to illustrate the rank-1,2,3,4 base codebooks.

Table 6 – C(4,1,6)

| # Binary Index | # m | C(4,1,6,m) |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 0.5000 | -0.5000 | 0.5000 | -0.5000 |
| 000001 | 1 | -0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 000010 | 2 | -0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 000011 | 3 | 0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 000100 | 4 | -0.5000 | 0.0000 - 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 000101 | 5 | -0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 - 0.5000i |
| 000110 | 6 | 0.5000 | 0.5000 | 0.5000 | 0.5000 |
| 000111 | 7 | 0.5000 | 0.0000 + 0.5000i | 0.5000 | 0.0000 + 0.5000i |
| 001000 | 8 | 0.5000 | 0.5000 | 0.5000 | -0.5000 |
| 001001 | 9 | 0.5000 | 0.0000 + 0.5000i | -0.5000 | 0.0000 + 0.5000i |
| 001010 | 10 | 0.5000 | -0.5000 | 0.5000 | 0.5000 |
| 001011 | 11 | 0.5000 | 0.0000 - 0.5000i | -0.5000 | 0.0000 - 0.5000i |
| 001100 | 12 | 0.5000 | 0.3536 + 0.3536i | 0.0000 + 0.5000i | -0.3536 + 0.3536i |
| 001101 | 13 | 0.5000 | -0.3536 + 0.3536i | 0.0000 – 0.5000i | 0.3536 + 0.3536i |
| 001110 | 14 | 0.5000 | -0.3536 - 0.3536i | 0.0000 + 0.5000i | 0.3536 - 0.3536i |
| 001111 | 15 | 0.5000 | 0.3536 - 0.3536i | 0.0000 – 0.5000i | -0.3536 - 0.3536i |
| 010000 | 16 | 0.5000 | -0.4619 - 0.1913i | 0.3536 + 0.3536i | -0.1913 - 0.4619i |
| 010001 | 17 | 0.3117 | 0.6025 + 0.1995i | -0.4030 – 0.4903i | -0.1122 - 0.2908i |
| 010010 | 18 | 0.3117 | -0.6025 - 0.1995i | -0.1122 – 0.2908i | 0.4030 + 0.4903i |
| 010011 | 19 | 0.3058 | 0.1901 - 0.6052i | 0.1195 + 0.2866i | 0.4884 - 0.4111i |
| 010100 | 20 | 0.5000 | -0.1913 + 0.4619i | -0.3536 – 0.3536i | 0.4619 - 0.1913i |
| 010101 | 21 | 0.5000 | 0.1913 - 0.4619i | -0.3536 – 0.3536i | -0.4619 + 0.1913i |
| 010110 | 22 | 0.5000 | 0.4619 + 0.1913i | 0.3536 + 0.3536i | 0.1913 + 0.4619i |
| 010111 | 23 | 0.3082 | 0.0104 + 0.3151i | 0.4077 + 0.4887i | -0.4783 + 0.4145i |
| 011000 | 24 | 0.3117 | 0.3573 - 0.2452i | 0.6025 – 0.1995i | -0.1578 + 0.5360i |
| 011001 | 25 | 0.3117 | 0.2452 + 0.3573i | -0.6025 + 0.1995i | 0.5360 + 0.1578i |
| 011010 | 26 | 0.3082 | -0.3666 + 0.2426i | 0.6092 – 0.1842i | 0.1615 - 0.5298i |
| 011011 | 27 | 0.3117 | -0.2452 - 0.3573i | -0.6025 + 0.1995i | -0.5360 - 0.1578i |
| 011100 | 28 | 0.3117 | 0.4260 + 0.0793i | 0.1995 + 0.6025i | 0.2674 + 0.4906i |
| 011101 | 29 | 0.3117 | -0.0793 + 0.4260i | -0.1995 – 0.6025i | 0.4906 - 0.2674i |
| 011110 | 30 | 0.3117 | -0.4260 - 0.0793i | 0.1995 + 0.6025i | -0.2674 - 0.4906i |
| 011111 | 31 | 0.3117 | 0.0793 - 0.4260i | -0.1995 – 0.6025i | -0.4906 + 0.2674i |
| 100000 | 32 | 0.5636 | -0.3332 - 0.2672i | 0.1174 + 0.5512i | -0.3308 - 0.2702i |
| 100001 | 33 | 0.5587 | 0.3361 + 0.2735i | -0.3361 – 0.2735i | -0.1135 - 0.5471i |
| 100010 | 34 | 0.5587 | -0.3361 - 0.2735i | -0.1135 – 0.5471i | 0.3361 + 0.2735i |
| 100011 | 35 | 0.5587 | 0.2735 - 0.3361i | 0.1135 + 0.5471i | 0.2735 - 0.3361i |
| 100100 | 36 | 0.3082 | -0.4887 + 0.4077i | -0.6092 – 0.1842i | 0.2837 - 0.1205i |
| 100101 | 37 | 0.5636 | 0.2673 - 0.3331i | -0.1222 – 0.5501i | -0.2673 + 0.3331i |
| 100110 | 38 | 0.5636 | 0.3691 + 0.5142i | 0.3331 + 0.2673i | 0.0862 + 0.3032i |
| 100111 | 39 | 0.5587 | -0.2990 + 0.0880i | 0.3361 + 0.2735i | -0.5216 + 0.3616i |
| 101000 | 40 | 0.5587 | 0.0880 - 0.2990i | 0.3361 – 0.2735i | -0.3616 + 0.5216i |
| 101001 | 41 | 0.5587 | 0.2990 + 0.0881i | -0.3362 + 0.2735i | 0.5216 + 0.3616i |
| 101010 | 42 | 0.5587 | -0.0880 + 0.2990i | 0.3361 – 0.2735i | 0.3616 - 0.5216i |
| 101011 | 43 | 0.5587 | -0.2990 - 0.0880i | -0.3361 + 0.2735i | -0.5216 - 0.3616i |
| 101100 | 44 | 0.5636 | 0.2741 - 0.1559i | 0.2672 + 0.3332i | 0.1081 + 0.6236i |
| 101101 | 45 | 0.5636 | 0.1559 + 0.2741i | -0.2672 – 0.3332i | 0.6236 - 0.1081i |
| 101110 | 46 | 0.5587 | -0.2737 + 0.1492i | 0.2735 + 0.3361i | -0.1132 - 0.6245i |
| 101111 | 47 | 0.5587 | -0.1492 - 0.2737i | -0.2735 - 0.3361i | -0.6245 + 0.1132i |
| 110000 | 48 | 0.5000 | -0.4619 + 0.1913i | 0.3536 - 0.3536i | -0.1913 + 0.4619i |
| 110001 | 49 | 0.3117 | 0.4030 + 0.4903i | -0.6025 - 0.1995i | -0.1122 - 0.2908i |
| 110010 | 50 | 0.3117 | -0.4029 - 0.4904i | -0.1184 - 0.2883i | 0.6067 + 0.1865i |
| 110011 | 51 | 0.3082 | 0.4887 - 0.4077i | 0.1205 + 0.2837i | 0.1842 - 0.6092i |
| 110100 | 52 | 0.5000 | 0.1913 + 0.4619i | -0.3536 + 0.3536i | -0.4619 - 0.1913i |
| 110101 | 53 | 0.5000 | -0.1913 - 0.4619i | -0.3536 + 0.3536i | 0.4619 + 0.1913i |
| 110110 | 54 | 0.5000 | 0.4619 - 0.1913i | 0.3536 - 0.3536i | 0.1913 - 0.4619i |
| 110111 | 55 | 0.3117 | -0.2452 + 0.3573i | 0.6025 + 0.1995i | -0.5360 + 0.1578i |
| 111000 | 56 | 0.3117 | 0.3117 - 0.0000i | 0.4030 - 0.4903i | -0.4030 + 0.4903i |
| 111001 | 57 | 0.3117 | -0.0000 + 0.3117i | -0.4030 + 0.4903i | 0.4903 + 0.4030i |
| 111010 | 58 | 0.3082 | -0.3152 - 0.0036i | 0.4076 - 0.4888i | 0.4040 - 0.4872i |
| 111011 | 59 | 0.3082 | 0.0036 - 0.3152i | -0.4076 + 0.4888i | -0.4872 - 0.4040i |
| 111100 | 60 | 0.3117 | 0.2204 + 0.2204i | 0.4903 + 0.4030i | 0.0618 + 0.6317i |
| 111101 | 61 | 0.3117 | -0.2204 + 0.2204i | -0.4903 - 0.4030i | 0.6317 - 0.0618i |
| 111110 | 62 | 0.3082 | -0.2154 - 0.2302i | 0.4887 + 0.4077i | -0.0451 - 0.6313i |
| 111111 | 63 | 0.3082 | 0.2254 - 0.2204i | -0.4888 - 0.4076i | -0.6302 + 0.0588i |

Table 7 – C(4,2,6)

| # C(4,2,6,m)=C(4,1,6,i)C(4,1,6,j) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary index | m | i,j | Binary index | m | i,j | Binary index | m | i,j |  | Binary index | m | i,j |
| 000000 | 0 | 6,0 | 010000 | 16 | 0,4 | 100000 | 32 | 12,14 | 110000 |  | 48 | 32,34 |
| 000001 | 1 | 6,1 | 010001 | 17 | 0,5 | 100001 | 33 | 12,15 | 110001 |  | 49 | 34,35 |
| 000010 | 2 | 6,2 | 010010 | 18 | 0,33 | 100010 | 34 | 12,31 | 110010 |  | 50 | 35,7 |
| 000011 | 3 | 0,1 | 010011 | 19 | 1,3 | 100011 | 35 | 13,14 | 110011 |  | 51 | 40,11 |
| 000100 | 4 | 0,2 | 010100 | 20 | 1,34 | 100100 | 36 | 5,23 | 110100 |  | 52 | 41,43 |
| 000101 | 5 | 1,2 | 010101 | 21 | 1,55 | 100101 | 37 | 14,15 | 110101 |  | 53 | 44,46 |
| 000110 | 6 | 7,4 | 010110 | 22 | 2,3 | 100110 | 38 | 14,47 | 110110 |  | 54 | 45,47 |
| 000111 | 7 | 7,5 | 010111 | 23 | 8,9 | 100111 | 39 | 17,2 | 110111 |  | 55 | 49,6 |
| 001000 | 8 | 3,4 | 011000 | 24 | 2,39 | 101000 | 40 | 17,3 | 111000 |  | 56 | 52,53 |
| 001001 | 9 | 3,5 | 011001 | 25 | 8,11 | 101001 | 41 | 18,19 | 111001 |  | 57 | 56,10 |
| 001010 | 10 | 6,4 | 011010 | 26 | 8,27 | 101010 | 42 | 18,6 | 111010 |  | 58 | 56,58 |
| 001011 | 11 | 6,5 | 011011 | 27 | 9,10 | 101011 | 43 | 24,9 | 111011 |  | 59 | 57,11 |
| 001100 | 12 | 7,1 | 011100 | 28 | 9,42 | 101100 | 44 | 25,10 | 111100 |  | 60 | 57,59 |
| 001101 | 13 | 7,2 | 011101 | 29 | 10,11 | 101101 | 45 | 28,13 | 111101 |  | 61 | 60,14 |
| 001110 | 14 | 8,10 | 011110 | 30 | 10,43 | 101110 | 46 | 29,14 | 111110 |  | 62 | 61,15 |
| 001111 | 15 | 13,15 | 011111 | 31 | 12,13 | 101111 | 47 | 30,15 | 111111 |  | 63 | 61,63 |

Table 8 – C(4,3,4)

| Binary index | m | i,j,k | Binary index | m | i,j,k | Binary index | m | i,j,k |  | Binary Index | m | i,j,k |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 6,0,1 | 000100 | 4 | 7,3,4 | 001000 | 8 | 6,0,4 | 001100 |  | 12 | 8,9,10 |
| 000001 | 1 | 6,0,2 | 000101 | 5 | 7,3,5 | 001001 | 9 | 6,4,5 | 001101 |  | 13 | 8,10,11 |
| 000010 | 2 | 6,1,2 | 000110 | 6 | 7,4,5 | 001010 | 10 | 7,3,1 | 001110 |  | 14 | 12,13,15 |
| 000011 | 3 | 0,1,2 | 000111 | 7 | 3,4,5 | 001011 | 11 | 7,1,2 | 001111 |  | 15 | 13,14,15 |

Table 9 – C(4,4,3)

| Binary index | m | i,j,k,p | Binary index | m | i,j,k,p | Binary index | m | i,j,k,p |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 000000 | 0 | 6,0,1,2 | 000010 | 2 | 6,0,4,5 | 000100 | 4 | 8,9,10,11 |
| 000001 | 1 | 7,3,4,5 | 000011 | 3 | 7,3,1,2 | 000101 | 5 | 12,13,14,15 |

In terms of the chordal distance, the hierarchical structure of C(4,1,6) is depicted in Figure 1. In this hierarchical structure, it is shown that C(4,1,6) consists of 16 codeword clusters. Each codeword cluster has four codewords, of which one codeword is from Layer 1 and the three other codewords are from Layer 2. For any given Layer 2 codeword, its chordal distance to all other Layer 1 codewords of different clusters is always much larger than that distance to the Layer 1 codeword of its same cluster.

Figure 2: Chordal distance map of C(4,1,6)

As a potential benefit, this hierarchical structure can facilitate codeword searching. More specifically, codeword searching in C(4,1,6) can start from all Layer 1 codewords. Only when a Layer 1 codeword satisfies a certain criterion, associated Layer 2 codewords within the same cluster need to be searched.

The binary indices of the codewords in cluster i, i∈[0,⋯,15], is given by Table xxx

Table 10 – Binary indices of the codewords in cluster i

| Codeword in cluster i | Layer 1 codeword | Layer 2 codewords |  |  |
| --- | --- | --- | --- | --- |
|  |  | Codeword 1 | Codeword 2 | Codeword 3 |
| Binary index | 00xi3xi2xi1xi0 | 01xi3xi2xi1xi0 | 10xi3xi2xi1xi0 | 11xi3xi2xi1xi0 |
| i=xi3×23+xi2×22+xi1×2+xi0, xij∈[0,1], i∈[0,⋯,15], j∈[0,1,2,3] |  |  |  |  |

                - MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in 3.3.1.1.10.4.2.2.1.

              - Base codebook for eight transmit antennas

                - SU-MIMO base codebook

- The base codebook is constructed from two matrices V8(:,:,1) and V8(:,:,2), which are constructed as described below.

- Define

- (16)

- (17)

- The two rank-8 matrices used for rank-2 to rank-8 transmission for SU-MIMO are

- (18)

- (19)

- The third base matrix, V8(:,:,3), is used for rank-1 transmission for MU-MIMO. The j-th column vector of the base matrix V8(:,:,3) is given by

- where the set θj, j=1,...,16, is given by

- The 4bits 8Tx base codebook is constructed from the three 8×8 base matrices and is specified in Table 11 and Table 12. Note that only the column indices of the corresponding base matrices are shown in Table 7 for brevity.

Table 11 – Rank 1 of SU MIMO 4bit 8Tx base codebook

| Codebook Matrix Index (CMI) | Base Matrix | C(8,1,4) |
| --- | --- | --- |
| 1 | V8(:,:,3) | V8(:,1,3) |
| 2 |  | V8(:,2,3) |
| 3 |  | V8(:,3,3) |
| 4 |  | V8(:,4,3) |
| 5 |  | V8(:,5,3) |
| 6 |  | V8(:,6,3) |
| 7 |  | V8(:,7,3) |
| 8 |  | V8(:,8,3) |
| 9 |  | V8(:,9,3) |
| 10 |  | V8(:,10,3) |
| 11 |  | V8(:,11,3) |
| 12 |  | V8(:,12,3) |
| 13 |  | V8(:,13,3) |
| 14 |  | V8(:,14,3) |
| 15 |  | V8(:,15,3) |
| 16 |  | V8(:,16,3) |

Table 12 – Ranks 2 to 8 of SU MIMO 4bit 8Tx base codebook

| Codebook Matrix Index (CMI) | Base Matrix | C(8,2,4) | C(8,3,4) | C(8,4,4) | C(8,5,4) | C(8,6,4) | C(8,7,4) | C(8,8,4) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 1 5 | 1 3 5 | 1537 | 12357 | 123567 | 1234567 | 12345678 |
| 2 |  | 2 6 | 2 4 6 | 2648 | 12468 | 124568 | 1234568 | n/a |
| 3 |  | 3 7 | 2 3 7 | 3726 | 23467 | 234678 | 1234678 | n/a |
| 4 |  | 4 8 | 1 4 8 | 4815 | 13458 | 134578 | 1234578 | n/a |
| 5 |  | 5 3 | 3 5 7 | 5372 | 23567 | 234567 | 2345678 | n/a |
| 6 |  | 4 6 | 4 6 8 | 6481 | 14568 | 134568 | 1345678 | n/a |
| 7 |  | 2 7 | 2 6 7 | 7264 | 24678 | 124678 | 1245678 | n/a |
| 8 |  | 8 1 | 1 5 8 | 8153 | 13578 | 123578 | 1235678 | n/a |
| 9 |  | 1 3 | 1 2 3 | 1234 | 12345 | 123456 | 1234567 | 12345678 |
| 10 |  | 2 4 | 1 2 4 | 1246 | 12456 | 124567 | 1245678 | n/a |
| 11 |  | 2 3 | 2 3 4 | 2437 | 23478 | 123478 | 1234578 | n/a |
| 12 |  | 1 4 | 1 3 4 | 1348 | 13478 | 134678 | 1234678 | n/a |
| 13 |  | 5 8 | 5 7 8 | 3578 | 23578 | 235678 | 1235678 | n/a |
| 14 |  | 6 7 | 6 7 8 | 4678 | 14678 | 145678 | 1345678 | n/a |
| 15 |  | 5 7 | 5 7 6 | 5678 | 35678 | 345678 | 2345678 | n/a |
| 16 |  | 6 8 | 5 6 8 | 1568 | 13568 | 123568 | 1234568 | n/a |

                - MU-MIMO base codebook

The base codebook for MU-MIMO is same as the rank 1 base codebook for SU-MIMO, defined in 3.3.1.1.10.4.2.3.1.

              - Codebook subset selection

In codebook-based precoding, the precoding matrix W(k) shall be derived from PMI within the base codebook or a subset thereof. Subset information such as BC_SI, SU_CT, or MU_CT [TBD] is transmitted in [TBD].

Base Codebook Subset Indication (BC_SI) field determines which element of rank-1 codebook element is restricted or recommended for PMI feedback in case of MIMO mode 2 and 4. If the i-th element of BC_SI is set to 0, then i -th element of rank-1 codebook, C(Nt, 1, NB, i), is restricted for PMI feedback. This field shall be ignored when Codebook Coordination Enable (CCE) is set to 0b0. CCE is transmitted in [Feedback_Allocation_IE].

                - OL MIMO subset

                - CL SU-MIMO subset

- 15.3.7.2.6.4.2.4.2.1. CL SU-MIMO subset for four transmit antennas

Codebook subset selection for four transmit antennas is specified in Table 13.

Table 13 – Subset selection of the base codebook for four transmit antennas

| Rank | One | Two | Three | Four |
| --- | --- | --- | --- | --- |
| Subset selection | C(4,1,6,m) m = 0 to15 | C(4,2,6,m) m = 0 to15 | C(4,3,4,m) m = 0 to15 | C(4,4,3,m) m = 0 to 5 |

                - CL MU-MIMO subset

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #12 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #12 on adaptive codebook-based feedback and differential codebook-based feedback]

            - Adaptive codebook-based feedback mode

- The base codebooks and their subsets for SU and MU MIMO can be transformed as a function of the BS transmit correlation matrix. A quantized representation of the BS transmit correlation matrix shall be feedback by the MS as instructed by the BS.

- For the adaptive mode, the PMI feedback from a mobile station shall represent an entry of the transformed base codebook according to long term channel information.

In adaptive mode, both BS and MS transform the base codebook to a transformed codebook using the correlation matrix.

The transformation for codewords of rank 1 is of the form in equation (20).

(20)

The transformation for codewords of rank > 1 is of the form in equation (21).

Vi=orthR Vi						(21)

Where,

X is the input matrix (or vector),

Vi is the i-th codeword of the original codebook,

Vi is the i-th codeword of the transformed codebook,

R is the Nt × Nt transmit correlation matrix.

orthX converts the input matrix (or vector) X to an orthogonal matrix with orthogonal column(s) that span the same subspace as the columns of X. The correlation matrix R contains the averaged directions for precoding.

After obtaining the transformed codebook, both MS and BS shall use the transformed codebook for the feedback and precoding process.

The correlation matrix R shall be feedbacked to support adaptive mode of codebook-based precoding.

R is feedbacked every Nx superframes (Nx is TBD) and one correlation matrix is valid for whole band.

During some time period and in the whole band, the correlation matrix is measured as

R=E(HijHHij)						 (22)

Where

R is the Nt by Nt transmit covariance matrix.

Hij is the correlated channel matrix in the i-th OFDM symbol period and j-th subcarriers.

R matrix is updated every Nx super frames (Nx is TBD)

The measured correlation matrix has the format of

R=r11r12conjr12r22 (NT=2)

R=r11r12r13r14conjr12r22r23r24conjr13conjr23r33r34conjr14conjr24conjr34r44 (NT=4) 			 (23)

where the diagonal entries are positive and the non-diagonal entries are complex. Because of the symmetriy of the correlation matrix, only the upper triangular elements shall be feedbacked after quantization.

R matrix is normalized by the maximum element (amplitude), and then quantized to reduce the feedback overhead.

The equation of normalization is

R=Rmax(abs(ri,j))	(i,j=1:Nt)			 (24)

The normalized diagonal elements are quantized by 1 bit, and the normalized complex elements are quantized by 4 bits.

The equation for quantization is

q=a*exp(j∗b∗2π)					 (25)

a=[0.6 0.9] and b=0 for diagonal entries

| Diagonal Entries | a | b | q |
| --- | --- | --- | --- |
| q1 | 0.6 | 0 | 0.6000 |
| q2 | 0.9 | 0 | 0.9000 |

a=[0.1 0.5] and b=[0 1/8 1/4 3/8 1/2 5/8 3/4 7/8] for non-diagonal upper trangular entries.

| non-Diagonal Entries | a | b | q |
| --- | --- | --- | --- |
| q1 | 0.1 | 0 | 0.1000 |
| q2 | 0.1 | 1/8 | 0.0707 + 0.0707i |
| q3 | 0.1 | 1/4 | 0.0000 + 0.1000i |
| q4 | 0.1 | 3/8 | -0.0707 + 0.0707i |
| q5 | 0.1 | 1/2 | -0.1000 + 0.0000i |
| q6 | 0.1 | 5/8 | -0.0707 - 0.0707i |
| q7 | 0.1 | 3/4 | -0.0000 - 0.1000i |
| q8 | 0.1 | 7/8 | 0.0707 - 0.0707i |
| q9 | 0.5 | 0 | 0.5000 |
| q10 | 0.5 | 1/8 | 0.3536 + 0.3536i |
| q11 | 0.5 | 1/4 | 0.0000 + 0.5000i |
| q12 | 0.5 | 3/8 | -0.3536 + 0.3536i |
| q13 | 0.5 | 1/2 | -0.5000 + 0.0000i |
| q14 | 0.5 | 5/8 | -0.3536 - 0.3536i |
| q15 | 0.5 | 3/4 | -0.0000 - 0.5000i |
| q16 | 0.5 | 7/8 | 0.3536 - 0.3536i |

The total overhead is 6 bits for 2 transmit antennas and 28 bits for 4 transmit antenna.

The MS and BS shall use the same transformation based on the correlation matrix fed back by the MS.

            - Differential codebook-based feedback mode

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #13 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #13 on unquantized MIMO feedback]

          - Unquantized MIMO feedback for closed-loop transmit precoding

            - UL Sounding

To assist the BS in determining the precoding matrix to use for SU-MIMO or MU-MIMO, the BS may request the MS transmit a sounding signal in an UL sounding channel. The BS may translate the measured UL channel response to an estimated DL channel response. The transmitter and receiver hardware of BS and MS shall be calibrated.

The UL sounding channel defined in subclause [TBD] is used in MIMO transmission.

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #14 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #14 on transmission schemes for control channels and MIMO transmission schemes for E-MBS]

      - Transmission schemes for control channels

        - Superframe Header (SFH)

For two BS transmit antennas, the P-SFH and the S-SFH shall be transmitted using SFBC.

The input to the MIMO encoder is represented by a 2 × 1 vector.

(26)

The MIMO encoder generates the SFBC matrix.

(27)

The two-stream pilot pattern defined in 15.3.5.x is used for SFH transmission.

        - Advanced MAP (A-MAP)

MIMO mode 0 shall be used for transmission of the A-MAP.

Two stream pilot pattern defined in 15.3.5.x shall be used for A-MAP transmission.

      - MIMO transmission schemes for E-MBS

    - Uplink physical structure

    - Uplink physical control

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #15 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #15 to change the title of subcluase 15.3.10 to the proposed title]

    - Uplink MIMO transmission schemes

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #16 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #16 on uplink MIMO architecture and data processing]

      - Uplink MIMO architecture and data processing

The architecture of uplink MIMO at the transmitter side is shown in Figure 3.

Figure 3: UL MIMO Architecture

The MIMO encoder block maps a single layer (L = 1) onto Mt (≥1) streams, which are fed to the Precoder block. A layer is defined as a coding and modulation path fed to the MIMO encoder as an input. A stream is defined as an output of the MIMO encoder which is passed to the precoder.

For SU-MIMO and Collaborative spatial multiplexing (MU-MIMO), only one FEC block exists in the allocated RU (vertical MIMO encoding at transmit side).

The Precoder block maps stream(s) to antennas by generating the antenna-specific data symbols according to the selected MIMO mode.

The MIMO encoder and precoder blocks shall be omitted when the MS has one transmit antenna.

The subcarrier mapping blocks map antenna-specific data to the OFDM symbol.

        - Layer to stream mapping

Layer to stream mapping is performed by the MIMO encoder. The uplink MIMO encoder is identical to the downlink MIMO encoder described in section 3.3.1.1.1.

          - SFBC encoding

Uplink SFBC encoding is identical to the downlink SFBC encoding described in section 3.3.1.1.1.1.

          - Vertical encoding

Uplink vertical encoding is identical to the downlink vertical encoding described in section 3.3.1.1.1.2.

        - Stream to antenna mapping

Stream to antenna mapping is performed by the precoder. The uplink mapping is identical to the downlink mapping described in section 3.3.1.1.2.

          - Non-adaptive precoding

There is no precoding if there is only one transmit antenna at the MS.

- With non-adaptive precoding, the precoding matrix is an Nt×Mt matrix W(k), where Nt is the number of transmit antennas, Mt is the numbers of streams, and k is the physical index of the subcarrier where W(k) is applied. The matrix W is selected from a subset of size NW precoders of the base codebook for a given rank. The matrix W changes every N1PSC contiguous physical subcarriers according to equation (27-1), and it does not depend on the subframe number. W belongs to the subset of the base codebook specified in Section [TBD]. The Nt×Mt precoding matrix W(k) applied on subcarrier k is selected as the codeword of index i in the open-loop codebook subset of rank Mt, where i is given by

- .		(27-1)

          - Adaptive precoding

There is no precoding if there is only one transmit antenna at the MS.

With adaptive precoding, the precoder W is derived at the BS or at the MS, as instructed by the BS.

With 2Tx or 4Tx at the MS in FDD and TDD systems, unitary codebook based adaptive precoding is supported. In this mode, a MS transmits a sounding signal on the uplink to assist the precoder selection at the BS. The BS shall signal the uplink precoding matrix index to be used by the MS in the UL A-MAP IE.

With 2Tx or 4Tx at the MS in TDD systems, adaptive precoding based on the measurements of downlink reference signals is supported. The MS chooses the precoder based on the downlink measurements. The form and derivation of the precoding matrix does not need to be known at the BS.

        - Uplink MIMO transmission modes

There are five MIMO transmission modes for UL MIMO transmission as listed in Table 14Table 15.

Table 14 Uplink MIMO modes

| Mode index | Description | Reference |
| --- | --- | --- |
| Mode 0 | OL SU-MIMO (SFBC with non-adaptive precoder) |  |
| Mode 1 | OL SU-MIMO (SM with non-adaptive precoder) |  |
| Mode 2 | CL SU-MIMO (SM with adaptive precoder) |  |
| Mode 3 | OL Collaborative spatial multiplexing (MU-MIMO) |  |
| Mode 4 | CL Collaborative spatial multiplexing (MU-MIMO) |  |
| Mode 5 -7 | n/a | n/a |

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #17 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #17 on transmission schemes for data channels]

      - Transmission schemes for data channels

        - Encoding, precoding and mapping of SU-MIMO

          - Encoding of MIMO modes

            - MIMO mode 0

SFBC encoding of section 3.3.4.1.1.1 shall be used with MIMO mode 0.

            - MIMO mode 1

Vertical encoding of section 3.3.4.1.1.2 shall be used with MIMO mode 1. The number of streams is , where Mt is no more than 4.

            - MIMO mode 2

Vertical encoding of section 3.3.4.1.1.2 shall be used with MIMO mode 2. The number of streams is , where Mt is no more than 4.

          - Precoding of MIMO modes

            - MIMO mode 0

Non-adaptive precoding with Mt=2 streams of section 3.3.4.1.2.1 shall be used with MIMO mode 0.

            - MIMO mode 1

Non-adaptive precoding of section 3.3.4.1.2.1 shall be used with MIMO mode 1.

            - MIMO mode 2

Adaptive precoding of section 3.3.4.1.2.2 shall be used with MIMO mode 2.

        - Encoding, precoding and mapping of Collaborative spatial multiplexing (MU-MIMO)

MSs can perform collaborative spatial multiplexing onto the same RU. In this case, the BS assigns different pilot patterns for each MS. Collaborative spatial multiplexing can be used to implement Cooperative Transmission.

###### Encoding of MIMO mode 3

Vertical encoding of section 3.3.4.1.1.2 shall be used with MIMO mode 3.

###### Encoding of MIMO mode 4

Vertical encoding of section 3.3.4.1.1.2 shall be used with MIMO mode 4.

###### Precoding of MIMO modes

            - MIMO mode 3

Non-adaptive precoding of section 3.3.4.1.2.1 shall be used with MIMO mode 3.

            - MIMO mode 4

Adaptive precoding of section 3.3.4.1.2.2 shall be used with MIMO mode 4.

        - Mapping of data subcarriers

###### MIMO mode 0

{The example and figure to be added for DRU and reference for DL CRU}

###### MIMO mode 1 and mode 2

{The example and figure to be added, for DRU and reference DL CRU}

###### MIMO mode 3 and mode 4

{The example and figure to be added, for DRU and reference DL CRU}

        - Mapping of pilot subcarriers

        - Usage of MIMO modes

[Description of MIMO mode usage according to the type of permutation, usage of pilots, etc]

The following table shows the permutations supported for each MIMO mode. The definition of tile based DRU, mini-band based CRU, and subband based CRU are in 15.3.5.x.

Table 16 – Supported permutation for each UL MIMO mode

| Permutation MIMO Mode | Tile based DRU | Mini-band based CRU (diversity allocation) | Mini-band based CRU Sub-band based CRU (localized allocation) |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes | Yes | No |
| MIMO mode 1 | Yes | Yes | Yes |
| MIMO mode 2 | Yes | Yes | Yes |
| MIMO mode 3 | Yes | Yes | Yes |
| MIMO mode 4 | Yes | Yes | Yes |

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #18 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #18 on transmission schemes for control channels]

      - Codebook for closed-loop transmit precoding

              - Base codebook for two transmit antennas

                - SU-MIMO base codebook

The base codebook of uplink 2Tx is the same as the downlink 2Tx base codebook (SU MIMO base codebook), defined in 3.3.1.1.10.4.2.1.1.

                - MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as base codebook for SU-MIMO, defined in 3.3.4.3.1.1.1.1.1

              - Base codebook for four transmit antennas

                - SU-MIMO base codebook

- The codebooks for UL MIMO with four transmit antenna MS are constructed using the methodology described in section 3.3.1.1.10.4.2.1.1 for two transmit antenna case.

- The parameters for the generation of codebooks for four transmit antenna Nt = 4 and number of streams Mt =1, 2, 3, 4 are listed in Table 9.

- Table 9. – Generating parameters for four transmit antenna codebook

| Nt | Mt | NB | L | u=u1,u2,...,uNt in Qi(u) | s in H(s) |
| --- | --- | --- | --- | --- | --- |
| 4 | 1 | 6 | 6 | [18, 55, 22, 6] | [1, 0, 0, 0] |
| 4 | 2 | 6 | 6 |  |  |
| 4 | 3 | 6 | 6 |  |  |
| 4 | 4 | 6 | 6 |  |  |

                - MU-MIMO base codebook

The base codebook for UL collaborative spatial multiplexing MIMO is same as the base codebook for UL SU-MIMO, defined in 3.3.4.3.1.1.1.2.1

      - Transmission schemes for control channels

    - Multi-BS MIMO

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #19 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #19 on downlink signaling support of DL-MIMO modes into DL control sections]

Insert the following sub-section before 15.3.7.2.6.4. (Quantized MIMO feedback for closed-loop transmit precoding)

- 15.3.x.x.1.  Downlink signaling support of DL-MIMO modes

[Description of DL signaling support of MIMO modes, description of rank and mode adaptation at BS]

[Table x may eventually need to be split into broadcast and unicast parameters, and the parameters are highlighted for further discussion]

Table x – DL MIMO control parameters

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - Broadcast Information |  |  |  |  |
| - Nt | - Number of transmit antennas at the BS | - 0b00: 2 - 0b01: 4 - 0b10: 8 | - SFH (system information) | - Nt must be known before decoding the DL A-MAP IE |
| - OL_Region [TBD] | - OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) | - TBD | - Broadcast information |  |
| - SU_CT | - SU base codebook type |  | - Broadcast information | - SU base codebook subset indication |
| - MU_CT (TBD) | - MU base codebook type |  | - Broadcast Information | - MU base codebook subset indication |
| - BC_SI | - Rank-1 base codebook subset indication | - BitMAP - (Same size as rate-1 codebook for each number of transmit antenna) | - Broadcast information | - Rate-1 codebook element restriction/recommendation information - It shall be ignored if CCE = 0b0 |
| - MaxMt (TBD) | - Maximum number of streams | - 0b00: 2 - 0b01: 3 - 0b10: 4 - 0b11: reserved | - Broadcast information | - If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| - Unicast Information |  |  |  |  |
| - MEF | - MIMO encoder format | - 0b00: SFBC - 0b01: Vertical encoding - 0b10: Horizontal encoding - 0b11: n/a | - A-MAP IE (unicast) | - MIMO encoder format. - [MEF bitfield may not be explicitly indicated in DL A-MAP IE] |
| - Mt | - Number of streams in transmission | - 0b000: 1 - 0b001: 2 - 0b010: 3 - 0b011: 4 - 0b100: 5 - 0b101: 6 - 0b110: 7 - 0b111: 8 - (Mt <= Nt ) | - A-MAP IE (unicast) | - Number of streams in the transmission. - When MEF=0b00: Mt =2 MEF=0b10, Mt <= 4 - [Bit-field length is variable, depending on the number of Tx at BS] |
| - RU allocation (TBD) | - RU [and stream] indicator for the burst of data | - TBD | - A-MAP IE (unicast) | - Refer to DL control group. |
| - SI (TBD) | - Index of pilot stream allocation | - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 | - A-MAP IE (unicast) | - SI shall be indicated if MEF = 0b010 - [Bit-field length is variable, depending on the number of Tx at BS] - RU allocation and SI can be merged together depending on other DG’s decision |
| - Feedback Allocation IE |  |  |  |  |
| - MFM | - MIMO feedback mode | - Refer to Table 5 | - Feedback allocation IE (unicast) | - To decide the feedback content and related MS processing |
| - DLRU (TBD) | - Downlink RU, indicating - which RUs or which type of RU (DRU or miniband-based CRU) to work on for feedback | - TBD (Tree structure, bit map etc) | - Feedback allocation IE (unicast) | - To process CQI (PMI) estimation for the indicated RUs - Refer to other DG |
| - FT | - MIMO feedback type | - 0b00: codebook - 0b01: sounding | - Feedback allocation IE (unicast) |  |
| - CM | - Codebook feedback mode | - 0b00: standard - 0b01: adaptive - 0b10: differential | - Feedback allocation IE (unicast) | - Enabled when FT = 0b00 |
| - CCE | - Codebook Coordination Enable | - 0b0: Disable - 0b1: Enable |  | - CCE = 0b0: MS finds PMI within whole broadcasted codebook type entry - CCE = 0b1: When MS finds rate-1 PMI, it finds within broadcasted codebook entries indicated by BC_ST, [SU_CT and MU_CT] |

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #20 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #20 on DL control information for DL-MIMO modes into DL control section]

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #21 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #21 on MIMO feedback modes into UL control section]

Propose to defer this decision to the May meeting, and not to include this table in the AWD at this meeting.

- 15.3.x.x.3.  MIMO feedback modes

Each MIMO transmission mode can have one or several kind of MIMO feedback modes. When allocating a feedback channel, the MIMO feedback mode shall be indicated to the MS, and the MS will feedback information accordingly.

The description of MIMO feedback modes and corresponding supported MIMO transmission modes is shown in Table xx. The detailed description of feedback and MS processing are in the following subsections.

Table xx MIMO feedback modes

| Feedback mode | Description | Feedback content | Type of RU | Supported MIMO transmission mode | Parameters |
| --- | --- | --- | --- | --- | --- |
| Mode 0 | OL SU MIMO STBC/SM (Diversity) | 1. Rank 2. Wideband CQI | Diversity(DRU) | MIMO mode 0, and MIMO mode 1 (Mt=2), support the flexible adaptation between the two modes | TBD |
| Mode 1 | OL SU MIMO STBC/SM (diversity) | 1. Rank 2. Wideband CQI | Diversity (Miniband-based CRU) | MIMO mode 0, and MIMO mode 1 support the flexible adaptation between the two modes |  |
| Mode 2 (TBD) | OL SU MIMO SM (Diversity) | 1. Rank 2. Wideband CQI | DRU, Mini-band based CRU | MIMO mode 1 |  |
| Mode 3 | OL SU MIMO SM (localized) | 1. Rank 2. Wideband CQI 3. Subband CQI 4. Subband Selection | Localized (Subband based CRU or Mini-band based CRU ) | MIMO mode 1 |  |
| Mode 4 | CL SU MIMO (localized) | 1. Rank 2. Wideband CQI (TBD) 3. Wideband PMI (TBD) 4. Subband CQI 5. Subband PMI 6. Subband Selection 7. (correlation matrix) | Localized (Subband based CRU or Mini-band based CRU) | MIMO mode 2 |  |
| Mode 5 | CL SU MIMO (Diversity) | 1. Rank 2. Wideband CQI 3. Wideband PMI (TBD) 4. Correlation matrix (TBD) | Diversity (Mini-band based CRU) | MIMO mode 2 |  |
| Mode 6 | OL MU MIMO (localized) | 1. Subband CQI 2. Wideband CQI 3. Subband Selection 4. (Stream indicator) | Localized (Subband based CRU or Mini-band based CRU) | MIMO mode 3 |  |
| Mode 7 | CL MU MIMO (localized) | 1. Subband CQI 2. Wideband CQI (TBD) 3. Wideband PMI (TBD) 4. Subband PMI 5. Subband Selection 6. (Correlation matrix) | Localized (Subband based CRU or Mini-band based CRU) | MIMO mode 4 |  |
| Mode 8 | CL MU MIMO (Diversity) | 1. Wideband CQI 2. Wideband PMI (TBD) 3. Correlation matrix (TBD) | Diversity (Mini-band based CRU) | MIMO mode 4 |  |

- 15.3.x.x.3.1  MIMO feedback 0

{to describe what MS to do for feedback and the feedback content/format}

- 15.3.x.x.3.2.  MIMO feedback 1

{to describe what MS to do for feedback and the feedback content/format}

- 15.3.x.x.3.3.  MIMO feedback 2

{to describe what MS to do for feedback and the feedback content/format}

- 15.3.x.x.3.4.  MIMO feedback 3

{to describe what MS to do for feedback and the feedback content/format}

- 15.3.x.x.3.5.  MIMO feedback 4

{to describe what MS to do for feedback and the feedback content/format}

- 15.3.x.x.3.6.  MIMO feedback 5

{to describe what MS to do for feedback and the feedback content/format}

- 15.3.x.x.3.7.  MIMO feedback 6

{to describe what MS to do for feedback and the feedback content/format}

[~~~~~~~~~~~~~~~~~~~~~ Recommended Text Proposal #22 ~~~~~~~~~~~~~~~~~~~~~~~]

[Note: Recommended text proposal #22 into UL/DL control parts]

Insert the following sub-section after 15.3.10.2.6.15.3.10.2.5. Usage of MIMO modes

- 15.3.x.x.y.  Downlink signaling support of UL-MIMO modes

###### 15.3.x.x.y.1.  Broadcast information

The BS shall send parameters listed in Table 17 which are necessary for UL MIMO operation shall be sent by the BS in a control channel with a broadcast CID. The parameters may be transmitted depending on the type of operation.

Table 17 – Broadcast information for UL MIMO operation [TBD]

| - Parameter | - Description | - Value | - Notes |
| --- | --- | --- | --- |

###### 15.3.x.x.y.2.  Unicast information

The BS shall send parameters listed in Table yy, which are necessary for MIMO operation, by unicast in a control channel to a specific MS. The parameters may be transmitted depending on the type of operation.

Table yy - Unicast information for UL MIMO operation

| - Parameter | - Description | - Value | - Control channel - (IE) | - Notes |
| --- | --- | --- | --- | --- |
| - MEF | - MIMO Encoding Format | - 0b00: SFBC - 0b01: Vertical encoding - 0b10: CSM - 0b11: No encoding [One TX antenna MS] | - A-MAP IE (unicast) | - MIMO encoder format [MEF bit-field may not be explicitly indicated in DL A-MAP IE] |
| - Mt | - Number of streams | - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - (Mt <= Nt ) | - A-MAP IE (unicast) | Number of streams in the MS transmission. |
| - RU allocation (TBD) | - LRU allocation | - TBD | - A-MAP IE (unicast) | - Refer to DL control group |
| - MaxMt | - Total number of streams in the LRU | - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 | - A-MAP IE (unicast) | Enabled when MEF=0b10 indicates the total number of streams in the LRU |
| - SI | - First pilot index | - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 | - A-MAP IE (unicast) | Enabled when MEF =0b10 1 bit for 2Tx, 2 bit for 4Tx |
| - PF | - Precoding flag | - 0b0: non adaptive precoding - 0b1: adaptive codebook precoding | - A-MAP IE (unicast) | Disabled when MEF=0b11 |
| - PMI | - Precoding matrix index | - TBD | - A-MAP IE (unicast) | Enabled when PF = 0b1 [Bit-field length is variable, depending on the number of code matrices] |

-------------------------------  Text End  ---------------------------------------------------