| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized text on Femto Idle State |  |
| Date Submitted | 2009-03-11 |  |
| Source(s) | Tricci So, Yang Liu, Jerry Chow, ZTE Corporation Masato Okuda Fujitsu Fang-Ching Ren ITRI | Liu.yang8@zte.com.cn okuda@jp.fujitsu.com frank_ren@itri.org.tw |
| Re: | CR to tgmsdd |  |
| Abstract |  |  |
| Purpose | For discussion and adoption into SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized text on Femto Idle State

Introduction

It is noticed that the design decision to assign the PGID is out of 16m scope, however, the requirement of the uniqueness of PG ID to support the paging for the femtocells should not be out of scope.

SDD Text Proposal

Black text: existing text in SDD (C802.16m-08/003r7)

Red Strikethrough text: deleted text

Blue text: new text

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

17.6	Idle Mode

The OSG Femtocell BSs operate like macro BSs when paging an MS.

Femtocell BS shall support idle mode. The CSG Femtocell BSs may broadcast the paging messages that are

related to only the MSs of this CSG.

Dependent on topology design to support both Femtocell BS and Macro BS, one or more PGs may be assigned to a Femtocell or a Macro-BS.

The overlay macro BSs and the CSG Femtocell BS may share the same paging group ID to avoid frequent location update. When an MS detects the change of PGID, it shall perform the location update.

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++