| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the DL and UL Basic Assignment A-MAP Information Elements in the 802.16m AWD |  |
| Date Submitted | 2009-04-30 |  |
| Source(s) | Roshni Srinivasan, Alexei Davydov, Guangjie Li, Yi Hsuan, Hujun Yin Intel Corporation Hyunkyu Yu, David Mazzarese, Mihyun Lee, Hokyu Choi Samsung Electronics Co. Ltd. Jinsoo Choi, Wookbong Lee, Jin Sam Kwak LG Electronics Yih-Shen Chen, Chih-Yuan Lin, Paul Cheng Mediatek Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, Hsi-Min Hsiao, Yutao Hsieh ITRI | roshni.m.srinivasan@intel.com hk.yu@samsung.com emptylie@lge.com yihshen.chen@mediatek.com zhengyanxiu@itri.org.tw |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the DL and UL Basic Assignment A-MAP Information Elements in the 802.16m AWD

Roshni Srinivasan, Alexei Davydov, Guangjie Li, Yi Hsuan, Hujun Yin

Intel Corporation

Hyunkyu Yu, David Mazzarese, Mihyun Lee, Hokyu Choi

Samsung Electronics Co. Ltd.

Jinsoo Choi, Wookbong Lee, Jin Sam Kwak

LG Electronics

Yih-Shen Chen, Chih-Yuan Lin and Paul Cheng

Mediatek

Zheng Yan-Xiu, Yu-Chuan Fang, Chang-Lan Tsai, Chung-Lien Ho, Hsi-Min Hsiao, Yutao Hsieh

ITRI

- Introduction

This contribution proposes changes to the Basic DL/UL A-MAP IEs currently defined in the 802.16m AWD [1] to include support for MIMO operation. All MIMO (OL/CL, SU/MU) modes and HARQ IR are supported in the Basic Assignment IEs for the DL and the UL.

Additional A-MAP IE types are also reserved to specify additional information elements that may be required for control signaling.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

- Amendment text proposal #1 for inclusion in 802.16m-09/0010r1a

-------------------------------  Start Text Proposal #1  ---------------------------------------------------

# DL Basic Assignment A-MAP IE

Table 661 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL .

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 661.

Table 661: DL Basic Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE() { | - | - |
| A-MAP IE Type | [4] 4 | TBD DL Basic Assignment A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| } else |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Determines the start, length and number of subframes spanned the allocation. Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length Indicator | TBD1 | Defines Indicates number of subframes spanned by allocations if not specified in resource allocation information. by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | TBD1 | Defines allocation relevance if not specified in resource allocation information Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| Boosting | [3] | 000: Normal (not boosted) 001: +6dB; 010: –6dB 011: +9dB; 100: +3dB 101: –3dB; 110: –9dB 111: –12dB |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] 4 | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between UL/DL, SU/MU OL/CL MIMO operation, persistent/non-persistent, single user/group resource allocation, basic/extended IEs. Additional IE types are reserved for future use.

MM: MIMO Mode includes indicators for OL/CL operation, SU/MU allocations, rate and the number of streams.

MEF: MIMO Encoder Format

PSI: Allocated pilot stream index for horizontal encoding

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

RA: Resource Allocation determines the start, length and number of subframes spanned the allocation. information is used to signal the type of resource unit allocated (DRU/CRU), the location (start/end) and allocation size.

PS: Payload size defines the size of the allocation, units TBD.

Long TTI Length Indicator: Indicator to signal allocations span multiple subframes in time.

Allocation Relevance: Subframe index corresponding to the A-MAP region in which the resource is allocated

HFA: TBD allocation for HARQ feedback.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.

Padding/Reserved Bits: padding to the nearest byte boundary.

Table 662 describes Assignment A-MAP IE Types

Table 662: A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | Reserved |
| 0b0011 | Reserved |
| 0b0100 | Reserved |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

-------------------------------  End Text Proposal #1 ---------------------------------------------------

Amendment text proposal #2 for inclusion in 802.16m-09/0010r1a

-------------------------------  Start Text Proposal #2 ---------------------------------------------------

# UL Basic Assignment A-MAP IE

Table 663 662 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 663 662 UL Basic Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL-MAP_IE() { | - | - |
| A-MAP IE Type | [4] 4 | TBD UL Basic Assignment A-MAP IE types distinguish between UL/DL, SU/MU, OL/CL MIMO operation, persistent/non-persistent allocation, basic/extended IEs |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MM | [4] | MIMO Mode TBD MIMO modes for the basic MIMO IE include commonly used modes. Additional modes may be specified in extended IEs. |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| If(MEF == 0b1){ |  | Parameters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| If(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | - 1 | - First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | - 2 | - First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding |
| If(PF == 0b1){ |  |  |
| PMI Indicator | 1 | - PMI Indicator - 0b0: the MS shall use the precoder of rank Mt of its choice - 0b1: the indicated PMI of rank Mt shall be used by the MS for precoding |
| if (PMI Indicator == 0b1){ |  |  |
| if(Nt == 2){ |  |  |
| PMI | - 3 | - Precoding Matrix Index for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | - 6 | - Precoding Matrix Index for Nt = 4 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Determines the start, length and number of subframes spanned the allocation. Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Payload Size | TBD | Defines payload size if not included in resource allocation information. Clarification on the use of this field is required |
| Long TTI Length Indicator | TBD1 | Defines Indicates number of subframes spanned by allocations if not specified in resource allocation information. by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | TBD1 | Defines allocation relevance if not specified in resource allocation information Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | TBD2 | Defines allocation relevance if not specified in resource allocation information Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| } |  |  |
| else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | TBD1 | Defines allocation relevance if not specified in resource allocation information Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | [4] [5] 3 | HARQ channel identifier |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

- TNS: Total number of streams in the LRU for CSM

- SI: First pilot index for CSM

- PF: Precoding flag to indicate adaptive or non-adsptive precoding

- PMI: Precoding matrix index

-------------------------------  End Text Proposal #2 ---------------------------------------------------