| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text of Persistent Allocation for the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-04-23 |  |
| Source(s) | Zhaohua Lu, Junhu Wang, Yanfeng Guan, Bo Sun, Huiying Fang ZTE Corporation | lu.zhaohua@zte.com.cn wang.junhu@zte.com.cn |
| Re: | Category: AWD comments / Area: Chapter 15.2.6 (Persistent scheduling) “Comments on AWD 15.3.6 Persistent scheduling” |  |
| Abstract | The contribution proposes text for DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of Persistent Allocation
for the IEEE 802.16m Amendment

- Introduction

This contribution proposes text on Persistent allocation to be included in the DL control section of the 802.16m amendment working document (AWD).

- References

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

- 4. Abbreviations and acronyms

[Note to Editor: Insert the following abbreviations and acronymns (Section 4):]

A-Preamble	advanced preamble

PA-Preamble	primary advanced preamble

SA-Preamble	secondary advanced preamble

P-SFH		primary superframe header

S-SFH 		secondary superframe header

A-MAP		advanced MAP

GRA		group resource allocation

PA		persistent allocation

IE		information element

MLRU		A-MAP logical resource unit

15.2.6 Persistent Scheduling in the Advanced Air Interface

15.2.6.x Allocation Mechanism

If an AMS receive DL Persistent Allocation A-MAP in frame k, the ABS shall not utilize the DL persistent resource to transmit data to the AMS until the ABS receive ‘ACK’ information on the relevant MAP ACK Channel.

If an AMS receive UL Persistent Allocation A-MAP in frame k, the AMS shall not utilize the UL persistent resource to transmit data to the ABS until the AMS send ‘ACK’ information on the relevant MAP ACK Channel.

15.2.6.y Error Handling Procedure

If an AMS have been allocated persistent resource in frame k, an indication of PA A-MAP existence or not shall be transmitted in frame (k + N * AP) to the AMS, N = 1, 2, … , the channel that is used to transmit this indication (called PA A-MAP Existence Indicate Channel) shall be indicated to the MS by Persistent Allocation A-MAP. Multiple AMS that have been allocated persistent resource may share the same PA A-MAP Existence Indicate Channel.

If an AMS have been allocated persistent resource in frame k, and receive ‘existence’ information on the PA A-MAP Existence Indicate Channel in frame (k + N * AP), N = 1, 2, … , and fail to get PA MAP in the frame, it shall transmit ‘NACK’ information through MAP NACK channel.

15.3.6.4 DL Control Information Elements

# 15.3.6.4.2.5 DL PA A-MAP IE

Table 664: DL Persistent Allocation A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| DL Persistent A-MAP IE () { | -- | -- |
| AMAP type | 4 | DL Persistent A-MAP IE |
| Persistent Region ID | TBD |  |
| Number of allocations | 5 | Number of allocation specified |
| RSTID Type | TBD |  |
| ACK Region Index | TBD | The index of the ACK region associated with all allocations defined in this DL Persistent A-MAP IE |
| Shared MAP NACK Channel Index | TBD | Index to a shared MAP NACK channel within the Fast Feedback region |
| PA A-MAP Existence Indicator Channel Index | TBD | This channel is used to transmit information that the PA A-MAP exist or not in relevant following subframe. |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| Region information | TBD | TBD Information that specifies the region relevant to this persistent scheduling instance |
| } else { | -- | -- |
| Region ID | 8 | Index to the DL region defined in DL region definition S-SFH |
| } | -- | -- |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| RSTID_IE() | variable | Specifies the station ID in RSTID format |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| Resource Offset | TBD | Indicates the start of this persistent allocation in the allocation region. |
| If (Persistent Flag = 1) { |  |  |
| If (j != 0 ){ |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. |
| If (Allocation Period and N_ACID Indicator = 1) { | - | - |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| }else{ |  |  |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| Shared MAP NACK Channel Index Indicator | 1 | Indicate the persistent allocation whether use Shared MAP NACK Channel 0 = not use 1 = use |
| If (Shared MAP NACK Channel Index Indicator = 0) | - | - |
| MAP NACK Channel Index | 6 | TBD Index to a shared MAP NACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| } |  |  |
| MAP ACK Channel Index | 6 | TBD Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| If (j = 0){ |  |  |
| Duration | TBD | Duration in number of LRUs. |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| Boosting | [3] | Specifies the boosting used for this allocation |
| ACID | TBD | Specifies the HARQ channel identifier used for this allocation |
| } else { |  |  |
| Duration Indicator | [1] | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this allocation. Otherwise, this allocation will use the same Duration as the previous allocation. |
| If (Duration Indicator = 1) { | - | - |
| Duration | TBD | Duration in number of LRUs. |
| } |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous allocation. |
| If (Allocation MCS indicator =1) { |  |  |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator =1) { |  |  |
| Boosting | [3] | Specifies the boosting used for this allocation |
| } |  |  |
| Allocation ACID indicator | 1 | If Allocation ACID Indicator is 1, it indicates that ACID is explicitly assigned for this allocation. Otherwise, this allocation will use the same ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation ACID indicator =1) { |  |  |
| ACID | TBD | Indicates the ACK channel to be used for this sequence of sub-bursts |
| } |  |  |
| } |  |  |
| ACK Channel Index | TBD | Indicates the ACK channel to be used for this sequence of allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| CRC | [16] | 16 bit CRC |
| } |  |  |

# 15.3.6.4.2.6 UL PA A-MAP IE

Table 665: UL Persistent Allocation A-MAP IE

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| UL Persistent A-MAP IE () { | -- | -- |
| AMAP type | 4 | UL Persistent A-MAP IE |
| Persistent Region ID | TBD |  |
| Number of allocations | 5 | Number of allocation specified |
| RSTID Type | TBD |  |
| Shared MAP NACK Channel Index | TBD | Index to a shared MAP NACK channel within the Fast Feedback region |
| PA A-MAP Existence Indicator Channel Index | TBD | This channel is used to transmit information that the PA A-MAP exist or not in relevant following subframe. |
| While (data remaining) { | -- | -- |
| Region ID use indicator | 1 | 0: Region ID not used 1: Region ID used |
| if (Region ID use indicator ==0) { |  |  |
| Region information | TBD | TBD Information that specifies the region relevant to this persistent scheduling instance |
| } else { | -- | -- |
| Region ID | 8 | Index to the UL region defined in UL region definition S-SFH |
| } | -- | -- |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| RSTID_IE() | variable | Specifies the station ID in RSTID format |
| FID | 4 | flow identifier |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| Resource Offset | TBD | Indicates the start of this persistent allocation in the allocation region. |
| If (Persistent Flag = 1) { |  |  |
| If (j != 0 ){ |  |  |
| Allocation Period and N_ACID Indicator | 1 | If Allocation Period and N_ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (N_ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Period and N_ACID Indicator = 1) { | - | - |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| }else{ |  |  |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| Number of ACID (N_ACID) | 3 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| Shared MAP NACK Channel Index Indicator | 1 | Indicate the persistent allocation whether use Shared MAP NACK Channel 0 = not use 1 = use |
| If (Shared MAP NACK Channel Index Indicator = 1) | - | - |
| MAP NACK Channel Index | 6 | TBD Index to a shared MAP NACK channel within the Fast Feedback region. The value 111111 is reserved. When MAP NACK Channel Index = 111111, it indicates NO MAP NACK channel is assigned to this allocation. |
| } |  |  |
| MAP ACK Channel Index | 6 | TBD Index to a MAP ACK channel within the Fast Feedback region |
| } |  |  |
| If (j = 0){ |  |  |
| Duration | TBD | Duration in number of LRUs. |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| Boosting | [3] | Specifies the boosting used for this allocation |
| ACID | TBD | Specifies the HARQ channel identifier used for this allocation |
| } else { |  |  |
| Duration Indicator | [1] | If Duration Indicator is 1, it indicates that Duration is explicitly assigned for this allocation. Otherwise, this allocation will use the same Duration as the previous allocation. If j is 1 then this indicator shall be 1. |
| If (Duration Indicator = 1) { | - | - |
| Duration | TBD | Duration in number of LRUs. |
| } |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous allocation. |
| If (Allocation MCS indicator =1) { |  |  |
| MCS | [6] | TBD Depends on supported modulation and coding schemes Specifies the modulation, coding and repetition coding used for this allocation |
| } |  |  |
| Allocation Boosting indicator | [1] | If Allocation Boosting Indicator is 1, it indicates that Boosting is explicitly assigned for this allocation. Otherwise, this allocation will use the same Boosting as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation Boosting indicator =1) { |  |  |
| Boosting | [3] | Specifies the boosting used for this allocation |
| } |  |  |
| Allocation ACID indicator | 1 | If Allocation ACID Indicator is 1, it indicates that ACID is explicitly assigned for this allocation. Otherwise, this allocation will use the same ACID as the previous allocation. If j is 0 then this indicator shall be 1. |
| If (Allocation ACID indicator =1) { |  |  |
| ACID | TBD | Specifies the HARQ channel identifier used for this allocation |
| } |  |  |
| } |  |  |
| AI_SN | 1 | HARQ identifier sequence number |
| SPID/CoRE Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| } |  |  |
| } |  |  |
| Padding | variable | Padding to bytes boundary; padding value shall be set to zero. |
| CRC | [16] | 16 bit CRC |
| } |  |  |

-------------------------------  Text End  ---------------------------------------------------