| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Facilitating Femtocell BS Detection with Location-Related Information |  |
| Date Submitted | 2009-04-24 |  |
| Source(s) | Feng Xie Zhaohua Lu Yang Liu ZTE Corporation | xie.feng2@zte.com.cn lu.zhaohua@zte.com.cn liu.yang8@zte.com.cn |
| Re: | Category: SDD comments / Area: Chapter 15 (Femtocell) “Comments on SDD 15 Femtocell” |  |
| Abstract | This contribution discusses Femtocell BS Detection |  |
| Purpose | To be discussed and adopted by TGm for use in the IEEE 802.16m SDD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Facilitating Femtocell BS Detection with Location-Related Information

Feng Xie, Zhaohua Lu, Yang Liu

ZTE Corporation

SDD Text Proposal

Black text: existing text in SDD (C802.16m-08/003r8)

Red Strikethrough text: deleted text

Blue text: new text

# Introduction

Femtocell BS is a low-power base station which can be deployed flexibly for improving the coverage or capacity of a small region. Some of the femtocell BSs, known as OSG, are open to all users without discretion while other CSG femtocell BSs are open to specific users with privileges. Given the high QoS offered by femtocell BSs, a user may prefer to access the femtocell BSs in the neighborhood. For example, a business man may be satisfied if he finds some accessible femtocell BS(s) in the visited place.

One can imagine that more and more MS(s) will be equipped with GPS or other positioning modules (such as those defined in Chapter 13 Support for Location Based Services). With the positioning technologies, a mobile user can easily get to know his actual locations and the neighbor environments. Generally, there might be some femtocells installed in the neighbor environment. However, a user may not be aware of the locations of some or all of the femtocell BSs that can be accessed. Unfortunately, he may miss some chance to move to the coverage area of femtocells and enjoy the high quality experience offered by femtocell BS. To address this problem, the network can help the users to access femtocell BSs by providing the location-related information of femtocell BSs.

Furthermore, location-related information of accessible femtocells can be used by an LBS-enabled MS to facilitate scan and handover. With location-related information of femtocells, handover between a macro BS and a femtocell BS can be triggered after the MS comes into the service area of accessible femtocells. For example, a GPS-enabled MS with the location-related information of accessible femtocells can easily determine if it enters the coverage area of femtocells and initiate the handover process.

# Proposed Text Modification to P802.16m System Description Document

[In IEEE 802.16m-08/003r8, Section 15.4.2, add the following text as the 3rd paragraph ]

-------------------------------  Text Start  ------------------------------------------------------------------------------------

MS may use location-related information of femtocells for femtocell BS detection. To this end, Macro BS or femtocell BSs may send location-related information to MS unsolicited or upon MS request. Based on the location-related information, MS can determine the proximity between femtocells and itself. After that, MS can start scanning for accessible femtocell BSs.

-------------------------------  Text End  ------------------------------------------------------------------------------------

[In IEEE 802.16m-08/003r8, Section 15.5.1, modify the following text]

-------------------------------  Text Start  ------------------------------------------------------------------------------------

15.5.1	HO from Macro BS to Femtocell BS

The network provides certain system information (e.g., carrier frequency of the Femtocells, and mapping information of overlay macro BSs and Femtocell BSs, and location-related information of femtocell BSs) to MSs for supporting handover between a macro BS and a Femtocell BS. An MS may cache this information for future handover to the specific Femtocell.

HO should be triggered based on certain criteria, such as signal strength, the proximity of MS to the Femtocell BS, and /or loading, etc. The macro BSs shall not broadcast neighbor CSG Femtocell BSs system configurations information in its neighbor list. At the time of handover preparation, the system information of a target Femtocell may be unicast or multicast to the MS upon MS request/network trigger or obtained by the MS monitoring the Femtocell, or based on the cached information of the MS.

The macro BSs may unicast or broadcast certain information (e.g. Cell ID, carrier frequency etc.) of OSG Femtocell BSs to facilitate MSs scanning for this kind of Femtocell BSs. An MS may report information of surrounding Femtocells to help macro BS generates optimized neighbor list. The MS may also request the accessible neighbor OSG/CSG Femtocell BSs information (e.g. location-related information of femtocell BSs) from the overlay macro BS when certain conditions are met.

-------------------------------  Text End  ------------------------------------------------------------------------------------