| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on sounding command IE and contents of S-SFH to support sounding operation |  |
| Date Submitted | 2009-04-27 |  |
| Source(s) | Alexei Davydov Roshni Srinivasan Intel Corporation Chih-Yuan Lin Pei-Kai Liao Paul Cheng Mediatek | alexei.davydov@intel.com chihyuan.lin@mediatek.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 (DL-CTRL)” |  |
| Abstract | The contribution proposes sounding commands for the 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposal on sounding command IE to support sounding operation

Alexei Davydov, Roshni Srinivasan

Intel Corporation

Chih-Yuan Lin, Pei-Kai Liao, Paul Cheng

Mediatek

# Introduction

To facilitate operation of the uplink sounding the parameters of the sounding channel and sounding instructions needs to be transmitted. This contribution proposes fields for broadcast channel and sounding instructions for unicast channel.

# Proposed text

Add the fields to the to the Table 658 in the section 15.3.6.4.1.2

-------------------------------------------------- Start Text Proposal #1 --------------------------------------------------

Table 658 – S-SFH SP4 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP4 IE () { |  |  |
| … |  |  |
| Uplink subframe bitmap Sounding Region | 8 | Uplink subframe bitmap with sounding symbol |
| Multiplexing Type [TBD] | 1 | 0b0: Frequency division multiplexing (FDM) 0b1: TBD Code division multiplexing (CDM) |
| If (Multiplexing Type == 0) { |  |  |
| Decimation value D | 2 | [TBD] |
| } else { |  |  |
| Max Cyclic Shift Index P | 2 | [TBD] |
| } |  |  |
| … |  |  |
| } |  |  |

----------------------------------------------- End of Text Proposal #1 -------------------------------------------------

Add the text to the section 15.3.6.4.2

-------------------------------------------------- Start Text Proposal #2 ------------------------------------------------

Line 60, page 67, insert 15.3.6.4.2.9 UL Sounding Command A-MAP IE

Table X - UL Sounding Command A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL Sounding Command IE format() { | - | - |
| A-MAP IE type | 4 |  |
| Sounding subframe | 3 | Indicates the sounding subframe [TBD] |
| Sounding subband bitmap | Variable [max. 24 for 2048] | [TBD] FFT size dependant TBD |
| If (Multiplexing type == 0) { |  |  |
| Decimation offset d | 4 | Unique decimation offset |
| } else { |  |  |
| Cyclic time shift n | 4 | Unique cyclic shift [TBD] |
| } |  |  |
| Periodicity | 3 | 0b000 = Single command, not periodic, or terminate the periodicy. Otherwise, repeat sounding once per 2(n-1) frames, where n is decimal value of the periodicy field |
| Multi-antenna sounding | 1 | 0b0: sounding from single antenna 0b1: sounding from multiple antennas |
| MCRC | 16 | 16 bit CRC masked by Station ID |
| } |  |  |

Table X specifies the fields of UL Sounding Command A-MAP IE used by the ABS to request sounding transmission by the AMS. Decimal equivalent of the sounding subframe indicates the uplink subframe with sounding symbol. The sounding subband bitmap filed is used to indicate the sounding subbands used in the sounding allocation. For that purpose, the Nused contiguous subcarriers are further subdivided into sounding subbands, where each sounding subband compromises N1 * Nsc adjacent subcarriers with Nsc = 18. For multiplexing type equal to 0 the first subcarrier index of kth transmit antenna within sounding allocation is determined from g = F(d, k, Frame Index), where function F() is TBD. The three periodicy bits are used to indicate the MS to periodically repeat the sounding transmission. Setting periodicy bits to 0b000 indicates a single sounding command or terminates the sounding if periodic sounding command is being performed. For multi-antenna flag equals to 0 the MS sounds only single transmit antenna, while for multi-antenna flag equals to 1 the MS sounds all transmit antennas.

----------------------------------------------- End of Text Proposal #2 -------------------------------------------------