| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Implementation issues for CSSI-based system |  |
| Date Submitted | 2009-04-18 |  |
| Source(s) | Xin Su, Tsinghua Univ. Hanyi Zhang, Tsinghua Univ. Yi Xu, Tsinghua Univ. Xibin Xu, Tsinghua Univ. Jing Wang, Tsinghua Univ. Shidong Zhou, Tsinghua Univ. Yunzhou Li, Tsinghua Univ. Xiaofeng Zhong, Tsinghua Univ. Xiaolu Dong, CATR Ying Du, CATR Shanpeng Xiao, CMCC Wenqi Liao, CMCC Lian Yang, Huawei Technologies Shiqiang Suo, Datang Mobile Zhongkai Wang, Tianjin Hi-Tech Industrial Park, Tianjin, China Weijun Xu, Tianjin Municipal Government, Tianjin, China Tim Ma, Legend Silicon Corp., Fremont, CA, USA | suxin@tsinghua.edu.cn zhanghy@wireless.mdc.tsinghua.edu.cn yi-xu07@mails.tsinghua.edu.cn xuxb@tsinghua.edu.cn wangj@tsinghua.edu.cn zhousd@tsinghua.edu.cn liyunzhou@tsinghua.edu.cn, zhongxf@tsinghua.edu.cn dongxiaolu@mail.ritt.com.cn duying@mail.ritt.com.cn xiaoshanpeng@chinamobile.com liaowenqi@chinamobile.com yang.lian@huawei.com suoshiqiang@datangmobile.cn wangzk@thip.gov.cn xwj@tipp.gov.cn tim.ma@legendsilicon.com |
| Re: | Response to IEEE 80216m-09_0010r1a “ IEEE 802.16m Amendment Working Document (AWD)” |  |
| Abstract | This document proposes a method to generate randomized interleaver patterns and some principles for patterns’ allocation in CSSI. |  |
| Purpose | For discussion and approval by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6>and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Implementation issues for CSSI-based system

Xin Su, Hanyi Zhang, Xibin Xu, Jing Wang, Shidong Zhou, Yunzhou Li, Xiaofeng Zhong

Tsinghua Univeristy

Dong Xiaolu, Du Ying

CATR

Xiao Shanpeng, Liao Wenqi

China Mobile

Lian Yang

Huawei Technologies

Shiqiang Suo

Datang Mobile

Zhongkai Wang

Tianjin Hi-Tech Industrial Park, Tianjin, China

Weijun Xu,

Tianjin Municipal Government, Tianjin, China

Tim Ma,

Legend Silicon Corp., Fremont, CA, USA

- 1. Introduction

CSSI (Cell/Sector-Specific Interleaving) is a potential technology which aims at suppressing the inter-cell/sector interference. The principle of CSSI is to employ distinct interleaving patterns in the neighbouring cells/sectors so that the UE can distinguish the signals from different cells/sectors by means of cell/sector-specific interleavers.

In this document, some detailed issues for CSSI implementation are proposed, including interleaver design, channel estimation and other requirements.

# 2. Implementation Issues

In an OFDM system, two neighbouring cells/sectors may first try to avoid reusing a chunk in the cell/sector-edge area by using inter-cell/sector interference coordination, as shown in Fig.1 (a). However, if the two cells/sectors have to reuse a chunk in the cell/sector-edge area (e.g. both the cells/sectors are heavy-loaded, or too complex to perform the interference coordination), CSSI provides an alternative way to mitigate the inter-cell/sector interference, as shown in Fig.1 (b). Some issues should be considered for implementing CSSI or iterative multi-cell/sector detection. Channel estimation and interleaver design should be carefully considered to enable CSSI. Multi-cell/sector detection will bring some extra requirements on e.g. synchronization and chunk size. However, when only a single-cell/sector detection is employed, these requirements can be released.

(a)

(b)

Fig.1. (a) Chunk allocation based on TDMA/FDMA

(b) Chunk allocation based on TDMA/FDMA and CSSI

## 2.1 Interleaver design

Usually, an interleaver is used as a component of a channel encoder to enhance the coding gain, or as a channel interleaver to combat the time/frequency coherent fading by scrambling burst errors into random errors. However, CSSI can also be used to randomize the inter-cell/sector interference. Since a number of random interleavers are required, the random interleaving, rather than block interleaving, should be used. The well-known random interleavers, e.g. s-random interleaver, could be considered. Of course a more powerful interleaver will bring a larger performance gain.A method to generate randomized interleaver patterns is introduced below:

Considering an interleaver with length L, and m is the minimum k which satisfies 2k>L, we will introduce an positive integer parameter , which is less than 2m. The input sequence is {xi}, each element can be a coded bit.

Let Ij denote the original index of the input sequence for the jth output element, i.e., the output sequence {yj} is y0y1y2⋯yL−1=xI0xI1xI2⋯xIL−1.

Let the m sequence generator with order m be gm(x), and can be written in binary form gm, e.g., g3(x)=x3+x2+1 can be written as gm=1101(B).

The Ij, j=0~L-1 can be generated by the following steps, the operator  means bit-by-bit module 2 sum. a<<b means the left shifting a by b bits. A>>b means the right shifting a by b bits.

s=;

j=0;

for k=0: 2m-1

t=s-1;

if t<L

Ij=t;

j=j+1;

end if;

if s>>(m-1)==1

s=(s<<1)gm;

else

s=s<<1;

end if;

end;

Different generating seeds  will lead to different interleaving patterns for different cells/sectors. The number of the different interleaver patterns generated with the above method is up to the length of interleaver.

Regarding the allocation of cell/sector-specific interleavers, different interleaver patterns can be allocated to different cells/sectors, by generating the interleavers with the corresponding seeds. A base station or subscriber station can identify the interleaver pattern (seed) of each cell/sector by checking its interleaver pattern ID.

In principle, hundreds of distinct seeds can be provided. However, using a distinct pattern for each cell/sector in the system is unnecessary. Hence, it is recommend to reuse a relatively small number of interleaver patterns (seeds) generation seed. The reuse of the seeds can be realized in a manner similar to that of frequency reuse in a cellular system.

## 2.2 Synchronization Requirement for multi-cell/sector detection

If CSSI is only used for signal randomisation and interference whitening, the inter-cell/sector synchronization is not required. However, if the performance gain provided by interference cancellation is desired, the use of multi-cell/sector detection will introduce the requirement to inter-cell/sector synchronization. (The inter-NodeB synchronization should be available for supporting multi-NodeB MBMS.) However, even though the neighbouring cells/sectors transmit signals simultaneously, the timing offsets between cells/sectors (due to the different distances to the UE) may result in self-interference. Let ΔToffset denote the timing offsets between adjacent cells/sectors (as shown in Fig.2), ΔTdelay−spread denote the channel delay spread, and  denote the CP length, if ΔToffset+ΔTdelay−spread<, the self-interference can be avoided.

Fig.2 Illustration of asynchronization between Node-Bs/sectors

- 3. Proposed Text for AWD

Insert the following text into Advanced Air Interface (i.e. Chapter 15 in [X]):

-------------------------------  Text Start  ---------------------------------------------------

15.3.12.1.6 Interleaving

15.3.12.1.6.x Interference mitigation using cell/sector-specific interleaving

15.3.12.1.6.x.3.1 Generation and allocation of interleaver patterns for CSSI

Random interleaving is suitable for the CSSI, which generates different interleaver patterns with different seeds.

Regarding the allocation of cell/sector-specific interleavers, different interleaver patterns can be generated with different seeds, and allocated to different cells/sectors.

In principle, hundreds of distinct seeds can be provided. However, using a distinct pattern for each cell/sector in the system is unnecessary. Hence, it is recommend to reuse a relatively small number of interleaver patterns generation seeds. The reuse of the seeds can be realized in a manner similar to that of frequency reuse in a cellular system.

-------------------------------  Text End  ---------------------------------------------------